3GPP TSG-SA3 Meeting #102-e 	S3-210071

- e-meeting, 18th - 29th January 2021

- Source:	KPN

- Title:	Updated Solution #22: Representation of identities during broadcast

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is requested to approve this update to Solution #22: Representation of identities during broadcast

# 2	References

[1]	3GPP TR 33.847 Study on security aspects of enhancement for proximity based services in the 5G System (5GS), version 0.3.0

# 3	Rationale

This contribution updates Solution #22 (previously called: 'Masked representation of identities') in order to resolve the editor's notes in the introduction and the solution details. As a consequence of the resolution of the editor's note on the need for having a Temporary Broadcast Identifier, the title of the solution has been revised to 'Representation of identities during broadcast'.

The following changes have been made:

- The Tempoary Broadcast Identifier has been removed from the solution.

- Instead of Masked representation (of the TBI), the solution now refers to a Representation of the UE identity.

- The applicability and working of the solution in out-of-coverage scenarios has been described.

- Examples/explanations are given (in the form of Notes) about the assignment/construction of Codes and Filters.

# 4	Detailed proposal

START OF CHANGE 1

## 6.22	Solution #22: Representation of identities during broadcast

### 6.22.1	Introduction

This solution addresses Key Issue #11, UE identity protection during ProSe discovery.

The solution is based on the idea to use representations of the UE identity during announcing (i.e. broadcasting) and monitoring (i.e. filtering). Using a representation enables resistance to tracing and impersonation of UEs.

The solution is described using the framework and terminology of ProSe Direct Discovery as described in TS 23.303 [5], i.e. for Model A discovery, the terms Announcing UE and Monitoring UE are used, and for Model B discovery, the terms Discoverer UE and Discoveree UE are used.

Part of the ProSE Direct Discovery procedure can work in out-of-coverage scenarios in the following sense: at the start of the discovery process, codes and filters are provided through a connection with the 5GC. After this step the discovery process can continue without the need for coverage. The connection with the 5GC can be direct or via a relay.

6.22.2	Solution details

#### 6.22.2.1 	Solution for Model A

The solution for Model A consists of the following components:

- 	The use of a Discovery Code (DC), representing the UE identity, that is broadcast by the Announcing UE for announcing its presence and its identity;

-	The use of a Discovery Filter (DF), that can be used by Monitoring UEs for recognizing and identifying the Announcing UE.

The corresponding information flows are described in Figure 6.22.2.1-1 for the Announcing UE and in Figure 6.22.2.1-2 for the Monitoring UE.

Figure 6.22.2.1-1 Direct discovery flows for Announcing UE

The steps performed by the Announcing UE during direct discovery are as follows, see also Figure 6.22.2.1-1.

1.	At the start of direct discovery the Announcing UE requests and receives from the 5GC a Discovery Code (DC) which is a representation of the UE identity;

NOTE 1:	It is implementation dependent how the 5GC assigns a DC to a particular UE; in its simplest form it can be (pseudo) random number; it also can be a hash applied to a UE identitier. If two distinct UEs are assigned the same DC, they will be indistinguishable during ProSe discovery.

Editor's note:	which entity in 5G is creating the DC is FFS.

Editor's note:	what interface is used for provisioning the DC is FFS.

NOTE 2:	The DC is carried in the response to the Discovery Request message, which is protected according to the solutions for Key Issue #10.

2.	The Announcing UE broadcasts its DC so that it can be discovered by other UEs; in case of restricted discovery only authorized UEs should be able to use the DC they receive.

Figure 6.22.2.1-2 Direct discovery flows for Monitoring UE

The steps performed by the Montoring UE during direct discovery are as follows, see also Figure 6.22.2.1-2.

1.	At the start of direct discovery the Monitoring UE requests and receives from the 5GC a Discovery Filter (DF) which is a representation of the Discovery Code broadcast by Announcing UEs that can be used to check for a match;

NOTE 3:	The Discovery Filter is constructed in the same way as has been described for LTE based ProSe (e.g. in clause 4.6.4.2a of TS 23.303 [5]).

Editor's note:	which entity in 5G is creating the DF is FFS.

Editor's note:	what interface is used for provisioning the DF is FFS.

NOTE 4:	The DF is carried in the response to the Discovery Request message, which is protected according to the solutions for Key Issue #10.

2.	The Monitoring UE uses the DF in order to try to match it against any received DCs; in case of a match it has discovered the Announcing UE that it looked for.

#### 6.22.2.2 Solution for Model B

The solution for Model B consists of the following components:

- 	The use of a Query Code (QC), that is broadcast by the Discoverer UE for announcing its query in search for (one or more) Discoveree UEs; the Query Code both represents the (properties of the) Discoveree UE and the Discoverer UE;

-	The use of a Query Filter (QF), that can be used by Discoveree UEs for recognizing the query sent by the Discoverer UE; it also can be used to identity the Discoverer UE (and based on that authorize the discovery of the Discoveree UE);

-	The use of a Response Code (RC), that is broadcast by Discoveree UE for announcing its response to the query received from the Discoverer UE; the Response Code also represents the Discoveree UE;

-	The use of a Response Filter (RF), that can be used by Discoverer UEs for recognizing the response sent by the Discoveree UE; it also can be used to identify the Discoveree UE.

The corresponding information flows are described in Figure 6.22.2.2-1 for the Discoverer UE and in Figure 6.22.2.2-2 for the Discoveree UE.

Figure 6.22.2.2-1 Direct discovery flows for Discoverer UE

The steps performed by the Discoverer UE during direct discovery are as follows, see also Figure 6.22.2.2-1.

1.	At the start of direct discovery the Discoverer UE requests and receives from the 5GC a Query Code (QC) and a Response Filter (RF); the Query Code is a representation of the identity of the Discoverer UE and of the identity of the Discoveree UEs that it wants to discover; the Response Filter is a representation of the Response Code broadcast by Discoveree UEs that can be used to check for a match;

NOTE 1:	The Response Filter is constructed in the same way as has been described for LTE based ProSe (e.g. in clause 4.6.4.2a of TS 23.303 [5]).

Editor's note:	which entity in 5G is creating the QC and RF is FFS.

Editor's note:	what interface is used for provisioning the QC and RF is FFS.

NOTE 2:	The QC and RF are carried in the response to the Discovery Request message, which is protected according to the solutions for Key Issue #10.

2.	The Discoverer UE broadcasts its QC so that it can be received by the Discoveree UEs that it looks for;

3.	The Discoverer UE uses the RF in order to try to match it against any received RCs; in case of a match it has discovered a Discoveree UE that it looked for.

Figure 6.22.2.2-2 Direct discovery flows for Discoveree UE

The steps performed by the Discoveree UE during direct discovery are as follows, see also Figure 6.22.2.2-2.

1.	At the start of direct discovery the Discoveree UE request and receives from the 5GC a Query Filter (QF) and a Response Code (RC); the Query Filter is representation of the Query Code broadcast by Discoverer UEs that can be used to check for a match; the Response Code is a representation of the identity of the Discoveree UE;

NOTE 3:	The Query Filter is constructed in the same way as has been described for LTE based ProSe (e.g. in clause 4.6.4.2a of TS 23.303 [5]).

Editor's note:	which entity in 5G is creating the QF and RC is FFS.

Editor's note:	what interface is used for provisioning the QF and RC is FFS.

NOTE 4:	The QF and RC are carried in the response to the Discovery Request message, which is protected according to the solutions for Key Issue #10.

2.	The Discoveree UE uses the QF in order to try to match it against any received QCs; in case of a match it has received a query from an interested Discoverer UE;

3.	The Discoveree UE broadcasts its RC so that it can be received by the interested Discoverer UE.

### 6.22.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

END OF CHANGE 1