3GPP TSG-SA3 Meeting #102-e 	S3-210261

- e-meeting, 18 - 29 January 2021											Revision of S3-20xxxx

- Source:	Address Editor’s Note in solution#14

- Title:	Huawei; HiSilicon

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

Kindly Request this contribution can be accepted by the group

# 2	References

NA

# 3	Rationale

The “Editor’s Note: The function of this solution out of coverage is FFS” is deleted, because the solution is designed for remote UE which has been out of coverage. Relay shall be always in the coverage.

The “Editor’s Note: security for PC5 is FFS” is deleted. Because step 4 mentions clause 6.7 of TR 33.847. Solutions to address KI#12 will provide the security solution.

The “Editor’s Note: this step needs to be revisited based on the RAN WG progress” is deleted, because the message forwarding method specified by the RAN groups does not affect on the security. Security has been established in step 6 of the solution.

# 4	Detailed proposal

**** 1st change****

## 6.14	Solution #14: A security solution for UE-to-Network Relay based on Layer 2 Relay

### 6.14.1	Introduction

This solution addresses Key Issue #3 on Security of UE-to-Network Relay. The solution is based on TR 23.752[2] solution #7 which is a Layer 2 Relay solution.

### 6.14.2	Solution details

Figure 6.14.2-1: UE-to-Network Relay solution for Layer2

0. If the Remote UE is in the coverage, the Remote UE perform initial registration to the network according to the registration procedure in TS 23.502[10]. If the Remote UE is not in the coverage, the Remote UE will perform the Initial Registration via the UE-to-Network Relay in step 7.

1. If in coverage, the Remote UE and UE-to-Network Relay UE independently get the service authorization for indirect communication from the network. If the Remote UE is not in coverage, the pre-configured information will be used.

2-3. The Remote UE and UE-to-Network Relay UE perform UE-to-Network Relay UE discovery and selection. For details of UE-to Network Relay discovery and selection for Layer2 UE-to-Network Relay see clause see clause 6.7.2.9 and Solution #19, Solution #41 of TR 23.752[2].

4. Remote UE initiate a one-to-one communication connection with the selected UE-to-Network Relay UE over PC5 using the solutions to address KI#12 in this document.

NOTE: when remote UE out of coverage, how to authorize the remote UE is addressed in key issue#4.

5. If the UE-to-Network Relay UE is in CM_IDLE state, triggered by the communication request received from the Remote UE, the UE-to-Network Relay UE sends a Service Request message to its serving AMF. The relay UE transitions to the connected state by sending a service request.

6. The Remote UE initials AS connection with NG-RAN via the UE-to-Network Relay UE to establish AS Connection with the same NG-RAN serving the Relay UE.

7. The Remote UE sends a NAS message to the serving AMF. The NAS message is encapsulated in an RRC message that is sent over PC5 to the UE-to-Network Relay UE, and the UE-to-Network Relay UE forwards the message to the NG-RAN. The NG-RAN derives Remote UE's serving AMF and forwards the NAS message to this AMF.

If the Remote UE has registered to the network in step0, then the NAS message is integrity protected by using the NAS security context derived in step0, and the UE puts 5G-GUTI in the NAS message. Both the UE and AMFshould perform the procedures defined in TS 33.501[14].

If the Remote UE has not registered to the network, then the UE should send a NAS message with a SUCI and perform primary authentication with the Remote UE’s AMF. Both the UE and AMF should perform the procedures defined in TS 33.501[14].

8. Remote UE may trigger the PDU Session Establishment procedure as defined in clause 4.3.2.2 of TS 23.502 [10]. The user plane security between the Remote UE and the gNB should reuse the procedure defined in clause 6.6 of TS 33.501[14].

9.	The data is transmitted between Remote UE and UPF via UE-to-Network Relay UE and NG-RAN. The UE-to-Network Relay UE forwards all the data messages between the Remote UE and NG-RAN using RAN specified L2 relay method.

6.14.3	Evaluation

**** End of change****