3GPP TSG-SA3 Meeting #102e 	draft_S3-210287-r1

- e-meeting, 18 – 29 January 2021

- Source:	Huawei, Hisilicon

- Title:	Addressing the EN on solution2

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

This contribution proposes to address the EN on solution 2 for TR33.850.

# 2	References

# 3	Rationale

In SA2, interim conclusion for Architecture: architecture option in Annex A.3 "5G MBS converged system architecture" is chosen as the baseline architecture for 5G MBS. The entity to handle the payload part to cater for the service level functions is defined as “MBSF-U”. The contribution is proposed to update the name to address the “Editor’s Note: The message name and flow may be updated to align with the conclusion from SA2 and RAN WGs.”

# 4	Detailed proposal

It is proposed to approve the following changes in MBS TR.

***	BEGIN OF 1st CHANGE	***

## 6.2	Solution #2: protect MBS traffic in service layer

### 6.2.1	Solution overview

This solution addresses Key Issue 2&3 to support the secure MBS traffic delivery from context provider to multiple UEs through 5GS. In the agreedarchitecture  in TR 23.757 [2], the MBSF-U (Multicast/Broadcast Service Function-User plane) is defined as a new entity to handle the payload part to cater for the service level functions and management. MBSF-C (Multicast/Broadcast Service Function-Control plane) is defined as a new entity to functionality to handle the control plane signalling. This solution protects the MBS traffic between the MBSF-U in the operator domain and the UE. It is independent to the protection in the application layer from the content provider.

The keys for protection of MBS traffic are generated in the MBSF-C. Afterwards, the keys are distributed to UEs and MBSF-U respectively. The UEs, which belongs to a multicast group, acquire the same keys in the MBSF-U. The keys can be updated in an efficient way.

### 6.2.2	Solution details

3.PDU session modification request

(multicast_group_info)

4.Nsmf PDU session update SMcontext

(multicast_group_info)

5.Multicast distribution session check

6.NamfcommunicationN1N2messageTransfer

7.N2 session request

2. Multicast announcement

1. UE registration and PDU session establishment

AMF

RAN

UE

(MB)-SMF

MBSF-CU

AUSF

UDM

16. generate K_group, K_transport_i, and select the security algorithms

8. RRC reconfiguration request

9. UE derive MUK based on Kausf and multicast_group_info

15. continue with the multicast service initiation procedure

Content Provider

11. AUSF derive MUK based on Kausf and multicast_group_info

10.MUK request

(multicast_group_info)

12.MUK response

(MUK)

13.MUK distribution

(MUK)

14. ACK

18a. transport key response

EMUK（key_ID, K_enc,  K_int, security algorithms）

17. transport and traffic key request

(token)

18b.key response and update

EMUK transport_1（group_key_ID, , K_group）,…, EK_transport_M (group_key_ID, K_group), H=Hash(K_group_ID)

Figure 6.2.2-1.The procedure to protect MBS traffic in service layer

The procedure is described as follows:

- The UE registers 5GS and establishs a PDU session.

- The content provider announces the availability of multicast using higher layers (e.g., application layer).

- The UE sends the PDU Session Modification Request. Information about multicast group including identifer of the multicast group which UE wants to join, shall be sent. Multicast_group_ID can be multicast address or other identifier.

- The AMF invokes Nsmf_PDUSession_UpdateSMContext, in which information about multicast group is included.

Editor’s Note: Step 3&4 need to be revised if SA2 agrees to support UE’s multicast session join/leave operation via UP e.g. IGMP Join/Leave.

- If MBS context is not available in (MB)-SMF, (MB)-SMF interacts with UDM/UDR to check whether a multicast context for the multicast group exists in the system.

- (MB)-SMF requests the AMF to transfer a message to the RAN node using the Namf_N1N2MessageTransfer service to create a multicast context in the RAN, if it does not exist already. IP address of MBSF-C may be included if needed for UE to find MBSF-C.

- The N2 session modification request is sent to the RAN.

- RAN sends RRC reconfiguration request message to UE.

- If UE is allowed to access the MBS service, UE derives Multicast User Key (MUK). The input key KEY is Kausf. The parameters are used to form the input string to the KDF including Multicast_group_ID. When re-authentication runs, the UE generates a new MUK and deletes the old MUK.

NOTE: The details of MUK derivation will be discussed in the normative work.

Editor’s Note: Key update procedure after reauthentication is FFS.

- SMF requests MUK and sends Multicast_group_ID to AUSF.

- AUSF derives Multicast User Key (MUK) based on Kasuf and Multicast_group_ID. When re-authentication runs, the AUSF generates a new MUK and deletes the old MUK.

- AUSF responds to SMF with MUK.

- SMF distributes MUK to MBSF-C.

- MBSF-C receives and stores the MUK. Afterwards, ACK is reponded to SMF.

- Continue with the multicast service initiation procedure.

- MBSF-C checks whether the MBS security context for this multicast group is available. MBS security context, which is used for MBS traffic protection, includes the key_ID, K_group_enc,  K_group_int, encryption and integrity algorithms. The key_ID is used to indicate which key pair is used. K_group_enc and  K_group_int are used for encryption and integrity protection of MBS traffic respectively.

If not, MBSF-C generates K_group and derives the K_group_enc and K_group_int. The encryption and integrity algorithms are selected.

The MBS security context is distributed from MBSF-C to MBSF-U.

- UE calculates token based on MUK and requests traffic key to MBSF-C. The token is secured with digital signatures or Message Authentication Codes (MAC). The input parameter includes UE_id, Multicast_group_ID, and fresh parameters.

- MBSF-C verifies the token using MUK and distributes the MBS security context to UE if succeeded.

Editor’s Note: Whether the roaming aspect is addressed based on the conclusion from SA2.

***	END OF 1st CHANGE	***