3GPP TSG-SA3 Meeting #102-e 	draft_S3-210410-r1

- e-meeting, 18 - 29 January 2021												Revision of S3-21xxxx

- Source:	Ericsson

- Title:	Resolving CP provisioning EN in solution #10

- Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

It is proposed to resolve one of the ENs in solution #10

# 2	References

[1]		3GPP TR 23.700-07: "Study on enhanced support of non-public networks (Release 17)"

# 3	Rationale

In solution #10 "Secure initial access to an SNPN onboarding network" the following EN is present:

Editor’s Note: How to protect the provisioning procedure via Control Plane regarding to the trust relationship between Onboarding SNPN and PS owner’s domain is FFS.

The security of provisioning would be part of key issue #2 and is therefore not in the scope of this solution. It is hence proposed to remove the Editor's Note.

# 4	Detailed proposal

***START OF CHANGES***

## 6.10	Solution #10: Secure initial access to an SNPN onboarding network

### 6.10.1	Introduction

This solution addresses key issue#4 Securing initial access for UE onboarding between UE and SNPN. The proposed solution relies on the deployment scenario described in Key issue #1 Credentials owned by an external entity where the deployment utilizes an external AAA-S. Therefore, the solution assumes the UE has been provisioned with default credentials to be used for primary authentication, the solution uses EAP-TLS as an example. Note that any solution candidate to Key issue #1 fits the concept of this solution. Using a key generating EAP method allows for derivation of keys to use protecting the air interface and the DCS provides a temporary SUPI to the onboarding network. This way the UE becomes uniquely identifiable and verifiably secure. Since EAP-TLS credentials does not have storage requirements on USIM, the UE cannot be assumed to have a USIM therefore it might not be possible to have a Home network public key available at the UE, therefore an anonymous SUCI is adopted.

### 6.10.2	Solution details

Figure 6.10.2-1: Initial access with key derivation

1.	The UE sends a registration request to the onboarding SNPN acting as onboarding network. The UE includes an onboarding indication and an anonymous SUCI as described in clause B 2.1.2.2 of TS 33.501 [2].

2.	AMF forwards the registration request to AUSF.

3.	The AUSF decides based on the onboarding indication that an external authentication is to be performed and uses the realm part of the SUCI to route the request to the right DCS.

4.	The AUSF interacts with the DCS in order to have the DCS perform primary authentication. The AUSF uses a AAA-P/IWF to interact with the DCS.

5.	UE and DCS performs primary authentication based on EAP-TLS. Since the SUCI was anonymous in line with clause B 2.1.2.2 of TS 33.501 [2] the tunnel is setup first before certificates are exchanged.

6.	The DCS sends an EAP response to the AUSF. Including keying material and a SUPI. In this case the UE ID from the certificate would act as SUPI.

7.	The AUSF sends a success message to the AMF including keying material and the SUPI.

8.	The AMF sends an indicator on how the UE shall derive its keys to the UE in a NAS message.

9.	The UE derives its keys and the registration is complete.

This solution describes initial access of the UE to the onboarding network using authentication with the DCS. The connection between UE and PS is not in scope of this solution.

Editor’s Note: Security implications of UE information pre-configuration, e.g. UE identity, authentication method, in O-SNPN regarding to the trust relationship between O-SNPN, DCS owner and PS owner is FFS.

Editor’s Note: The need for sending the ABBA parameter and the key indicator is FFS.

Editor’s Note: It is ffs that what should be preconfigured in the UE and how does the UE select the O-SNPN who can route the UE to the specific PS since the DCS may be shared by multiple O-SNPNs.

Editor’s Note: Privacy implication of disclosing the SUPI to the O-SNPN is FFS.

### 6.10.3	System impact

UE

Potentially key hierarchy depending on the outcome of KI#1.

AMF

Relay of potentially needed indicator for how the UE should derive keys. Relay of onboarding indicator.

AUSF

AAA-P functionality in order to communicate with external party.

AAA-S

Depends on if KI#1 decides the AAA-S should be 5G aware or not.

### 6.10.4	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

***END OF CHANGES***