3GPP TSG-SA3 Meeting #102-e 	S3-210417-r1

- e-meeting, 18 - 29 January 2021												Revision of S3-21xxxx

- Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- Title:	KI on Verification of UE in subscription and notification in "Subscribe-Notify" NF Service illustration 1 scenarios

- Document for:	Approval

Agenda Item:	5.20

# 1	Decision/action requested

KI added according to objectives of the SID added.

# 2	References

[1]	3GPP TS

# 3	Rationale

Key issue on verification of URI in subscription and notification for the "Subscribe-Notify" NF Service illustration 1 is proposed according to objectives of the SID added.

The new key issue is mainly copied from TR 33.855 for simplicity.

# 4	Detailed proposal

It is proposed to approve the following changes.

*********  START OF CHANGES

## 4.X	Key Issue #X: Service access authorization in the "Subscribe-Notify" NF Service illustration 1 scenarios

### 4.X.1	Key issue details

"Subscribe-Notify" NF Service illustration 1 specified in TS 23.501, clause 7.1.2, allows one NF (e.g. NF_A) to subscribe to notifications of NF producer (e.g. NF_B). The subscription request includes the notification endpoint (e.g. the notification URL) of the NF Service Consumer. In this scenario, NF_A subscribes the service of NF_B for itself.

Figure 4.X.1-1: "Subscribe-Notify" NF Service illustration 1

The security issue of "Subscribe-Notify" NF Service illustration 1 is that NF_B may redirect the notification message to an unauthorized NF consumer according to the unauthorized notification endpoint (e.g. the notification URL) of the subscribe request message. Furthermore, the notification message may include sensitive information, such as location report.

The existing access token based authorization defined in Rel15 and Rel16 cannot be reused directly for this "Subscribe-Notify" NF Service illustration 1 scenario, since the URI in the subscribe request message is not authorized.

This key issue seeks for solutions on how to verify the URI of the subscribe message in the "Subscribe-Notify" NF Service illustration 1 scenarios by the NF service producer, since otherwise notification messages could be forwarded to a NF, which is not authorized to receive the notification message.

### 4.Y.2	Security threats

TBD

### 4.Y.3	Potential security requirements

TBD

*********  END OF CHANGES