3GPP TSG-SA3 Meeting #102-e 	draft_S3-210427-r1

- e-meeting, 18 - 29 January 2021												Revision of S3-210427

- Source:	Nokia, Nokia Shanghai Bell

- Title:	KI on Dynamic authorization between SCPs or NF and SCP

- Document for:	Approval

Agenda Item:	5.20

# 1	Decision/action requested

KI added according to objectives of the SID added.

# 2	References

[1]	3GPP TS

# 3	Rationale

KI added according to objectives, i.e. copy from SID justification of the objective.

First proposal for threat clause added.

# 4	Detailed proposal

*********  START OF CHANGES

- 4.X	Key issue #X: Authorization of SCP to act on behalf of an NF or another SCP

- 4.X.1	Key issue details

This key issue is about authorization of SCP to request services on behalf of an NF or of another SCP and how this authorization is verified by the NRF or NF Service Producer.

- 4.X.2	Security threats

If the NRF cannot verify if the SCP has been authorized by the NF Service Consumer, the SCP can send a service request and receive a valid service response on behalf of NF Service Consumer, even though the NF Service Consumer has not authorized the SCP.

If the NF Service Producer cannot verify if the SCP has been authorized by the NF Service Consumer, the NF Service Producer can provide a service response to an unauthorized entity.

- 4.X.3	Potential security requirements

The 5GS should provide a mechanism for how an NRF or NF Service Producer can verify an SCP has been authorized by an NF Consumer to request access tokens or services on behalf of the consumer.

*********  END OF CHANGES