3GPP TSG-SA3 Meeting #102e 	S3-210461-r2

- e-meeting, 18th – 29th January 2021

- Source:	Samsung

- Title:	pCR: AMF re-allocation by re-directing UE to new AMF

- Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

SA3 is kindly requested to approve the proposed new solution to 33.864.

# 2	Rationale

This contribution proposes a new solution to avoid the registration failure during AMF re-allocation via RAN procedure in slice isolation scenarios.

# 3	Detailed proposal

It is proposed to add a new solution for AMF re-allocation procedure when AMF determines that AMF reallocation will require 5G NAS security context to be is rerouted via RAN.

- ****First Change***

- 6.X	Solution #X: AMF re-allocation by re-directing UE to new AMF

### 6.X.1	Solution Overview

The solution proposes that the Initial-AMF, upon determining that AMF re-allocation is needed and it cannot communicate with the new Target-AMF directly, sends Registration Accept message to the UE, containing a re-route assistance information. The re-route assistance information includes following information:

- A 5G-GUTI that is encoded for Target-AMF (set). It comprises of:

- AMF-Set ID in GUAMI set to that of Target-AMF, as returned from NSSF/NRF

- AMF Pointer set to 0xFFFFFF, or a reserved value

- 5G-TMSI set to random number

- An indication that UE needs to re-register to the network using 5G-GUTI provided in re-route assistance information.

A UE, receiving this information in Registration Accept Message re-initiates registration procedure by sending Registration Request with 5G-GUTI.

RAN, upon receiving the new routing information (derived from 5G-GUTI) along with new Registration Request, directly forwards the request to an AMF in target AMF set.

The new (target) AMF, upon receiving registration request containing a 5G-GUTI whose AMF Pointer is set to a reserved value (or 0xFFFFFF), understands that this is a re-routed registration request and proceeds with Primary Authentication procedure. Thus, it can now proceed with setting up fresh security context and registration procedure can succeed.

### 6.X.2	Solution Details

Figure  6.X.2-1 shows the detailed procedure.

Figure 6.X.2-1: Redirecting UE to Target-AMF

Editor’s Note:	AN update of the figure is needed to match the description

Step #1: UE initiates Registration procedure to connect to the network by sending registration request. RAN forwards the request to Initial-AMF. Initial-AMF may perform Primary authentication procedure.

Step #2: Initial-AMF may perofrms NAS SMC procedure with the UE. From this point onwards, UE only accepts ciphered/protected messages from the network. As part of “Security Mode Complete” message, UE also sends complete Registration Request to the UE, which includes Requested S-NSSAIs.

Step #3: Initial-AMF then initiates Nudm_SDM_Get procedure with UDM to download UE’s subscription data. The subscription data includes information about UE’s Subscribed S-NSSAIs.

Step #4: Based on UE’s Requested S-NSSAIs, Subscribed S-NSSAIs and other (e.g. locally configured) information, Initial-AMF determines if it cannot serve all the S-NSSAI(s) from the Requested NSSAI permitted by the subscription information. Following sequence of events follows:

- Initial-AMF invokes the Nnssf_NSSelection_Get service operation towards NSSF to retrieve Allowed NSSAI. The request to NSSF includes UE’s Requested S-NSSAIs, Subscribed S-NSSAIs, current tracking area (TAI) etc.

- NSSF responds to Initial-AMF with an AMF-Set-ID, or a list of AMF nf-Instance-IDs (e.g. of Target-AMF) which are better suited to serve the UE, along with allowed NSSAI.

Editor’s Note:	How the solution works when NRF/NSSF returns a subset of AMFs is FFS.

Step #5: If Initial-AMF, based on local configuration, determines that it cannot forward the Registration Request to Target-AMF directly and/or may need to go via RAN, it sends a Registration Accept message to the UE. The message includes re-route assistance information containing:

- A 5G-GUTI that is encoded for Target-AMF (set). It comprises of:

- AMF-Set ID in GUAMI set to that of Target-AMF, as returned from NSSF/NRF

- AMF Pointer set to 0xFFFFFF, or a reserved value

- 5G-TMSI set to random number

- An indication that UE needs to re-register to the network using 5G-GUTI provided in re-route assistance information.

Editor’s Note:	How the initial AMF learns about the new UE capability (to receive re-route assistance information) is FFS.

Step #6: UE re-initiates Registration procedure by sending Registration Request with 5G-GUTI as received above. RAN forwards the request to Target-AMF.

Step #7: Target-AMF, based on the presence of reserved value of AMF-Pointer in 5G-GUTI, determines that this is a re-routed Registration requests, and proceeds with identity request/response followed by Primary authentication procedure. Since no Inter-AMF routing via RAN is involved now, the registration procedure is able to proceed.

Editor’s Note:	The solution behavior for various cases mentioned in clause 4.3 “Architecture and security assumptions” is FFS

### 6.X.3	 Evaluation

Since the solution does not involve routing security context via RAN, or introducing a new NF connecting isolated slices, there is no compromise with security.

NOTE:	This solution has impacts to SA2 procedures (e.g. as defined in 3GPP TS 23.502, Clause 4.2.2.2.3), and SA2 may need to be requested to review and update their specifications.

****End of Changes ***