3GPP TSG-SA3 Meeting #103-e 	S3-211554

# e-meeting, 17 - 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0069 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on external file system mount restrictions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S3 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G |  |  |  |  |  | Date: |  |  | 2021-4-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As specified in the Docker community, for the overlay2 storage driver, the size option is only available if the backing fs is xfs and mounted with the pquota mount option: https://docs.docker.com/engine/reference/commandline/run/#set-storage-driver-options-per-container It means that the docker service need to check the quota is validated by creating a file device. However, if the administrators set the options nodev in the /etc/fstab, the storage check can not pass and result in docker service start failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This requirement does not apply when the docker is used to mount file system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing may bring wrong result. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** 1st CHANGE ****

4.3.3.1.6	External file system mount restrictions

Requirement Name: External file system mount restrictions

Requirement Description:

If normal users are allowed to mount external file systems (attached locally or via the network), OS-level restrictions shall be set properly in order to prevent privilege escalation or extended access permissions due to the contents of the mounted file systems.

Implementation example: In Linux® systems, administrators shall set the options nodev and nosuid in the /etc/fstab for all filesystems, which also have the "user" option.

NOTE: 	This requirement does not apply when the docker is used to mount file system.

Test Case:

Test Name: TC_EXTERNAL_FILE_SYSTEM_MOUNT_RESTRICTIONS

Purpose:

Verify that OS-level restrictions are set properly for users that are allowed to mount external file systems (attached locally or via the network). This is to prevent privilege escalation or extended access permissions due to the contents of the mounted file systems.

Procedure and execution steps:

Pre-Condition:

Tester has admin access to check and configure the external filesystem mount permissions in the OS.

Tester has username and password of a user in the network product that has external filesystem mount privileges.

Execution Steps

Execute the following steps:

1.	The tester shall verify that OS-level restrictions are set properly in order to prevent privilege escalation due to the contents of the mounted file systems (e.g. In Linux® systems, administrators shall set the options nodev and nosuid in the /etc/fstab for all filesystems, which also have the "user" option). The tester checks that OS-level parameters are configured correctly on the system.

2.	The tester mounts an external filesystem prepared by the tester with files exploiting privilege escalation methods (e.g. with writable SUID/GUID files).

3.	The tester tries to gain privileged access to system by using a suitable privilege escalation method using the contents of the mounted file system and then confirms that privilege escalation doesn't happen.

Expected Results:

The OS-level restrictions are set properly in order to prevent privilege escalation or extended access permissions due to the contents of the mounted file systems.

Any privilege escalation method used by the tester should be blocked.

Expected format of evidence:

Screenshot containing the configuration file showing that OS-level restrictions are set properly for users that are allowed to mount external file systems.

**** END OF CHANGES ****