3GPP TSG-SA3 Meeting #103-e 	S3-21xxxx

- e-meeting, 17 - 28 May 2021															Revision of S3-202007

- Source:	Xiaomi

- Title:	eNS2: Add Threat Analysis to KI #1

- Document for:	Approval

Agenda Item:	5.21

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed update to key issue #1 of TR 33.874.

# 2	References

[1]	3GPP TR 33.874 v0.1.0		Study on enhanced security for Network Slicing Phase 2

[2]			3GPP TR 38.832	v1.0.0		Study on enhancement of Radio Access Network (RAN) slicing

[3] 			3GPP TS 23.501	v17.0.0	System architecture for the 5G System (5GS)

# 3	Rationale

In current version of TR 33.874 [1], the example of slice related information broadcast in SIB by a gNB is NSSAI. However, according to RAN2 discussion, NSSAI is not to be included in the SIB, due to the concern of the large payload size of possibly multiple NSSAIs supported by the gNB. The examples of slice related information given in the latest version of TR 38.832 [2] include SST, slice grouping or slice associated UAC information. As NSSAI is the key information inducing the discussion on privacy issue in SA3, this pCR proposes to update the examples of slice related information in key issue details.

Accordingly, the threat analysis focusing on the SST is proposed, based on the definition in TS 23.501 [3].

# 4	Detailed proposal

*************** Start of the Change ****************

## 5.1	Key Issue #1: privacy issue on broadcasting slice information

### 5.1.1	Key issue details

A gNB may support multiple and different network slices, and on different frequencies in different regions.

In TR 38.832 [6], in order to support fast cell selection and cell reselection for particular network slices, solutions based on broadcasting slice related information are being studied. The broadcast slice related cell info may contain e.g. NSSAI, SST, slice grouping or slice associated information. In this key issue, the following questions are to be addressed:

- Whether broadcasting slice related information in this scenarios will cause any privacy issue

- If yes, mitigation solutions need to be provided

Editor’s Note: as per current TR 33.832 [6], NSSAI is not contained in the broadcast SIB. Whether NSSAI is already excluded from the broadcast SIB or not is to be confirmed by RAN2.

5.1.2	Security threats

According to TS 23.501 [1], SST refers to the expected Network Slice behaviour in terms of features and services. An SST could be represented with a standardised SST value or without a standardised SST value. The currently standardized SST values can indicate the slice types of eMBB, URLCC, MIoT and V2X, from which sensitive information of a specific slice can hardly be derived. Hence there is no privacy issue if SST is included in the broadcast SIB.

Editor’s Note: In case the S-NSSAI supported by RAN node consists only of an SST field value (without SD field), the privacy implication of broadcasting SST is FFS.

Editor’s Note: the privacy issue of slice grouping and slice associated info is FFS depending on their definition to be made by RAN2.

### 5.1.3	Potential security requirements

*************** End of the Change ****************