3GPP TSG-SA3 Meeting #104-e 	S3-212411

# e-meeting, 16 - 27 August 2021

# Source:	3GPP SA2

# Title:	LS on UE capabilities indication in UPU

# Document for:	Information, Discussion

# Agenda Item:  4.17

# SA WG2 Meeting #143-e (e-meeting)	S2-2101072

# 24 February- 9 March 2021, Elbonia

Title:		LS on UE capabilities indication in UPU

Reply to:	-

Release:	Release 17

Work Item:	eNPN/FS_eNPN

Source:	SA2

To:		CT1, SA3

Cc:		-

Contact person:	Haris Zisimopoulos

harisz@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

In TR 23.700-07 it is already agreed to use UE Parameters Update (UPU) procedure (what is called also “CP provisioning”) for the following parameters:

- Provisioning of NSSAA credentials (as per conclusion of KI#4 in TR 23.700-07)

- Provisioning of PDU session authentication credentials (as per conclusion of KI#4 in TR 23.700-07)

- Preferred SNPN and Group ID list (KI#1 in TR 23.700-07, TBD whether UPU or SoR is to be used

In addition, the following may also be supported with UPU:

- Provisioning of SNPN credentials (for SNPN, tentative agreement in KI#4 in TR 23.700-07)

In rel.16 SA2 understands that the UPU procedure was used for provisioning of Routing ID and default S-NSSAI and support for update of Routing ID and default configured NSSAI is currently mandatory at the UE and therefore there is no indication of UE support.

Taking into account that support for the new parameters that SA2 agreed to be provisioned with UPU are going to become optionally supported in the UE, there is a need for an indication of UE support for each of these new parameters to UDM. For the case of provisioning of SNPN credentials, SA2 has not reached consensus whether the intermediate node (AMF) should be aware of the UE and network capabilities.

Since the UPU procedure was mostly specified between CT1 and SA3 and SA2 only did alignment CRs in TS 23.502, SA2 expects that CT1 and SA3 find a solution for indicating capabilities for the parameters contained in UPU procedure in rel.17.

# 2	Actions

To CT1 and SA3

ACTION: 	SA2 asks CT1 and SA3 to find a solution for capability negotiation for the parameters contained in UPU procedure in rel.17.

# 3	Dates of next TSG SA WG2 meetings

3GPPSA2#144-e	12-16 April 2021	Electronic Meeting

3GPPSA2#145-e	17-28 May 2021	Electronic Meeting