3GPP TSG-SA3 Meeting #104-e 	S3-21xxxx

- e-meeting, 16 - 27 August 2021														Revision of S3-212957

- Source:	Xiaomi

- Title:	ProSe: Addional Evaluation for Solution #18

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed additional evaluation for solution #18 of TR 33.847.

# 2	References

[1]	3GPP TR 33.847 v0.6.0		Study on security aspects of enhancement for proximity based services in the 5G System (5GS)

# 3	Rationale

This pCR proposes to add more evaluation for solution #18 in TR 33.847 [1].

# 4	Detailed proposal

*************** Start of the Change ****************

## 6.18	Solution #18: Authorization and PC5 link setup for UE-to-Network relay

### 6.18.1	Introduction

This solution addresses the KI #3 and the KI #4. This solution provides a mechanism to setup a PC5 link between a remote UE and UE-to-network relay. In addition, this solution describes how a Remote UE and UE-to-network relay get authorized by the ProSe Key Management Function and verify each other’s role. This solution assumes 5G Prose function and Prose Key Management Function as in LTE Prose. This solution only describes the PC5 link setup procedure that is common for both L2 and L3 UE-to-network relay.

### 6.18.2	Solution details

Figure 6.18.2-1: Authorization and secure PC5 link establishment procedure for UE-to-network relay

NOTE: In this solution, the remote UE needs to be provisioned with the discovery security materials and Remote User Key when it is in coverage. Also, those security materials are associated with an expiration time, after which they become invalid. When the security materials become invalid the Remote UE needs to be in coverage to obtain fresh ones to be able to connect via relay.

Editor’s Note: The architecture of this solution needs to be aligned with SA2.

0a. The Remote UE gets the discovery parameters and ProSe Key management function (PKMF) address from the 5G DDNMF.

NOTE: The Remote UE may get multiple PKMF addresses for different PLMNs. If the Remote UE receives multiple PKMF addresses, it will contact each of PKMFs separately. The remote UE may contact those PKMFs directly or via the PKMF of its home PLMN.

0b. The Remote UE is authorized to receive UE-to-network relay service and gets the discovery security material from the PKMF

0c. The UE-to-network relay gets the discovery parameters and ProSe Key management function (PKMF) address from the 5G DDNMF.

0d. The UE-to-network relay is authorized to act as a relay and gets the discovery security material from the PKMF.

The remote UE and relay UE communicate with the PKMF via PC8 reference point (like in LTE Prose [6]). Security for PC8 interface relies on Ua security if GBA [12] is used or Ua* when AKMA [7] is used.

NOTE: For commercial services, the PKMF is located in the operator’s network. For Public Safety use cases, the PKMF can be managed by the Public Safety service provider.

1a. The Remote UE sends a Prose Remote User Key (PRUK) Request message to the PKMF of the UE-to-network relay.

1b. The ProSe Key Management Function checks that the Remote UE is authorised to receive UE-to-network Relay service. This is done by using the Remote UE’s identity that is bound to the keys that established the secure connection between the Remote UE and PKMF in step 0b. If the Remote UE is authorised to receive the service, the PKMF sends a PRUK and PRUK ID to the Remote UE.

2. The discovery procedure is performed between the Remote UE and the UE-to-network Relay using the discovery parameters and discovery security material.

Editor’s Note: The detail of discovery security material is FFS, and how it impacts on discovery procedure needs to be clarified

3. The Remote UE sends a Direct Communication Request that contain the PRUK ID, Relay Service Code (RSC) of the UE-to-network relay service and KNRP freshness parameter 1.

4a. The UE-to-network relay sends a Key Request message that contains PRUK ID, RSC and KNRP freshness parameter 1 to the PKMF.

4b. On receiving the Key Request message, the PKMF checks that the UE-to-network relay is authorized to act as a relay to the Remote UE. This is done by using the relay’s identity that is bound to the keys that established the secure connection between the relay and PKMF in step 0d. If the UE-to-network relay is authorized to provide the relay service, the PKMF generates KNRP freshness parameter 2 and derives KNRP using PRUK identified by PRUK ID, KNRP freshness parameter 1 and KNRP freshness parameter 2. Then, the PKMF sends a Key Response message that contains KNRP and KNRP freshness parameter 2 to the UE-to-network relay.

5a. The UE-to-network relay sends a Direct Security Mode Command message to the Remote UE (see 6.5.2.2). This message contains the KNRP Freshness Parameter 2 and protected based on the session key (KNRP-SESS) derived from KNRP. The Direct Security Mode Command message is integrity protected using the integrity protection key (KNRPIK) derived from the session key (KNRP-SESS).

Editor’s Note: How to support flexibility between remote UE and relay UE is FFS.

5b. The Remote UE derives KNRP from its PRUK, RSC, KNRP Freshness Parameter 1 and the received KNRP Freshness Parameter 2. It then derives the session key (KNRP-SESS) in the same manner as the UE-to-network relay and processes the Direct Security Mode Command. The Remote UE further derives the integrity protection key (KNRPIK) and encryption key (KNRPEK) from the session key (KNRP-SESS). Then, the Remote UE checks the integrity of the Direct Security Mode Command message. If the integrity check is successful, the Remote UE is assured that the UE-to-network relay is authorized to provide the relay service.

5c. The Remote UE responds with a Direct Security Mode Complete message to the UE-to-network relay. The Direct Security Mode Complete message is ciphered and integrity protected.

5d. On receiving and processing the Direct Security Mode Complete message, the UE-to-network relay checks the integrity of the Direct Security Mode Complete message. If the integrity check is successful, the UE-to-network relay is assured that the Remote UE is authorized to get the relay service.

6. The remote UE and UE-to-network relay continues the rest of procedure for the relay service over the secure PC5 link.

NOTE: The rest of procedure is determined depending on the UE-to-network relay types (i.e., L2 or L3 relay).

### 6.18.3	Evaluation

This solution fulfils security requirements of KI#3 as follows:

- This solution supports a secure means to establish a PC5 link between the remote UE and the U2N relay based on the security materials provisioned by the PKMF.

This solution fulfils the security requirements of KI#4 based on the PKMF’s authorization of the remote UE and the U2N relay. The UE authorization by PKMF in this solution is different from the UE authorization specified in TS 23.304 [16].

This solution reuses the ProSe UE-to-network relay security defined for LTE in TS 33.303 [6], which involves the ProSe Key Management Function (PKMF) of the Relay UE. The PKMF is only used for Public Safety in LTE. Introducing PKMF for commercial Prose services in 5G may require different deployment model than in LTE.

In this solution, the PC5 link root key KNRP is sent over the air interface to the Relay UE. It requires that the NAS message carrying KNRP is both integrity and confidentiality protected. Otherwise, there is the risk that the root key KNRP is exposed in clear text if confidentiality protection of the NAS signaling is not activated.

This solution requires the Remote UE to be in coverage to get provisioned with the discovery security materials and Remote User Key.

Editor’s Note: Further evaluation is FFS.

*************** End of the Change ****************