3GPP TSG-SA3 Meeting #107e 	draft_S3-220850-r1

- e-meeting, 16 - 20 May 2022

- Source:	Huawei, HiSilicon

- Title:	Key derivation related clarification in CP-based UE-to-Network relay procedures

- Document for:	Approval

Agenda Item:	4.7

# 1	Decision/action requested

Approve this clarification to fix mistakes.

# 2	References

[1]			3GPP TS 33.503 Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS).

# 3	Rationale

The contribution fixs the following mistakes:

- In step 2-5, the ‘PC5 security signalling policy’, should be modified to ‘the PC5 signalling security policies’.

- In step 12, based on TS 33.536, the Direct Security Mode Command is sent only with integrity protection. Clarification about key derivation is added based on TS 33.536.

# 4	Detailed proposal

*************** Start of the 1st Change ****************

##### 6.3.3.3.2	Connection with 5G ProSe UE-to-Network Relay connection with setup of network Prose security context during PC5 link establishment

This subclause describes a procedure for a 5G ProSe Remote UE to establish a PC5 link between a 5G ProSe Remote UE and a 5G ProSe UE-to-Network Relay. The procedure includes how the 5G ProSe Remote UE is authenticated by AUSF via 5G ProSe UE-to-Network Relay and 5G ProSe UE-to-Network Relay's AMF during 5G ProSe PC5 establishment. The mechanism can be used by a 5G ProSe Remote UE while out of coverage.

Figure 6.3.3.3.2-1: 5G ProSe UE-to-Network Relay security procedure with setup of network Prose security context during PC5 link establishment

0.	The 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay shall be registered with the network. The 5G ProSe UE-to-Network Relay shall be authenticated and authorized by the network to provide UE-to-Network relay service. The 5G ProSe Remote UE shall be authenticated and authorized by the network to receive UE-to-Network relay service. PC5 security policies are provisioned to the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay respectively during this authorization and information provisioning procedure.

1.	The 5G ProSe Remote UE shall initiate discovery procedure using any of Model A or Model B method as specified in clause 6.3.1.2 or 6.3.1.3 of TS 23.304 [2] respectively.

2-5.	After the discovery of the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE shall send a Direct Communication Request to the 5G ProSe UE-to-Network Relay for establishing secure PC5 unicast link. The 5G ProSe Remote UE shall include its security capabilities and PC5 signalling security policies in the DCR message as specified in TS 33.536 [6]. The message shall also include SUCI, Relay Service Code, Nonce_1. Upon receiving the DCR message, the 5G ProSe UE-to-Network Relay shall send the Relay Key Request to the AMF of the 5G ProSe UE-to-Network Relay, including partial parameters received in the DCR message. The 5G ProSe UE-to-Network Relay shall also include in the message a transaction identifier that identifies the 5G ProSe Remote UE for the subsequent messages over 5G ProSe UE-to-Network Relay's NAS messages and PC5 messages. The AMF of the 5G ProSe UE-to-Network Relay shall verify whether the 5G ProSe UE-to-Network Relay is authorized to provide the UE-to-Network relay service. The AMF of the 5G ProSe UE-to-Network Relay shall select an AUSF based on SUCI and forward the parameters received in Relay Key Request to the AUSF in Nausf_UEAuthentication_ProseAuthenticate Request message. The Nausf_UEAuthentication_ProseAuthenticate Request message shall contain the 5G ProSe Remote UE’s SUCI, Relay Service Code, Nonce_1. The AUSF shall initiate a 5G ProSe Remote UE specific authentication using the ProSe specific parameters received (i.e., RSC, etc.). The serving network name handling is same as defined in TS 33.501 [3]. The security policy negotiation and protection of messages hereafter shall follow the one-to-one security establishment described in clause 6.2.3 of the present document.

6.	The AUSF of the 5G ProSe Remote UE shall retrieve the Authentication Vectors from the UDM via Nudm_UEAuthentication_GetProseAv Request message and trigger authentication of the 5G ProSe Remote UE . This authentication is performed between the AUSF of the 5G ProSe Remote UE and the 5G ProSe Remote UE via the AMF of the 5G ProSe UE-to-Network Relay and the 5G ProSe UE-to-Network Relay. Based on SUPI, the UDM shall choose the authentication method.

7a. If EAP-AKA' is selected by UDM, the AUSF of  the 5G ProSe Remote UE shall trigger authentication of the 5G ProSe Remote UE based on EAP-AKA'. The AUSF of the 5G ProSe Remote UE generates the EAP-Request/AKA'-Challenge message defined in clause 6.1.3.1 of TS 33.501 and send EAP-Request/AKA'-Challenge message to the AMF of the 5G ProSe UE-to-Network Relay in a Nausf_UEAuthentication_ProSeAuthenticate Response message.

7b. The AMF of the 5G ProSe UE-to-Network Relay shall forward the Relay Authentication Request (including the EAP-Request/AKA'-Challenge) to the 5G ProSe UE-to-Network Relay over NAS message, including transaction identifier of the 5G ProSe Remote UE in the message. The NAS message is protected using the NAS security context created for the 5G ProSe UE-to-Network Relay.

7c. Based on the transaction identifier, the 5G ProSe UE-to-Network Relay shall forwards the EAP-Request/AKA'-Challenge to the 5G ProSe Remote UE over PC5 messages.

The USIM in the 5G ProSe Remote UE verifies the freshness of the received values by checking whether AUTN can be accepted as described in TS 33.102 [11].

For EAP-AKA', the USIM computes a response RES. The USIM shall return RES, CK, IK to the ME. The ME shall derive CK' and IK' according to Annex A.3 in TS 33.501.

7d. The 5G ProSe Remote UE shall return EAP-Response/AKA'-Challenge to the 5G ProSe UE-to-Network Relay over PC5 messages.

7e. The 5G ProSe UE-to-Network Relay forwards the EAP-Response/AKA'-Challenge together with the transaction identifier of the 5G ProSe Remote UE to the AMF of the 5G ProSe UE-to-Network Relay in a NAS message Relay Authentication Response.

7f. The AMF of the 5G ProSe UE-to-Network Relay forwards EAP-Response/AKA'-Challenge to the AUSF of the 5G ProSe Remote UE via Nausf_UEAuthentication_ProSeAuthenticate Request.

The AUSF of the 5G ProSe Remote UE performs the UE authentication by verifying the received information as described in TS33.501.

For EAP-AKA’, the AUSF of the 5G ProSe Remote UE and the 5G ProSe Remote UE may exchange EAP-Request/AKA’-Notification and EAP-Response /AKA’-Notification messages via the AMF of the 5G ProSe UE-to-Network Relay. After the exchanges, the AUSF of the 5G ProSe Remote UE derives KAUSF without calculating the KSEAF.

The AUSF of the 5G ProSe Remote UE and the 5G ProSe Remote UE shall derive a new KAUSF_P (different from KAUSF). NAS SMC procedure is not performed between 5G ProSe Remote UE and AMF of the 5G ProSe UE-to-Network Relay.

8.	On successful authentication, the AUSF of the 5G ProSe Remote UE and the 5G ProSe Remote UE shall generate 5GPRUK as specified in Annex A.2 and 5GPRUK ID as specified in Annex A.3 using the newly derived KAUSF_P.

9.	The AUSF of the 5G ProSe Remote UE shall generate the KNR_ProSe key as defined in Annex A.4.

10-11.	The AUSF of the 5G ProSe Remote UE shall send the KNR_ProSe, Nonce_2 in Nausf_UEAuthentication_ProseAuthenticate Response message to the 5G ProSe UE-to-Network Relay via the AMF of the 5G ProSe UE-to-Network Relay. When receiving a KNR_ProSe from the AUSF of the 5G ProSe Remote UE, the AMF of the 5G ProSe UE-to-Network Relay shall not attempt to trigger NAS SMC procedure with the 5G ProSe Remote UE. The 5G ProSe UE-to-Network Relay derives PC5 session key Krelay-sess and confidentiality key Krelay-enc (if applicable) and integrity keys Krelay-int from KNR_ProSe, as defined in clause 6.3.3.3.3 of this document. KNR_ProSe ID and Krelay-sess ID are established in the same way as KNRP ID and KNRP-sess ID in TS 33.536 [6].

12.	The 5G ProSe UE-to-Network Relay shall send the received Nonce_2 and PC5 signalling security policies to the 5G ProSe Remote UE in Direct Security mode command message, which is integrity protected using Krelay-int.

13-15.	The 5G ProSe Remote UE shall use the 5GPRUK ID to locate the KAUSF_P/5GPRUK to be used for the PC5 link security. The 5G ProSe Remote UE shall generate the KNR_ProSe key to be used for Remote access via the 5G ProSe UE-to-Network Relay in the same way as defined in step 9. The 5G ProSe Remote UE shall derive PC5 session key Krelay-sess and confidentiality and integrity keys from KNR_ProSe the same way as defined in step 11. The 5G ProSe Remote UE shall send the Direct Security Mode Complete message containing its PC5 user plane security policies to the 5G ProSe UE-to-Network relay, which is protected by Krelay-int or/and Krelay-enc derived from Krelay-sess according to the negotiated PC5 signalling policies between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay. After the successful verification of the Direct Security Mode complete message, the 5G ProSe UE-to-Network Relay responds a Direct Communication Accept message to the 5G ProSe Remote UE to finish the PC5 connection establishment procedures.

Further communication between the 5G ProSe Remote UE and the Network takes place securely via the 5G ProSe UE-to-Network Relay.

Editor's note:	Further details on the needs and usage of 5GPRUK ID are FFS.

************** End of the Change(s) ****************