3GPP TSG-SA3 Meeting #107-e 	S3-220883

- e-meeting, 16 - 20 May 2022

- Source:	Huawei, HiSilicon

- Title:	Update general clause for secondary authentication

- Document for:	Approval

Agenda Item:	4.7

# 1	Decision/action requested

This contribution is requested by the group.

# 2	References

NA

# 3	Rationale

It was agreed in the study phase that the secondary authentication is different what is defined in TS 33.501. This evaluation should be captured here.

# 4	Detailed proposal

********* Begin 1st change*********

###### 6.3.3.3.4.1		General

This clause specifies the5G Prose Remote UE specific secondary authentication between a 5G ProSe Remote UE, which is is different from the seconday authentication defined in TS 33.501[3].  via a 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and an external data network (DN) based on network-controlled authorization (i.e., using 5G ProSe Remote UE primary authentication) as described in clause 6.3.3.3.2. This procedure is optional to support.

The SMF of the 5G ProSe UE-to-Network Relay triggers the secondary authentication of the 5G ProSe Remote UE based on the subscription information and the local configuration of the SMF when it receives a NAS message (e.g., Remote UE Report) from the 5G ProSe UE-to-Network Relay.

The EAP framework specified in RFC 3748 [12] shall be used for authentication between the 5G ProSe Remote UE and a DN-AAA server in the external data network.

Following clause describes the procedures for initial secondary authentication of the 5G ProSe Remote UE with the external DN-AAA server.

********* End of change*********