3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222844-r2

- e-meeting, 10 - 14 October 2022						merger of S3-222482, S3-222582, S3-222877

- Source:	Samsung

- Title:	Key issue on ProSe multi-path transimission (redundant PDUs) for UE-to-Network relay

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

It is requested to approve the proposed key issue in TR 33.740.

# 2	References

[1]	3GPP TR 33.740 Study on security aspects of Proximity Based Services (ProSe) in 5G System (5GS) phase 2

[2]	3GPP TR 23.700-33 Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2

# 3	Rationale

This contribution proposes a new key issue for secure ProSe multi-path transimission. SA2 is currently studying ProSe phase 2 enhancement, one of the key issue (KI#5) is about support of multi-path transmission for UE-to-Network Relay. TR 23.700-33 captures various solution for the same, and solutions like solution#39, #41 considers redundant PDU sessions over multi-path scenario. Therefore, this key issue is proposed to address the security concerns that arises due to redundant transmission over multi-path.

# 4	Detailed proposal

******Start of Change******

## 5.X	Key Issue #X: Key issue on ProSe multi-path transimissionfor UE-to-Network relay

### 5.X.1	Key issue details

SA2 is studying redundant PDU sessions over multi-path transmission in ProSe phase 2 to improve reliability for the Remote UE. As illustrated in figure 5.X.1-1, a remote UE use path #1 and path #2 for multi-path transmission, where path #1 is direct network communication path, and path #2 is indirect network communication path with UE-to-Network Relay. This Key Issue is intended to cover the related security and privacy aspects of the multi-path scenario (one direct Uu communication and one indirect U2NW relay path, including L2, L3 N3IWF and L3 without N3IWF UE-to-Network relays).

Figure 5.X.1-1: Example scenario of multi-path transmission using UE-to-Network Relay

Considering redundant PDU sessions over multi-path scenario, in Layer 3 relay system, there is a risk that if one of the two interfaces (i.e., PC5 or Uu) in direct or indirect path is protected and the other is unprotected because of security policy misalignment since the security policy determining or provisioning entities are different in both paths. Such interface would become the weakest link in the end-to-end security and an adversary will be able to take advantage of such weakest link to compromise the whole L3 UE-to-Network Relay security which will lead to security vulnerability as data may be exposed in an unprotected link during the multi-path transmission.

### 5.X.2	Security threats

Figure 5.X.2-1: Example scenario of the issue in redundant multi-path transmission using UE-to-Network Relay

If dissimilar security policies are applied, the end-to-end protection of data over L3 UE-to-Network Relay (over path#2) or Uu (over path#1 link between remote UE and Network) may be compromised as same data is protected using different security algorithm and/or security policy (for example, one "not required" and another "required"). Therefore, it may lead to disclosure of sensitive data’s over one of the path.

If security and/or privacy measures are not considered and applied during and/or after the multi-path transmission establishment procedures, an attacker can track the remote UE by linking the direct and indirect communications.

### 5.X.3	Potential security requirements

The system shall support means of enabling same confidentiality and integrity protection over Uu and PC5 links in support of end-to-end security for redundant PDU sessions over multi-path transmission.

The 5G system shall protect the privacy of the Remote UE during and/or after the multi-path establishment between direct Uu and indirect UE-to-Network Relaying paths.

******End of Change******