3GPP TSG-SA3 Meeting #108e 	S3-221768_r1

- e-meeting, 22 - 26 August 2022

- Source:	Nokia, Nokia Shanghai Bell

- Title:	EAP_AKA prime based authentication for AUN3 devices

- Document for:	Approval, Information, Discussion

Agenda Item:	5.13

# 1	Decision/action requested

It is requested to approve the new solution for authentication of AUN3 devices via EAP-AKA prime.

# 2	References

[1]	3GPP TR 33.887 Study on Security aspects for 5WWC Phase 2

[2]	3GPP TS 33.501 Security architecture and procedures for 5G system

# 3	Rationale

As per KI#1 defined in TR 33.887 [1], the authentication of AUN3 devices supporting EAP(EAP_TLS, EAP_AKA') is required. In this solution proposal, we are proposing the EAP_AKA prime based enhancement for the AUN3 devices.

Please note that 33501 Annex O defines the authentication of N5GC. However, Annex O has limitations and cannot cover AUN3 devices. For example, it describes only legacy wireline devices (e.g., devices connecting to the RG using Ethernet), which don’t perform any further security procedures with the RG. I.e., interface between AUN3 device and RG is not protected.

# 4	Detailed proposal

**** START OF CHANGE ****

## 6.Y	Solution #Y: EAP_AKA prime based authentication for AUN3 devices

### 6.Y.1	Introduction

This solution addresses the authentication of AUN3 devices based on the EAP_AKA prime method.

Please note, TS 33501[2] Annex O defines the authentication of N5GC. However, Annex O has limitations. For example, it describes only legacy wireline devices (e.g., devices connecting to the RG using Ethernet), which don’t perform any further security procedures with the RG. I.e., interface between AUN3 device and RG is not protected

### 6.Y.2	Solution details

- 6.X.2.1	Procedure

Figure 6.X.2.1-1: EAP-AKA prime based AUN3 authentication

1a.	The AUN3 device establishes a WLAN connection with the WLAN Access Network (AN), using procedures specified in IEEE 802.11.

1b, 1c.	L2 connection and EAP identity retrieval are performed.. AUN3 device sends back its Network Access Identifier (NAI) in the form of username@realm.

2a, 3a, 3b, 3c.	If the RG is an FN-RG, the FN-RG sends the EAP response/Identity including the NAI to the W-AGF. The W-AGF creates a registration request on behalf of the AUN3 device with a new indication that the registration is on behalf of an AUN3 device where protection is required for the interface between the AUN3 device and RG. The SUPI of the AUN3 device is the NAI as received from the device, and the W-AGF constructs the SUCI from this SUPI using the NULL scheme. The W-AGF selects the AMF/SEAF.The W-AGF sends to the AMF/SEAF a registration request on behalf of the AUN3 device. The registration request includes the NAI SUCI, wireline network name if available, and the new indication. The same message content is forwarded from AMF to AUSF and then from AUSF to UDM.

2b, 3b, 3c.		If the RG is a 5G-RG, the 5G-RG constructs a SUCI from the NAI-based SUPI of the AUN3 device and sends a NAS Registration Request message to the AMF, including the SUCI and the new indicator for encryption required for AUN3 device.

4.	Authentication procedure for EAP-AKA' is performed as defined in the section 6.1.3.1 of TS 33.501[2].

5.	Based on the indication in step 3, AMF derives the WAGF key.

6.	The AMF sends NAS Security Mode Command mode and provides the  WAGF key to W-AGF.

7.	W-AGF/RG derive the PMK key from the WAGF key.

Note: whether the PMK is derived by RG and W-AGF is out if 3GPP scope.

8.	RG and AUN3 device will derive WLAN keys from PMK.

9.	The AUN3 device performs a 4-way handshake to establish a secure connection with the WLAN AN.

Editorial Note: KEY derivation is FFS.

### 6.Y.3	Evaluation

TBD

**** END OF CHANGE ****