3GPP TSG-SA3 Meeting #108-e 	S3-221821r1

- e-meeting, 22 - 26 August 2022

- Source:	Qualcomm Incorporated

- Title:	Proposed text for gNB-CU-CP test cases from TS 33.501 for TS 33.742

- Document for:	Approval

Agenda Item:	4.6

# 1	Decision/action requested

This contribution proposes text for gNB-CU-CP test cases from TS 33.501 for TS 33.742.

# 2	References

[1]	draft TS 33.742 v0.1.0 (S3-221201)

# 3	Rationale

This contribution contains proposed changes for the gNB-CU-CP for TS 33.742. The justification for the changes is covered in S3-221819.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft CR to TS 33.742 for gNB-CU-CP.

**** START OF CHANGES ****

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements"

[4]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	3GPP TS 38.401: "NG-RAN; Architecture description".

[aa]	3GPP TS 33.501 (Release 15): "Security architecture and procedures for 5G system".

[bb]	3GPP TS 33.511: "Security Assurance Specification (SCAS) for the next generation Node B (gNodeB) network product class".

**** NEXT CHANGE ****

- 5.2	Security functional adaptations of requirements and related test cases

- 5.2.1	Introduction

The present clause contains gNB-CU-CP-specific security functional adaptations of requirements and related test cases. Many of the security functional requirements are directly inherited from the gNB product class.

- 5.2.2	Requirements and test cases deriving from 3GPP specifications

#### 5.2.2.1	Security functional requirements on the gNB-CU-CP deriving from 3GPP specifications – TS 33.501 [aa]

Editor’s note: The threat references for all security functional need to refer to a gNB-CU-CP one once they are defined

##### 5.2.2.1.1	Security functional requirements inherited from gNB

The security functional requirements in the following clauses of TS 33.511 [bb] apply to the gNB-CU-CP by changing the gNB to gNB-CU-CP for the entity under test in the test cases:

- 4.2.2.1.1	Integrity protection of RRC-signalling;

- 4.2.2.1.4	RRC integrity check failure;

- 4.2.2.1.6	Ciphering of RRC-signalling;

- 4.2.2.1.9	Replay protection of RRC-signalling;

- 4.2.2.1.12	AS algorithms selection;

- 4.2.2.1.13	Key refresh at the gNB;

- 4.2.2.1.14	Bidding down prevention in Xn-handovers;

- 4.2.2.1.15	AS protection algorithm selection in gNB change;

- 4.2.2.1.18	Key update at the gNB on dual connectivity; and

- 4.2.2.1.19	UP security activation in Inactive scenario.

##### 5.2.2.1.2	Control plane data confidentiality protection over N2/Xn/F1/E1 interface

NOTE 1: This is based on the security functional requirement on the gNB given in 4.2.2.1.16 of TS 33.511 [bb] but modified as the gNB-CU-CP supports the F1 and E1 interface.

Requirement Name: Control plane data confidentiality protection over N2/Xn/F1/E1 interface

Requirement Reference: TS 33.501 [aa], clauses 5.3.9, 5.3.10, 9.2 and 9.4

Requirement Description: "F1-C interface shall support confidentiality, integrity and replay protection.", "The E1 interface between CU-CP and CU-UP shall be confidentiality, integrity and replay protected.", "The transport of control plane data over N2 shall be integrity, confidentiality and replay-protected." "The transport of control plane data and user data over Xn shall be integrity, confidentiality and replay-protected." as specified in TS 33.501 [aa], clauses 5.3.9, 5.3.10, 9.2 and 9.4.

Threat References: TR 33.926 [4], clause D.2.2.1 – Control plane data confidentiality protection.

Test Case: the test case in subclause 4.2.3.2.4of TS 33.117 [2]

##### 5.2.2.1.3	Control plane data integrity protection over N2/Xn/F1/E1 interface

NOTE 1: This is based on the security functional requirement on the gNB given in 4.2.2.1.17 of TS 33.511 [bb] but modified as the CU-CP supports the F1 and E1 interfaces.

Requirement Name: Control plane data integrity protection over N2/Xn/F1/E1 interface

Requirement Reference: TS 33.501 [aa], clauses 5.3.9, 9.2 and 9.4

Requirement Description: "F1-C interface shall support confidentiality, integrity and replay protection.", "The E1 interface between CU-CP and CU-UP shall be confidentiality, integrity and replay protected.", "The transport of control plane data over N2 shall be integrity, confidentiality and replay-protected." "The transport of control plane data and user data over Xn shall be integrity, confidentiality and replay-protected." as specified in TS 33.501 [aa], clauses 5.3.9, 5.3.10, 9.2 and 9.4.

Threat References: TR 33.926 [4], clause D.2.2.2 – Control plane data integrity protection.

- Test Case: the test case in subclause 4.2.3.2.4of TS 33.117 [2].

**** END OF CHANGES ****