3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	draft_S3-232038-r2

- Electronic meeting, Online, 17 - 21 April 2023								(revision of S3-233038)

- Source:	Xiaomi

- Title:	33.893: Conclusion on Key Issue #1

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed conclusion to be added for key issue #1 in TR 33.893.

# 2	References

[1]	TR 33.893 v0.6.0		Study on Security Aspects of Ranging Based Services and Sidelink Positioning

# 3	Rationale

This pCR proposes to conclude on key issue #1 of TR 33.893 [1].

# 4	Detailed proposal

*************** Start of the Change ****************

# 7	Conclusions

7.x	Conclusions on Key Issue #1

To protect UE privacy of Ranging/SL Positioning services, the following conclusions are made:

-	For privacy protection during communication:

-	For unicast direct communication over SR5 reference point,  security policy configuration for confidentiality protection is used as the basis for normative work.

-	For Ranging/SL positioning result exposure to a Client UE over PC5, the method of PC5-U security policy setting for confidentiality protection (as defined for ProSe/V2X services) is used.

-	For Ranging/SL positioning result exposure to a Client UE over Uu using enhanced MO-LR procedure, NAS security for confidentiality protection is used.

-	For Ranging/SL positioning result exposure to a Client UE over Uu using Application exposure API, Uu UP security policies for confidentiality protection can be used.

Editor’s Note: Conclusion for non-trackability is FFS.

Editor’s Note: More conclusions are FFS.

*************** End of the Changes ****************