3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	S3-232065-r1

- Electronic meeting, Online, 17 - 21 April 2023									(revision of S3-yyxxxx)

- Source:	Xiaomi

- Title:	Update KI #2.1 conclusion of TR 33.739

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is proposed to approve the pCR to TR 33.739.

# 2	References

[1]	3GPP TR 33.739 Study on Security Enhancement of Support for Edge Computing — Phase 2

# 3	Rationale

This pCR proposes to update the conclusion for KI #2.1 of TR 33.739 [1].

For the mutual authentication between EEC and ECS/EES, ECS/EES should authenticate EEC rather than the UE.

- Solutions #1, #2, and #13 assume that EEC can be authenticated by the UE. And the authentication mechanism is out of the scope of 3GPP. If EEC is authencated, UE will provide EEC specific key to the EEC.

- Solutions #1 and #2 leverage the EEC-specific key to realize authentication between EEC and EES/ECS. And the EEC-specific key is derived based on the EEC ID and NAF key of EES/ECS.

- Solution #13 leverages the EEC-specific key to realize authentication between EEC and EES/ECS. And the EEC-specific key is derived based on the EEC ID and AKMA application key of EES/ECS.

If the network cannot aware that the UE has authenticated the EEC, no solution in this TR can be employed to handle the mutual authentication issue.

# 4	Detailed proposal

*************** Start of the Change ****************

## 7.5		Conclusions for Key Issue #2.1

It is concluded to reuse authentication and authorization mechanism specified in TS 33.558 to realize authorization and mutual authentication between UE and ECS/EES.

It is concluded that mutual authentication between EEC and ECS/EES is not supported.

Server side certificate-based TLS authentication is mandatory supported.

It is concluded to clarify in the normative work that ECS/EES can retrieve the GPSI from the core network if necessary and the ECS/EES should not trust the GPSI received from the EEC/UE.

NOTE: Token-based solutions (e.g., Solution #25 for EEC/UE authentication by the ECS/EES) are left to implementation.

Editor’s Note: If no EEC/UE side authentication and authorization performed and only Server side certificate-based TLS authentication is performed, then details on restricting the privileged services and/or UE specific information that to be provided from ECS/EES to the UE to be considerd in collobration with SA6 is FFS

*************** End of the Change ****************