3GPP TSG-SA3 Meeting #110	S3-231319

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1591 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 33.501, 5WWC, Authentication of AUN3 devices behind RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs, Nokia, Nokia Shanghai Bell, Rogers Communications, Thales, Charter Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2023-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Implement the conclusion of KI#1 in TR 33.887 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of normative clauses on the authentication of AUN3 devices behind RG not supporting 5G key hierarchy |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5WWC features will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 7B.1, 7B.X (new), Annex Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st CHANGE ***

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5G-AN	5G Access Network

5G-RG	5G Residential Gateway

NG-RAN	5G Radio Access Network

5G AV	5G Authentication Vector

5G HE AV	5G Home Environment Authentication Vector

5G NSWO	5G Non-Seamless WLAN Offload

5G SE AV	5G Serving Environment Authentication Vector

ABBA	Anti-Bidding down Between Architectures

AEAD	Authenticated Encryption with Associated Data

AES	Advanced Encryption Standard

AKA	Authentication and Key Agreement

AMF	Access and Mobility Management Function

AMF	Authentication Management Field

NOTE:	If necessary, the full word is spelled out to disambiguate the abbreviation.

ARPF	Authentication credential Repository and Processing Function

AUN3	Authenticable Non-3GPP Devices

AUSF	Authentication Server Function

AUTN	AUthentication TokeN

AV	Authentication Vector

AV'	transformed Authentication Vector

BAP	Backhaul Adaptation Protocol

BH	Backhaul

CCA	Client Credentials Assertion

Cell-ID	Cell Identity as used in TS 38.331 [22]

CH	Credentials Holder

CHO	Conditional Handover

CIoT	Cellular Internet of Things

cIPX	consumer's IPX

CKSRVCC	Cipher Key for Single Radio Voice Continuity

cNRF	consumer's NRF

CP	Control Plane

cPLMN	consumer's PLMN

cSEPP	consumer's SEPP

CTR	Counter (mode)

CU	Central Unit

DCS	Default Credentials Server

DN	Data Network

DNN	Data Network Name

DU	Distributed Unit

EAP	Extensible Authentication Protocol

EDT	Early Data Transmission

EMSK	Extended Master Session Key

EN-DC	E-UTRA-NR Dual Connectivity

ENSI	External Network Slice Information

EPS	Evolved Packet System

FN-RG	Fixed Network RG

gNB	NR Node B

GUTI	Globally Unique Temporary UE Identity

HRES	Hash RESponse

HXRES	Hash eXpected RESponse

IAB	Integrated Access and Backhaul

IKE	Internet Key Exchange

IKSRVCC	Integrity Key for Single Radio Voice Continuity

IPUPS	Inter-PLMN UP Security

IPX	IP exchange service

KSI	Key Set Identifier

KSISRVCC	Key Set Identifier for Single Radio Voice Continuity

LI	Lawful Intercept

MBSF	Multicast/Broadcast Service Function

MBSTF	Multicast/Broadcast Service Transport Function

MeNB	Master eNB

MN	Master Node

MO-EDT	Mobile Originated Early Data Transmission

MT-EDT	Mobile Terminated Early Data Transmission

MR-DC	Multi-Radio Dual Connectivity

MSK	Master Session Key

N3IWF	Non-3GPP access InterWorking Function

NAI	Network Access Identifier

NAS	Non Access Stratum

NDS	Network Domain Security

NEA	Encryption Algorithm for 5G

NF	Network Function

NG	Next Generation

ng-eNB	Next Generation Evolved Node-B

ngKSI	Key Set Identifier in 5G

N5CW	Non-5G-Capable over WLAN

N5GC	Non-5G-Capable

NIA	Integrity Algorithm for 5G

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NSSAI	Network Slice Selection Assistance Information

NSSAA	Network Slice Specific Authentication and Authorization

NSWO	Non-Seamless WLAN Offload

NSWOF	Non-Seamless WLAN Offload Function

PDN	Packet Data Network

PEI	Permanent Equipment Identifier

pIPX	producer's IPX

pNRF	producer's NRF

pPLMN	producer's PLMN

PRINS	PRotocol for N32 INterconnect Security

pSEPP	producer's SEPP

PUR	Preconfigured Uplink Resource

QoS	Quality of Service

RES	RESponse

SCG	Secondary Cell Group

SEAF	SEcurity Anchor Function

SCP	Service Communication Proxy

NOTE: Void.	Security Gateway

SEPP	Security Edge Protection Proxy

SgNB	Secondary gNB

SIDF	Subscription Identifier De-concealing Function

SMC	Security Mode Command

SMF	Session Management Function

SN	Secondary Node

SN Id	Serving Network Identifier

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TLS	Transport Layer Security

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

TSC	Time Sensitive Communication

UE	User Equipment

UEA	UMTS Encryption Algorithm

UDM	Unified Data Management

UDR	Unified Data Repository

UIA	UMTS Integrity Algorithm

ULR	Update Location Request

UP	User Plane

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

USIM	Universal Subscriber Identity Module

XRES	eXpected RESponse

*** 2nd CHANGE ***

# 7B	Security for wireline access to the 5G core network

## 7B.1	General

To support Wireless and Wireline Convergence for the 5G system, two new network entities, 5G-RG and FN-RG, are introduced in the architecture specificaction TS 23.501[2].

The 5G-RG acts as a 5G UE and can connect to 5GC via wireline access network (W-5GAN) or via Fixed Wireless Access (FWA). Existing security procedures defined in this document are reused. The 5G-RG also acts as end point of N1 and provides the NAS signalling connection to the 5GC on behalf of the AUN3/N5CW devices behind the 5G-RG.

The FN-RG can connect to 5GC via wireline access network (W-5GAN). The W-AGF performs the registration procedure on behalf of the FN-RG and N5GC and AUN3 devices behind the FN-RG. It acts as end point of N1 and provides the NAS signalling connection to the 5GC on behalf of the FN-RG.

A 5G -capable UE can connect to 5GC through an RG that’s connected to the 5GC via wireline access network (W-5GAN) or NG-RAN. The UE supports untrusted non-3GPP access and/or trusted non-3GPP access.

NOTE: Roaming of FN-RG and 5G-RG are not supported.

NOTE: Informative Annex O provides an example of how N5GC devices behind a residential gateway in private or in isolated scenarios with wireline access make use of additional EAP methods for authentication.

*** 3rd CHANGE ***

## 7B.X	Authentication for AUN3 devices behind 5G-RG and FN-RG

An AUN3 device behind 5G-RG or FN-RG is registered to the 5GC by the 5G-RG or the W-AGF respectively and is authenticated by 5GC using EAP-AKA’, as specified in RFC 5448 [12].

Figure 7B.X-1 Authentication Procedure for AUN3 devices using EAP-AKA’

1.  The AUN3 device initiates a layer 2 connection with the RG (5G-RG or FN-RG) either via Ethernet or WLAN. If the layer 2 connection is based on Ethernet, steps 20-21 are skipped.

2.	The RG (5G-RG or FN-RG) shall initiate the EAP authentication procedure by sending an EAP request/Identity to the AUN3 device in a layer 2 frame (e.g., EAPOL).

3.  The AUN3 device shall send back an EAP response/Identity including its Network Access Identifier (NAI) in the form of username@realm.  The username part of the NAI may be encrypted if the AUN3 device supports SUPI protection.

4a-4b.	If the RG is an FN-RG, the FN-RG shall send to the W-AGF an AAA message including the NAI of the AUN3 device. The W-AGF constructs a SUCI from NAI-based SUPI using NULL scheme and sends a NAS Registration Request message to the AMF, including the SUCI and AUN3 device indicator.

4c.  If the RG is a 5G-RG, the 5G-RG shall construct a SUCI from the NAI-based SUPI of the AUN3 device and send a NAS Registration Request message to the AMF, including the SUCI of the AUN3 device and an AUN3 device indicator.

5.	The AMF/SEAF shall select the AUSF based on the SUCI in the received registration request and send to the AUSF a Nausf_UEAuthentication_Authenticate Request message, including the SUCI of the AUN3 device and an AUN3 device indicator.

6. The AUSF shall send to the UDM a Nudm_UEAuthentication_Get Request message, including the SUCI of the AUN3 device and the AUN3 device indicator.

7. Upon reception of the Nudm_UEAuthentication_Get Request, the UDM shall invoke the SIDF to map the SUCI to the SUPI and select an authentication method based on the SUPI and the AUN3 device indicator. When the "username" part of the SUPI is "anonymous" or omitted, the UDM may select an authentication method based on the “realm” part of the SUPI, the AUN3 device indicator, a combination of the "realm" part and the AUN3 device indicator, or the UDM local policy. When EAP-AKA´ authentication method is selected, the UDM/ARPF shall generate an authentication vector using the Access Network Identity as the KDF input parameter.

8. The UDM shall send to the AUSF a Nudm_UEAuthentication_Get Response messag, including the EAP-AKA’ authentication vector (RAND, AUTN, XRES, CK´ and IK´) and the SUPI.

9. The AUSF shall store XRES for future verification. The AUSF shall send the EAP-Request/AKA'-Challenge message to the AMF/SEAF in a Nausf_UEAuthentication_Authenticate Response message.

10a-10b. The AMF/SEAF shall send the EAP-Request/AKA'-Challenge message to the RG via the W-AGF if the RG is the FN-RG. The EAP-Request/AKA'-Challenge message is carried in the NAS Authentication Response message from the AMF/SEAF to the W-AGF, and in the AAA message from the W-AGF to the RG.

10c. The AMF/SEAF shall send the EAP-Request/AKA'-Challenge message to the RG in the NAS Authentication Response message if the RG is the 5G-RG.

11. The RG shall send to the AUN3 device the EAP-Request/AKA'-Challenge message encapsulated in an layer 2 (L2) message.

12. At receipt the EAP-Request/AKA'-Challenge message, the AUN3 device shall verify the message, generate the authentication response, and derive keys as described in RFC 5448[12].

13. The AUN3 device shall send the EAP-Response/AKA'-Challenge message to the RG, encapsulated in a layer 2 message.

14a-14b. If the RG is an FN-RG, the FN-RG shall send to the W-AGF the EAP-Response/AKA'-Challenge message in a AAA message. The W-AGF shall send to the AMF/SEAF the EAP-Response/AKA'-Challenge message in an NAS Authentication Request message.

14c. If the RG is an 5G-RG, the 5G-RG shall send to the AMF/SEAF the EAP-Response/AKA'-Challenge message in an NAS Authentication Request message.

15. The AMF/SEAF shall send to the AUSF the EAP-Response/AKA'-Challenge message in an Nausf_UEAuthentication_Authenticate Request message.

16. The AUSF shall verify the AKA’-Challenge message as described in RFC 5448[12]. If successful, the AUSF shall generate the MSK as described in RFC 5448[12].  Based on the AUN3 device indicator received in step 5. The AUSF shall not generate the KAUSF.

17. The AUSF shall send to the AMF/SEAF an Nausf_UEAuthentication_Authenticate Response message including the EAP-Success, the MSK, and the SUPI.

18a-18b.	If the RG is an FN-RG, the AMF/SEAF sends to the W-AGF the EAP-Success message and the MSK in an Authentication Result message. The W-AGF sends to the FN-RG the EAP-Success message and the MSK in AAA message.

18c.  If the RG is a 5G-RG, the AMF/SEAF sends to the 5G-RG the EAP-Success message and the MSK in an Authentication Result message.

19.	 The RG sends to the AUN3 device the the EAP-Success message in a layer 2 frame.

20a-20b. If the layer 2 connection is over WLAN (IEEE 802.11), the AUN3 device and the RG use the first 256-bit of the MSK as the PMK, from which the WLAN keys are derived.

21.  The AUN3 and the RG performs four-way handshaking to establish WLAN secure connection.

*** 4th CHANGE ***

# Annex Y (normative): Authentication of AUN3 devices behind 5G-RG and FN-RG using additional EAP methods

## Y.1		General

This annex describes the authentication procedure for AUN3 devices behind Residential Gateways (RGs) in private networks or in isolated deployment scenarios (i.e., roaming is not considered) using any key generating EAP method.

An AUN3 device may be authenticated by 5GC or a Credential Holder using a AAA server.

## Y.2		Authentication of AUN3 devices by 5GC

Figure Y.2-1 Authentication Procedure for AUN3 devices by 5GC using key-generating EAP method

This authentication procedure is largely the same as clause 7B.X but differs in some steps.

Steps 1-6 are the same as steps 1-6 in clause 7B.X.

7.  The UDM shall select a key-generating EAP method. If EAP-AKA’ is selected, the UDM shall generate EAP-AKA’ authentication vector.

8. The UDM shall send to the AUSF a Nudm_UEAuthentication_Get Response message, including the SUPI and EAP-AKA’ authentication vector if EAP-AKA’ is selected or the selected authentication method if other key generating EAP method (e.g., EAP-TLS, EAP-TTLS, etc) is selected.

9. The AUN3 device and the AUSF performs the selected EAP authentication method.

Steps 10-15 are the same as steps 17-22 in clause 7B.X.

## Y.3		Authentication of AUN3 devices by AAA server

Figure Y.3-1 Authentication Procedure for AUN3 devices by AAA using key-generating EAP method

This authentication procedure is based on clause 7B.X and I.2.2.2.2.

Steps 1-6 are the same as steps 1-6 in clause 7B.X.

Steps 7-16 are the same as steps 4-13 in clause I.2.2.2.2.

Steps 17-22 are the same as steps 17-22 in clause 7B.X.

*** END OF CHANGES ***