3GPP TSG-SA3 Meeting #112 	S3-234024

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 33.501 | CR | 1776 | rev | <Rev#> | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving ENs in HONTRA Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | HN_Auth |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Living document of HONTRA (S3-233440-r1) is used as base for this CR. The HONTRA procedure have the following ENs in Clause 6.1.X.2. Editor’s Note: The factor the AAnF considers to request UDM for primary authentication is FFS, as AAnF request(s) should not lead to signalling overload as well as the AF key expiry/refresh issue should be handled. Editor’s Note: The name of notification message between AMF and UDM is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications were provided to resolve the 2 ENs. Regarding EN 1: It was already agreed in the SA3#111 that the authentication policy will be the main aspect which is used for all determination aspects involved in HONTRA procedure including the issue mentioned in this EN. So, a related clarification is provided. Regarding EN2, the figure 6.1.x.2-1 Home Network triggered primary authentication procedure, step 2 and 3 already provides clarification related to the EN2. So, the EN clarifications aligns with the already agreed and existing aspects. Also fixes some editorial errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Repeated authentication run for each and every AF key expiry i.e., with no optimal AF key expiration setting can lead to signalling overload issue. If the 2nd EN is not resolved, the HONTRA procedure will lack details on the notification message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.X.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Change ****

### 6.1.X	Home network triggered primary authentication procedure

#### 6.1.X.1	General

The support of Home Network triggered authentication is optional for the HN and the SN. If both the networks (HN and SN) support Home Network triggered primary authentication, the following clauses apply.

#### 6.1.X.2	Security mechanisms

The UDM may initiate primary authentication based onprocedures initiated by the UE (e.g. UE registration in 5GC) or towards the UE (e.g. SoR/UPU) or events from other NFs, considering the local policy into account as well.

Figure 6.1.x.2-1 Home Network triggered primary authentication procedure

0a.[optional] The UDM may be pre-configured with an operator authentication policy in order to determine when to trigger a primary authentication procedure.

0b.	The UE registers to the network. As part of the registration, the serving AMF registers the UE with the UDM via the Nudm_UECM_Registration as per TS 23.502 [8], clause 4.2.2.2.2. The UDM shall create an implicit subscription for the serving AMF for the UDM to later notify the AMF for potential re-authentication.

1. A prerequisite for the initiation of Home Network Triggered Primary Authentication is that the UDM already has the information about the AMF serving the UE. Otherwise, the UDM cannot contact any AMF in later steps.

The UDM decides itself based on events (e.g., SoR/UPU or if AAnF requests) or authentication policy and performs home network triggers primary authentication as described in the following steps. The AAnF sets the AF key expiry based on operator’s local authentication policy as described in TS 33.535 (e.g., in such a way that AF key is used for a maximum time same as primary authentication’s usage time as per operator’s authentication policy to prevent repeated AF key expiry). The AAnF considers the operator’s local authentication policy to send Nudm_UECM_AuthTrigger request to the UDM for primary authentication using the UDM services as described in clause 14.2.X.

NOTE: As HONTRA procedure considers operator’s local authentication policy to initiate primary (re-) authentication, and similarly as the AF key expiry is also set based on the operator’s authentication policy, when the AF key is about to expire, the UDM would have already initiated or run a primary authentication. No frequent AF key expiry can happen and so there is no risk of signalling overload. Only in the remote case if the AAnF finds that KAF remain unchanged since the latest primary authentication (as described in TS 33.535 Clause 6.4.3) and if it expires, the AAnF can request the UDM to trigger a primary authentication (i.e., to refresh the AF key eventually) based on Clause 14.2.X.

Based on a received event and the local operator authentication policy, if there is no ongoing primary authentication for the UE, and if the UDM determines to trigger the primary authentication, the UDM determines the serving AMF/SEAF of the target UE.

If there are different AMFs registered in the UDM for different access, and the procedure defined in 6.1X is supported, the UDM shall select one AMF to perform the reauthentication. The criteria for selecting the AMF are dependent of the local UDM authentication policy.

NOTE 1:	The reasons for the UDM determining that the UE needs to be authenticated can be different. For example, the UDM can determine to initiate a primary authentication when the AMF registers the UE upon the Registration procedure during the mobility from EPC or when SoR/UPU counters are about to wrap around, or when required based on authenticationpolicy, or based on the request from AAnF. The UDM behaviour is determined by operator policy which takes into account the support of certain features in the PLMN. For example, if the PLMN does not support the SoR/UPU feature, then SoR/UPU counter wrap around will not happen, and primary authentication will not be required for this case.

2. The UDM sends an (re-) authentication notification message to the AMF/SEAF with the UE’s SUPI.

3. After receiving the (re-) authentication notification  message from the UDM, the AMF/SEAF shall decide whether run the primary authentication procedure based on its own local authentication policy, and the UE state (e.g. if the UE is under handover, a similar way as some steps in Network Triggered Service Request (TS 23.502 [8], clause 4.2.3.3) can be reused, or if the UE is already under authentication by the AMF before receiving the authentication notification from the UDM). If the AMF/SEAF cannot run a primary authentication as described in steps 4, the AMF/SEAF sends the authentication response message to the UDM with a failure cause else it acknowledges the request. If the AMF/SEAF acknowledged the request but the AMF/SEAF is not able to initiate the primary authentication towards the UE (e.g., if UE is not reachable), the AMF/SEAF shall set the authentication pending flag in the UE context.

When UE re-attaches to the same AMF or becomes reachable, the AMF checks the authentication pending flag and performs the reauthentication if needed. Once UE reauthentication is done, the AMF resets the authentication pending flag. If the UE attaches to a new AMF (e.g., in a mobility scenario), then the new AMF may retrieve the UEContext from the previous AMF, determine that authentication is pending for the UE, and perform reauthenticationUpon receiving a failure from the AMF, the UDM may check if another AMF is available over the other access. If available, the UDM may select another AMF and send an authentication request.

Editor’s Note: If UDM has to send authentication requests to 2 AMFs, then how can it ensure double authentication is not performed? Therefore, upon receiving the successful authentication from one AMF, sending a cancellation authentication may be required. These aspects are FFS. While the authentication is pending, whether the UDM selects another AMF after receiving an acknowledgement is FFS. Requirement of authentication cancelation is also to be rechecked.

4. The AMF/SEAF starts the primary authentication procedure as defined in clause 6.2.1 of the present document.

The UDM may execute other procedures (e.g., SoR/UPU) depending on the reason that motivated the UDM triggered (re-)authentication procedure in step 1.

**** End of Change ****