3GPP TSG-SA3 Meeting #112 	S3-234338

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | 0122 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the 5G PKMF service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Prose |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the UE-to-Network Relay discovery, the Model A procedure and Model B procedure are defined in clause 6.1.3.2.2 of TS 33.503. In order to provide the discovery security materials, the 5G PKMFs in different PLMNs shall exchange the Authorization Req/Resp message. However, no related service operations are defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the service operation supported by the 5G PKMF according to the Model A and Model B procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No related service operation is supported by the 5G PKMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1 and 7.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

7.2	5G PKMF Services

7.2.1	General

For UE-to-Network discovery, the 5G PKMF supports the authorization request from the 5G PKMF in another PLMN via the new service Npkmf_Discovery.The 5G PKMF supports the key request from another 5G PKMF in another PLMN via the new service operation Npkmf_PKMFKeyRequest_ProseKey. The 5G PKMF also provides Remote User ID of a 5G ProSe Remote UE to be used in Remote UE Report and supports resolving Remote User ID to SUPI.

Table 7.2.1-1 shows the services exposed by 5G PKMF supporting 5G ProSe.

Table 7.2.1-1: 5G ProSe Services provided by 5G PKMF

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Npkmf_Discovery | AnnounceAuthorize | Request/Response | 5G PKMF |
|  | MonitorKey | Request/Response | 5G PKMF |
|  | DiscoveryKey | Request/Response | 5G PKMF |
| Npkmf_PKMFKeyRequest | ProseKey | Request/Response | 5G PKMF |
| Npkmf_ResolveRemoteUserId | Npkmf_ResolveRemoteUserId_Get | Request/Response | SMF, 5G PKMF |

*************** Start of the 2nd Change ****************

7.2.x	Npkmf_Discovery service

7.2.x.1	Npkmf_Discovery_AnnounceAuthorize service operation

Service operation name: Npkmf_Discovery_AnnounceAuthorize

Description: The consumer NF obtains the authorization from the 5G PKMF for announcing in the PLMN.

Input, Required: User Info ID, RSC.

Input, Optional: None.

Output, Required: Authorization result.

Output, Optional: None.

7.2.x.2	Npkmf_Discovery_MonitorKey service operation

Service operation name: Npkmf_Discovery_MonitorKey

Description: The consumer NF obtains the discovery key from the 5G PKMF for monitoring in the PLMN.

Input, Required: User Info ID, RSC, PC5 UE security capability.

Input, Optional: None,

Output, Required: The chosen PC5 ciphering algorithm, discovery security materials.

Output, Optional: Discovery User Integrity Key (DUIK).

7.2.x.3	Npkmf_Discovery_DiscoveryKey service operation

Service operation name: Npkmf_Discovery_DiscoveryKey

Description: The consumer NF obtains the discovery key from the 5G PKMF for a discoverer UE in the PLMN to operate Model B restricted discovery.

Input, Required: User info ID, RSC, PC5 UE security capability.

Input, Optional: None.

Output, Required: The chosen PC5 ciphering algorithm, discovery security materials.

Output, Optional: Discovery User Integrity Key (DUIK).

*************** End of the Change ****************