3GPP TSG-SA3 Meeting #115AdHoc-e	draft_S3-241142-r54

Electronic meeting, online, 15 - 19 April 2024

- Source:	Lenovo

- Title:	Solution to address KI#3

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

The contribution proposes a solution to address KI#3 in TR 33.757.

# 2	References

[1]	3GPP TR 33.757, ‘Study on security for PLMN hosting a NPN’, Release 19.

[2]	TS 33.501, ‘Security architecture and procedures for 5G System’, Release 18.

# 3	Rationale

TR 33.757 has a KI#3 on SUPI privacy issue in PLMN hosting NPN scenario. This contribution proposes a solution to address the KI#3.

# 4	Detailed proposal

SA3 is kindly requested to agree the pCR below to TR 33.757.

*****Start of Change 1*****

## 7.Y	Solution #Y: SUPI privacy protection in hosted NPN

### 7.Y.1	Introduction

This Solution address KI#3.

### 7.Y.2	Solution details

The solution uses the Primary authentication procedure described in TS 33.501 Clause 6.1 [3] as the baseline with the following adaptations.

- The authentication initiation steps are same as in TS 33.501 Clause 6.1.2.

- The UDM following the SUCI deconcealment and authentication method selection, based on Operator managed SUPI usage restriction policy, UDM/UDR fetches an identifier e.g., a privacy protected identifier related to the SUPI assigned by the operator based on operator’s local policy. Further aspects on if a new identifier (like serving network UE identifier/temporary identifier) is used or an existing identifier like GPSI containing external identifier defined in TS 23.003 reusage is sufficient are upto the normative discussions. A SUPI usage restriction policy can indicate if a SUPI usage is allowed/not for the UE context management external to operator’s security domain/network domainduring a hosted NPNs or serving network access.

- The UDM provides GPSI additionally along with SUPI usage restriction indication to the AUSF in authentication response.

NOTE: For secure transfer of SUPI across different security domains to facilitate kamf generation and to not impact UE, suitable transport security can be applied as described in Solution#X in this present document.

- The AUSF and UE exchange selected method specific authentication message based on TS 33.501 [3].

- Following a successful verification of the response, based on SUPI usage restriction indication the AUSF sends received GPSI along with SUPI and SUPI usage restriction indication to the SEAF.

- The SEAF following a successful Kamf derivation, it deletes SUPI and uses GPSI instead of SUPI for further UE context and subscription data management (e.g., for any Nudm service operation). If the AMF/SEAF wants to initiate primary authentication, it follows TS 33.501 [3] clause 6.12.4 and clause 6.1.2.

- 7.Y.3	Evaluation

Editor’s Note: Evaluation is FFS.

*****End of Change 1*****