3GPP TSG-SA3 Meeting #120	S3-250368

Athens, Greece, 17 -21 February 2025

# Source:	KDDI

# Title:	New SID on use of AEAD

# Document for:	Approval

# Agenda Item:	6 New Study/Work Item Proposals

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on supporting AEAD algorithms for next generation system

Acronym: tbd

Unique identifier:	tbd

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

# 3	Justification

ETSI SAGE and 3GPP SA3 have recently completed specifications for 256-bit cryptographic algorithms. For the very first time in 3GPP, these specifications also include Authenticated Encryption with Associated Data (AEAD) algorithms [1][2][3]. With the industry’s increasing focus on higher throughput and data-intensive applications, SA3 should also consider adoption of these AEAD algorithms to be used for NAS and AS security (including control and user plane security) in a next-generation 3GPP System.

Cryptographic algorithms operating in AEAD mode combine both encryption and integrity protection to a single operation. Generally speaking, this approach has two main benefits:

- Enhanced throughput and power consumption

  - Single-pass operation: Ideally, they provide data encryption and authentication in a single pass (in contrast to the separate ciphering and integrity protection algorithms used today).

  - Reduced power consumption: There is data that suggests that, for smaller payloads, AEAD algorithms can exhibit reduced power consumption compared to other symmetric key schemes that generate the MAC / authentication tag separately [5].

  - Enhanced throughput: When conducting both encryption and integrity, it is observed AEAD mode produces better efficiency compared to legacy mode encryption and integrity protection combined. [4]

- Simplified system design and key management

  - Fewer algorithms to implement: As encryption and integrity protection can be achieved by a single algorithm, the number of algorithms to be implemented can be reduced.

  - Fewer keys to manage: With AEAD, only one key is needed for both encryption and integrity protection, simplifying key management.

  - Less Error-Prone: Using separate algorithms can introduce complexity (e.g. security policy complexity) and potential errors in their combination. AEAD reduces the risk of such errors by providing a single, well-defined process.

Reference:

[1]	TS 35.240 Specification of the Snow 5G based 256-bits algorithm set

[2]	TS 35.243 Specification of the AES based 256-bits algorithm set

[3]	TS 35.246 Specification of the ZUC based 256-bits algorithm set

[4]	S3-24XXYY Study on Use of AEAD in Next-Generation 3GPP System

[5]	Power Consumption of Common Symmetric Encryption Algorithms on Low-Cost Microchips; Marc Ohm, Lars Taufenbach, Karsten Weber, Timo Pohl

# 4	Objective

This study aims to identify key issues and solutions for supporting AEAD algorithms for NAS and AS security (including control and user plane security) in the next-generation 3GPP System, including the following:

- Studying key issues and candidate solutions concerning the AEAD1 operating mode, such as:

  - Studying whether the current order of encryption and integrity protection of AEAD1 (Encrypt-then-MAC) is suitable for AS security, which currently uses MAC-then-Encrypt.

  - If the Mac-then-Encrypt order is to be maintained for AS security, studying how AEAD1 can support this while achieving adequate performance.

- Studying key issues and candidate solutions concerning the adoption of AEAD algorithms in the next-generation 3GPP system:

  - Key hierarchy and management to support AEAD algorithms

  - Negotiation of encryption and/or integrity protection when using AEAD algorithms

  - Creation and handling of AEAD algorithm inputs, such as Nonce and Associated Data

  - Backwards compatibility between AEAD-compatible systems and legacy deployments

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {e.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {e.g. "TSG#87"} | {e.g. "TSG#89"} | {<FamilyName>, <GivenName>, <Company>, <email address>. See Note 2} |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {e.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {e.g. "TSG#89"} | {Free text, e.g. "This TS covers Stage 2" or "This TS covers Stage 3" or "This TS covers both stages 2 and 3"} |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

# 8	Aspects that involve other WGs

For a Stage 2 WID requiring Stage 3 to be done by another group: on a best-effort basis, indicate which potential WG is expected to specify the Stage 3: {possible values: "Not applicable", " unknown", "CT WGs", etc}

# 9	Supporting Individual Members

Supporting IM name

KDDI

Apple

CableLabs

ZTE

Samsung