3GPP TSG-SA3 Meeting #120	S3-250357

# Athens, Greece, 17 - 21 February 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Pseudo-CR on proposed text for normative clause

Document for:	Approval

Agenda item:	4.12

Spec:	3GPP TS/TR <TS/TR number>

Version:	<TS version>

Work Item:

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

* * * First Change * * * *

#### 6.X.Y	Security mechanism and procedures for inter-CU LTM

Editor’s Notes: This clause contains the security procedure for inter-CU LTM in 5G.

In the case where CU is acting as SN and MN is unchanged, the security mechanism and procedures for SCPAC as specified in clause 6.10.2.4 apply.

Figure 6.x.y-1 Procedure for inter-CU LTM

LTE preparation phase:

1. The UE has already connected to the source gNB and is in UE connected state. The AS and NAS security has been activated and are applicable to traffic that is going between UE and Network.

2. The UE is sending measurement report.

3. The source gNB is preparing the LTM cell switch candidates, i.e., is computing the keys per candidate target gNB.

Horizontal Key Preparation:

Compute KgNB* = KNG_RAN* = (targetPCI, ARFCN, KgNB)

Example: { horiz._KgNB*_1, KgNB}

{ horiz._KgNB*_2, KgNB}

In case of Vertical Key Preparation:

Vertical if unused {NH, NCC} is available: Compute KgNB* = KNG_RAN* = (targetPCI, ARFCN, NH)

Example: { vert._KgNB*_1, NCC=2 }

{ vert._KgNB*_2, NCC=3 }, { vert._KgNB*_3, NCC=4 }

(Alt1) 4./4a./5. The source gNB is triggering handover preparation procedure over XnAP. The source gNB is sending to every of the candidates the corresponding AS security information (i.e., Key NG_RAN* and NCC). The XnAP handover preparation procedure is according to [4]. The sharing of the AS security information will enable the target gNB to run horizontal key derivation at handover unless otherwise requested.

Rationale: The Handover Request message as specified by [4] is providing the capability for the transfer of the KgNB* = KNG-RAN* and the corresponding NCC value. This message can be used to transfer the computed keys independent whether there will be horizontal or vertical key derivation. In case of a radio link failure recovery the UE selects a cell to recover which belongs to a different LTM prepared target in this case if the target has the security keys it can decode the RRC reconfiguration complete.

6./7. The source gNB is triggering RRC reconfiguration procedure and is includuding the next NCC value (e.g., in case of horizontal keying, NCC=1).

LTM cell switch execution phase:

8. The UE is periodically sending measurements.

9./10. The source gNB is deciding for inter-CU LTM cell switch and is sending a MAC CE cell switch command to the UE.

11. The UE is generating new keys based on NCC indication. Depending on the stored next NCC value UE will do vertical or horizontal key derivation (in this example with NCC=1 the UE will do horizontal keying.).

(Alt2): 12./12a. The source gNB is sending to the target gNB the AS security context related information. The target gNB is receiving the AS security context related information on demand/need basis.

13. The UE is sending a reconfiguration complete to the target gNB.

14./15. The target gNB is sending a NGAP path switch towards the AMF for the generation of a new { NH, NCC } pair and is receiving the acknowledge message.

16. The new RRC message includes the NCC and the KSCI information elements.

* * * End of Changes * * * *