3GPP TSG-SA3 Meeting #120	S3-250404

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on inventory of 5G security protocols

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.938

Version:	0.0.0

Work Item:	FS_CryptoInv

Comments

This contribution provides inventory of security protocols used in 5G systems for TR 33.938.

* * * First Change * * * *

## 4.x	Security protocol inventory by features

This clause provides inventory of security protocols used in 3GPP specifications for the 5G System (limited to the standalone mode).

Table 4.x-1 Inventory of security protocols in 5G systems (SA)

|  | 3GPP Specification, profile, clause | Protocols/ cryptography used | External specification | Type of cryptography | Cryptography property (i.e. “usage type”) |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE Authentication and key agreement (e.g. primary authentication, secondary authentication, NSSAA, NPN authentication etc.) | TS 33.501 (6.1.3.1) | 5G AKA | 3GPP internal | symmetric | Authentication |
|  | TS 33.501 (6.1.3.1) TS 35.206 TS 35.231 | MILEANAGE/TUAK | 3GPP internal | symmetric | Authentication, Integrity |
|  | TS 33.501 (6.1.3.2) | EAP-AKA’ | IETF RFC 5448 IETF RFC 9048 | symmetric | Authentication |
|  | TS 33.501 (6.1.1.2, 11.1, 16.3) | EAP framework | IETF RFC 3748 | symmetric and asymmetric | Authentication |
|  | TS 33.501 (Annex B) | EAP-TLS | IETF RFC 5216 | symmetric and asymmetric | Authentication |
| UE NAS security/AS security | TS 33.501 (Annex D) | AES 128 | NIST (AES) FIPS PUB 197 | symmetric | Integrity, Confidentiality |
|  | TS 33.501 (Annex D) TS 33.215 | SNOW 3G | 3GPP internal | symmetric | Integrity, Confidentiality |
|  | TS 33.501 (Annex D) TS 33.221 | ZUC 128 | 3GPP internal | symmetric | Integrity, Confidentiality |
| UE SUPI Privacy | TS 33.501 (Annex C) | Elliptic Curve Integrated Encryption Scheme (ECIES) | Standards for Efficient Cryptography Group (SECG) | Symmetric and asymmetric | Integrity, Confidentiality |
| NF/MF Authentication and authorization (e.g. SBI security, NEF Security, Slice Management Security etc.) | TS 33.501 (clauses 12, 13, 15) | TLS | IETF RFC 8446 | symmetric and asymmetric | Authentication, Integrity, Confidentiality |
|  | TS 33.501 (clauses 12, 13, 15) | HTTPS | IETF RFC 9113 (HTTP2) IETF RFC 2818 (HTTP over TLS) | symmetric and asymmetric | Authentication, Integrity, Confidentiality |
|  | TS 33.501 (clauses 12, 13, 15) | OAuth2.0 | IETF RFC 6749 | symmetric and asymmetric | Authorization |
| Network entity/interface protection (e.g. NDS, security for non-3GPP access etc.) | TS 33.501 (clauses 7, 9) | IPsec | IETF RFC 4301 | symmetric and asymmetric | Authentication, Integrity, Confidentiality |
|  |  | IKEv2 | IETF RFC 7296 | symmetric and asymmetric | Authentication, Integrity, Confidentiality |
|  |  | DTLS | IETF RFC 6083 | symmetric and asymmetric | Authentication, Integrity, Confidentiality |
|  |  | DNS over TLS | IETF RFC 7858/ 8310 | symmetric and asymmetric | Authentication, Integrity, Confidentiality |
| Security profiles and certificate management | TS 33.210 TS 33.310 TS 33.501 | Certificate X.509v3 | IETF RFC 5280 | asymmetric | Authentication, Integrity, Confidentiality |
|  |  | CRL | IETF RFC 5280 | asymmetric | Revocation |
|  |  | OCSP | IETF RFC 6960 | asymmetric | Revocation |
|  |  | PKCS #10 | IETF RFC 2986 | asymmetric | Authentication, Integrity, Confidentiality |
|  |  | CMPv2 | IETF RFC 4210/ 4211/6712/ 9480/ 9481 | asymmetric | Authentication, Integrity, Confidentiality |
|  |  | JOSE | IETF RFC 7518 | asymmetric | Authentication, Integrity, Confidentiality |
|  |  | JWS | IETF RFC 7515 | asymmetric | Authentication, Integrity, Confidentiality |
|  |  | JWE | IETF 7516 | asymmetric | Authentication, Integrity, Confidentiality |
|  |  | JWA | IETF RFC 7518 | asymmetric | Authentication, Integrity, Confidentiality |

* * * End of Changes * * * *