3GPP TSG-SA3 Meeting #120	S3-250476

Athens, GREECE, 17-21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security solution for interconnect |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Interconnect was not yet addressed in Rel-18 CAPIF security spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding solution in line with conclusion – based on solution 17 authentication, authorization and offboarding. Revocation – based on solution 4. Security method retrieval and API invoker ownership – based on solution 12 and 14. Authentication token request handling – based on solution 15. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No security solution for CAPIF-6/6e |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new), 6.X.1 (new), 6.X.2 (new), 6.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******* START OF CHANGES

## 6.X	Secure interconnect over CAPIF-6/6e reference points

### 6.X.1	General

Authentication and Authorization using Method 3 as specified in subclause 6.5.2.3 is applied in the CAPIF interconnection scenario.

Security aspects related to offboarding of API invokers in interconnect scenarios is addressed.

### 6.X.2	Authentication and authorization in interconnect

The API invoker onboarded to the CCF B is enabled to obtain the service API invocation authorization from the CCF A via CCF B in CAPIF interconnection. AEF registered to the CCF A can obtain security information from the CCF B via the CCF A to perform authentication and authorization with the API invoker.

Pre-condition:

1.	The API invoker has onboarded to the CCF B.

2.	The API invoker has discovered service APIs provided by an AEF using procedures specified in clause 8.25.3.2 and 8.25.3.3 of 3GPP TS 23.222 [i.2].

3.	The AEF has registered to the CCF A.

4.	The CCF A and the CCF B are connected to each other, and they have business agreement for service API authorization.

Figure 6.X.2-1: Procedure for API invoker authentication and authorization in CAPIF interconnect

1.	CAPIF-1e authentication and secure session is established as specified in subclause 6.3.1 of 3GPP TS 33.122 [4].

2.	The API invoker sends an Access Token Request to the CCF B for obtaining authorization to access the service API published by CCF A via CAPIF-6/6e.

Note: In CAPIF interconnection scenarios, the “scope” in Access token request message is proposed to be REQUIRED.

3.	The AEF service APIs information in “scope” indicate that these APIs are published by the CCF A, so the CCF B requests the CCF A to authorize the service API invocation of the API invoker. The request includes the API invoker ID and the AEF service APIs information. If the CCF A permits the service API invocation, the CCF A responds the CCF B with OAuth 2.0 access token defined in C.2.2 in TS 33.122 [4].

4.	 The CCF B returns Access Token Response message to the API invoker.

5.	To invoke service APIs published by CCF A, the API invoker sends Authentication Initiation Request to the AEF, including API invoker ID and the CCF B ID.

6.	The AEF requests for security information from the CCF A with the API invoker ID and the CCF B ID to perform authentication and secure interface establishment with the API invoker.

7.	The CCF A retrieves security information based on API invoker ID and the CCF B ID. If it has no security information, the CCF A requests for security information from the CCF B with API invoker ID.

8.	Receiving the security information from the CCF B, the CCF A responds the AEF. The security information is AEFPSK (TLS-PSK method) or root certificate of the API invoker (PKI method).

9.	Authentication and secure interface establishment between the AEF and the API invoker is performed with the security information. And the AEF validates the access token following step 6~8 of clause 6.5.2.3 specified in TS 33.122 [4].

### 6.X.3	API invoker offboarding in CAPIF interconnect

When the API invoker is offboarding from the CCF B, then CCF B needs to notify CCF A to delete the security information of the API invoker. If the CCF A and the AEF do not delete the security information, the API invoker offboarded from the CCF B could access the AEF unexpectedly using the obtained security information.

In the following the API invoker offboarding in CAPIF interconnect is detailed.

Pre-condition:

1.	The API invoker previously accessed the AEF service APIs published by CCF A.

2.	The CCF A and the AEF previously received security information of the API invoker from the CCF B.

Figure 6.X.3-1: Procedure for API invoker offboarding in CAPIF interconnect

1.	Security procedure for API invoker offboarding specified in clause 6.8 specified in TS 33.122 [4] is performed.

2.	The CCF B notifies the CCF A with the API invoker ID that this API invoker is no longer valid.

3.	If the security information related to the API invoker has been stored, the CCF A and the AEF registered to the CCF A perform clause 6.8 specified in TS 33.122 [4] to delete the security information.

4.	The CCF A sends an event notification acknowledge to the CCF B to indicate that the security related information associated with this API invoker is successfully deleted.

### 6.X.4	Security method retrieval

#### 6.X.4.1	Introduction

An API invoker can fetch from CCF (via CAPIF-1/1e) information about the security method to access the API service (via CAPIF-2/2e). An API invoker can also request an API Service, in which case the AEF is fetching the information about the API invoker from CCF (via CAPIF-3).

#### 6.X.4.2	General procedure

In interconnect scenarios the API invoker is registered with one service provider's CCF while the AEF offering the API services is registered in another service provider’s CCF, hence the API invoker invokes the API from a different service provider than an AEF known at its local CCF. The API invoker needs to retrieve the supported security method of the exposing API in the other trust domain.

The API invoker therefore includes in the Security Method Request message to its local CCF (CCF-B) (via CAPIF-1/1e) AEF details along with the security capability information to allow the local CCF to find the CCF where the AEF is registered.

Based on the AEF details received from the API invoker, CCF-B identifies the CCF responsible (i.e. CCF-A) where the AEF is registered and forwards the request to this CCF-A.

CCF-A selects a security method for the API service requested from its AEF, considering the information received from CCF-B, access scenarios and AEF capabilities. CCF-A sends a Security Method Response message back to CCF-B, providing the selected security method that allows to get authorization to access the AEF, and any security information related to the security method.

CCF-B sends the response to the API Invoker.

#### 6.X.4.3	Information flow

Figure 6.X.4.3-1: Information flow to retrieve security method in interconnect

Step 1: Mutual authentication based on client and server certificates shall be established using TLS between the API invoker and the CCF-B.

Step 2: The API invoker sends its CAPIF-2/2e security capability information along with AEF details to the CCF-B in the Security Method Request message.  API Invoker security capability information is to indicate the list of security methods it supports and the AEF details from which the security method information retrieval is requested.

Step 3: CCF-B identifies the CCF (CCF-A) associated with AEF from its locally stored data.

Step 4: CCF-B sends the request to CCF-A, optionally appending CCF-B information.

Step 5: CCF-A shall select a security method to be used over CAPIF-2/2e reference point for requested AEF, considering the information received from CCF-B, access scenarios and AEF capabilities.

Step 6: The CCF-A shall send Security Method Response message to the CCF-B (optionally adding CCF-A information), indicating the selected security method for the AEF, any security information related to the security method.

Step 7: The CCF-B sends the Security Method Response to APIInvoker. Additionally, CCF-B also provides an indication to APIInvoker that AEF belongs to a different domain.

### 6.X.5	APIInvoker ownership identification

If CCF-B is connected to multiple CCFs and the AEF associated to CCF-A has no knowledge yet, that the APIInvoker is associated to CCF-B, it first needs to send an APIInvoker ownership query to the CCFs it is collaborating with to find the correct CCF (CCF-B) first.

CCF-A forwards the APIInvokerID to all interconnected CCFs. If one of them possesses information about the APIInvokerID, it responses by confirming that the API invoker is registered with it (i.e. in the figure CCF-B).

CCF-A gets the APIInvoker information including the AEFPSK from CCF-B and authenticates the API invoker towards AEF. CCF-A responses to AEF.

### 6.X.6	Authorization token request handling

To allow for an API invoker belonging to CCF-B to request an access token from CCF-A in a different trust domain, the API invoker includes onboarding secret and possibly also a client credential assertion (CCA) information into its access token request. CCF-B processes the information and forwards to CCF-A. The onboard secret belongs to CCF-B and hence, is not sent to CCF-A.

CCF-A verifies the access token request and CCA, if available. If successfully verified, CCF-A provides back to CCF-B an access token, with the client identifier in the access token claims set to the source API invoker ID as verified before.

Note: For CCA to be used, the assumption is that cross-domain certification is enabled, which allows CCF-A to verify the signature of the requesting API invoker known in CCF-B before creating the access token.

CCF-B provides the access token to the API invoker, which then can establish a TLS connection with AEF and invoke the northbound API with an OAuth 2.0 Access Token.

******* END OF CHANGES