3GPP TSG-SA3 Meeting #120 	S3-250504

# Athens, Greece, 17 - 21 February 2025

# Source:	Apple

# Title:	Conclusion for LTM

# Document for:	Approval

Agenda Item:	4.12

1	Decision/action requested

Approve the pCR to the living CR of LTM security

- References

[1] S3-245164 Living CR for NR mobility enhancement

3	Rationale

This contribution proposes the conclusion for LTM security.

4	Detailed proposal

*** Start of 1st Change ***

5	Conclusions

5.1	Conclusion for key issue #1

For the case where CU is acting as MN and DC is not configured, it is agreed that the normative work is based on the following principles:

-	For the parameters in AS security context, which are not changing per inter-CU LTM handover (including UE’s 5G security capabilities, selected AS security algorithms, UE’s UP security policy, UE’s UP security activation status):

-	If the gNB receives mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) in the Path-Switch Acknowledge message after LTM switch,

-	it initiates an intra-cell handover procedure which includes RRC Connection Reconfiguration procedure indicating the change of selected algorithms (as defined in clause 6.7.3.1) and/or UE’s UP activation status (as defined in clause 6.6.1) to the UE.

-	it updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs.

Editor’s Note: The conclusion for the parameters in AS security context, which are changing per each inter-CU LTM handover (including NCC value, keySetChangeIndicator) is FFS.

Based on RAN2 LS (S3-250325/R2-2411136), the MAC CE based solution should be taken as the basis as the final solution.

-  NCC value is included in the cell switch command MAC CE

-  KNG-RAN* is distributed to one or more candidate gNBs before each inter-CU subsequent LTM execution

*** End of 1st Change ***