3GPP TSG-SA3 Meeting #120 	S3-250505

# Athens, Greece, 17 - 21 February 2025

# Source:	Apple

# Title:	Update to solution#4

# Document for:	Approval

Agenda Item:	4.12

1	Decision/action requested

Approve the pCR to the living CR of LTM security

- References

[1] S3-245164 Living CR for NR mobility enhancement

3	Rationale

This contribution proposes the update for solution#4.

There is an Editor’s Note in solution#4 on AS security context sync up “Editor’s Note: AS security context sync-up after path switch is ffs”.

CU1 configures UE using MAC CE with the NCC value to be used, as well as other candidate CUs. UE has the same NCC to be used for next handover. NH can be derived using the NCC of the previous KgNB.

4	Detailed proposal

*** Start of 1st Change ***

3.4	Solution #4: MAC CE based solution to deliver the NCC(s)

3.4.1	Introduction

This solution proposes the procedure for LTM key derivation. The legacy handover procedure in TS 33.501 clause 6.9.2 will be reused as much as possible. RAN2 procedures on LTM (no DC) are used as the basis.

3.4.2	Details

Pre-assumption:

- In this solution, gNB1, gNB2 and gNB3 are the candidate cells, UE is moving from gNB1 to gNB2, then from gNB2 to gNB3.

- Source gNB1 configures the UE, gNB2 and gNB3 with NCC1 to be used for the key derivation.

Figure 3.4.2-1: MAC CE based LTM procedure

Step 0. Source gNB1 configures the UE, gNB2 and gNB3 with NCC1 to be used for the key derivation.

NOTE 1: Integrity protection on MAC CE is not in the scope of this solution.

Step 1. gNB1 sends the MAC CE to the UE including NCC1(assuming NCC1 is the next NCC to be used for next handover.

Step2. gNB1 derives K_gNB21 based on NCC1 and sends it to gNB2, the key derivation method follows TS 33.501 Annex A.11.

Step3. gNB1 derives K_gNB31 based on NCC1 and sends it to gNB3, the key derivation method follows TS 33.501 Annex A.11.

NOTE 2: the order of step 2/3 and step1 follows RAN2 procedure.

Step4. UE triggers the inter-CU switch, handover from gNB1 to gNB2. UE derives K_gNB21 using NCC1.

Step5. After the handover, gNB2 sends NGAP PATH SWITCH COMPLETE message to AMF.

Step6. AMF sends the NGAP PATH SWITCH ACK message back to gNB2, including a new pair {NH2, NCC2}.

Step7. gNB2 derives K_gNB12 based on NCC2 and sends it to gNB1, the key derivation method follows TS 33.501 Annex A.11.

Step8. gNB2 derives K_gNB32 based on NCC2 and sends it to gNB3, the key derivation method follows TS 33.501 Annex A.11.

Step9. gNB2 sends the MAC CE to the UE including NCC2 to be used for next handover.

NOTE 3: the order of step 7/8 and step9 follows RAN2 procedure.

Step10. UE triggers the inter-CU switch, handover from gNB2 to gNB3. UE derives K_gNB32 using NCC2.

Step11. After the handover, gNB3 sends NGAP PATH SWITCH COMPLETE message to AMF.

Step12. AMF sends the NGAP PATH SWITCH ACK message back to gNB3, including a new pair {NH3, NCC3}.

NOTE 4: UE gets the same NCC with the candidate CUs. NH can be derived using the NCC of the previous KgNB.

3.4.3	Evaluation

This solution leverages MAC CE to deliver the NCC, which is aligned with RAN2 motivation for LTM feature. The NCC configuration can be carried within LTM Cell Switch Command MAC CE (TS 38.321) without any further impact on the system.

MAC CE has no integrity protection so far, this is a security concern not only to this solution#4, but also to other solutions based on MAC CE.

*** End of 1st Change ***