3GPP TSG-SA3 Meeting #120	S3-250726

Athens, Greece, 17 - 21 February 2025

# Source:	BSI, Montsecure

# Title:	Motivation for RES* verification SCAS test for AUSF in roaming scenarios

# Document for:	Information

Agenda Item:	4.1.1

1	Decision/action requested

This section discusses and explains the SCAS test cases that verify the proper functionality in case of roaming

2	References

[1]			3GPP TS 33.501: “Security architecture and procedures for 5G system”

[2]			3GPP TS 33.512: “Access and Mobility management function”

3	Rationale

Although 5G SA roaming is not yet widely deployed, ensuring its security is crucial to fulfill the 5G promise of secure roaming. This discussion will not focus on the security of the PRINS/TLS protocol and deployment scenarios. Instead, it will motivate the implementation security of the network function that responds to incoming roaming requests.

The inbound security of these network functions is essential for maintaining security requirements during roaming scenarios. While malfunctions in non-roaming scenarios have limited implications, they can have severe consequences in roaming scenarios.

A good example is the AUSF while performing the AKA procedure (see [1], clause 6.1.3.2.0 and figure below) in a roaming scenario. The AUSF is responsible for ensuring increased home control (see [1], clause 6.1.4). The SUPI is only provided to the SEAF when there is cryptographic proof that the UE is actually connected to the visiting network. This is why the SEAF (visiting network) sends the RES* to the AUSF (home network) in step 10 in the figure below. The AUSF verifies this in step 11, and upon successful verification, responds with the SUPI in step 12.

This means the AUSF must verify the RES* correctly—simply responding with the SUPI without proper verification undermines the increased home control. Furthermore, if the SUPI leaks through other means during this procedure, it compromises the confidentiality of the SUCI.

Such a malfunction might not have major implications in a non-roaming scenario, since all network functions are under the control of the same owner. However, in a roaming scenario, the SEAF has a different owner, so we must assume that (a) the SEAF might not conform to specifications and (b) the SEAF's intentions could be malicious. This can lead to attack scenarios where the attacker can harm users. In the case of the AUSF, the identity could be leaked without the UE actually being present in the network.

For the non-roaming case there is an existing AMF/SEAF SCAS test case (see [2], clause 4.2.2.1.2) that ensures RES* verification and error handling at the AMF/SEAF, however since in a roaming scenario this AMF/SEAF cannot be trusted, the responsibility for verifying and error handling RES* is up to the home network’s AUSF.

Note that the SEPP only translates requests to the corresponding Network Function—it does not verify the correctness of application layer functions.

Figure 1: [1], clause 6.1.3.2

4	Detailed proposal

These considerations serve as motivation for understanding threats and SCAS test cases that arise from roaming deployment.

Related new Threat Reference: S3-250727

Related new AUSG SCAS test case: S3-250728