3GPP TSG-SA3 Meeting #120	S3-250746

# Athens, Greece, 17 - 21 February 2025

Source:	Ericsson

Title:	Conclusion update for KI#2: Security of IMS based Avatar Communication

Document for:	Approval

Agenda item:	5.2

Spec:	3GPP TR 33.790

Version:	1.1.0

Work Item:	FS_NG_RTC_SEC_Ph2

Comments

The conclusions for KI#2: Security of IMS based Avatar Communication are incomplete with several Editor's Notes. This document proposes a way forward for the authorization aspects of the KI#2

Moreover a reference to TS 33.328 is added. The reference was included in S3-250192 but it was missed to be implemented.

* * * First Change * * * *

## 7.2		Conclusions for Key Issue #2: Security of IMS based Avatar Communication

The following principles are concluded for normative work:

1.	Any security normative work for this key issue will take the TS 23.228, Annex AC.11 into account.

2.	The Avatar ID sent by the UE1 should be verified by the DC AS (for the sending UE and receiving UE centric rendering) and the BAR for the network centric rendering about whether it belongs to the UE1. According to TS 23.228, AC.11.2 "The BAR stores Avatar Representation, its Avatar ID, and the associated information as described in clause AC.11.1. One or more Avatar Representation(s), is stored for a user, and each Avatar Representation is identified with Avatar ID". Therefore the BAR includes information to check whether an Avatar ID belongs to a specific UE.

3. 	The e2DCe protection mechanism specified in TS 33.328 [XY] is reused for authentication, integrity and confidentiality protection between UE and MF, UE and DC AS, and UE to DCSF for Avatar ID List downloading, Avatar ID transfer and avatar representation downloading via BDC/ADC. As a result there is no need for new normative work for authentication, integrity and confidentiality protection when bootstrap and application data channels are used between two communicating entities

4.	Reuse existing security specification in TS 33.328 [XY], Annex P.1, for security of aspects of SBA in IMS media control interface, to protect SBA communication between MF/DCSF and DC AS/BAR, or between DC AS and BAR, if the DC AS and BAR are deployed inside of operator’s domain.

Editor's Note: Whether there is an SBA interface between the DC-AS and BAR is in the remit of SA2.

5.	Reuse existing security specification in TS 33.501 [11], clause 12 for Protection of the NEF – AF interface, to protect communication between DCSF and DC AS if the DC AS is deployed outside of operator’s domain.

6.	The UE1 choice of the rendering mode (sending UE centric, receiving UE centric, network centric) provides implicit permission to UE1, UE2 and the MF respectively to retrieve the avatar representations for avatar rendering. The DC AS (for sending and receiving UE centric rendering) or the MF (for the network centric rendering) act on behalf of the UE1 and request the UE1 avatar representation from the BAR by presenting the UE1 identifier and UE1 chosen Avatar ID. Then the DC AS provides the avatar representation to the UE1 for the case of sending UE centric rendering or to the UE2 for the case of receiving UE centric rendering. The MF retrieves the avatar representation for the case of network centric rendering. These can be done because both DC AS and MF have authenticated the UE1. More specifically the behaviour is the following for the different rendering cases:

6.1)	For sending UE centric rendering, the DC AS requests the avatar representation of UE1 from the BAR by presenting the UE1 identity and Avatar ID chosen by the UE1.

6.2)	For receiving UE centric rendering, the UE1 sends the chosen Avatar ID to UE2 using the P2A2P data channel which is secure. The UE2 requests the avatar representation from the BAR via the DC AS by presenting the Avatar ID provided by UE1. The DC AS checks if the UE2 has provided the right Avatar ID chosen by the UE1 and if the UE1 has allowed the UE2 to retrieve the avatar representation. If these checks are passed, the DC AS retrieves the avatar representation from the BAR on behalf of UE1 by presenting the UE1 identifier and Avatar ID chosen by the UE1 and sends the avatar representation to UE2.

6.3)	For network centric rendering, the MF acts on behalf of UE1 to request the avatar representation from the BAR by providing the UE1 identity and the Avatar ID chosen by the UE1.

7)	At the end of the IMS avatar session, depending on the rending option chosen by UE1, the UE2 and MF delete the Avatar ID and the avatar representation if these pieces of information don't belong to them.

Editor's Note: It is in the remit of SA2 to specify the interfaces between DCSF and DC-AS for the IMS avatar communication and whether NEF is used for control plane messages between the DC-AS and DCSF.

Editor's Note: Protection of communication between MF and DC AS/BAR if the DC AS/BAR is deployed outside of operator’s domain is FFS.

Editor's Note: Protection of communication between DC AS and BAR if one of them is deployed outside of operator’s domain and another is deployed inside of operator’s domain is FFS.

Editor's Note: It is in the remit of SA2 to decide the different combination of cases for the deployment of the DC-AS and BAR with respect to inside/outside the 3GPP operator domain.

NOTE: if both DC AS and BAR are deployed outside of operator’s domain, the security mechanism is out of scope.

Editor's Note: Whether the integrity and privacy of the avatar representation during the transmission should be protected is FFS.

Editor's Note: Further conclusion is FFS.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-77: ""Study on system architecture for next generation real time communication services; Phase 2".

[3]	3GPP TR 33.890: "Study on security support for next generation real time communication services".

[4]	3GPP TR 24.229: " IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP);Stage 3".

[5]	ATIS-1000074: "Signature-based Handling of Asserted information using Tokens (SHAKEN)|

[6]	IETF draft-ietf-stir-passport-rcd-26: "PASSporT Extension for Rich Call Data"

Editor's Note:	The above document cannot be formally referenced until it is published as an RFC.

[7]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2"

[8]	IETF draft-ietf-sipcore-callinfo-rcd-09: "SIP Call-Info Parameters for Rich Call Data".

Editor's Note:	The above document cannot be formally referenced until it is published as an RFC.

[9]	draft-ietf-sipcore-callinfo-rcd-03: "SIP Call-Info Parameters for Rich Call Data".

[10]	IETF RFC 8224: "Authenticated Identity Management in the Session Initiation Protocol (SIP)".

[11]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[12]	IETF RFC 7515: "JSON Web Signature (JWS)"

[13]	3GPP TS 33.122: " Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs "

[14]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System architecture for the 5G System (5GS); Stage 2"

[15]	3GPP TS 33.203: "3G security; Access security for IP-based services"

[XY]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security"

* * * End of Changes * * * *