3GPP TSG-SA3 Meeting #120	S3-250796

# Athens, Greece, 17 - 21 February 2025

Source:	Samsung

Title:	Resolving EN on solution #38

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_Ambient_IoT_Sec

Comments

Depending on use cases, one-way authentication may be sufficient. This aspect will be decided in conclusion part. So the following Editor's Note can be removed.

Editor's Note: Whether one-way authentication is sufficient is FFS.

* * * First Change * * * *

## 6.38	Solution #38: Authentication and privacy of AIoT device

### 6.38.1	Introduction

This solution addresses key issue #3: Privacy by protecting AIoT device identifiers and key issue #5: Authentication in Ambient IoT service.

It is assumed that an AIoT device has power or computational resource limitation.

In this solution, encryption is used to protect AIoT device identifier (AIoT ID) and one-way authentication (i.e., network authenticating AIoT device) is performed.

NOTE: Whether one-way authentication is sufficient will be decided in the conclusion part.

It is assumed that AIoT device and AF are provisioned with AIoT ID and a key (K).

Editor’s Note: Alignment with conclusion from TR 23.700-13 [4] is FFS.

### 6.38.2	Solution details

Figure 6.38.2-1 Security procedure for AIoT

0. AIoT ID and a key (K) are provisioned to the AIoT device and AF. AF generates KAIoT from K and RANDAF.

1. AF sends AIoT service request message to AIoTF (AIoT Function). This message may be sent via NEF. AIoT service request message includes command, AIoT ID, RANDAF, and KAIoT.

2. AIoTF generates RANDAIoTF. AIoTF derives encryption key (Kenc) and integrity key (Kint), and then encrypts AIoT ID using RANDAIoTF and Kenc.3. AIoTF sends AIoT Service Request including RANDAIoTF, RANDAF, encrypted AIoT ID, and command.

NOTE: If necessary, command may be encrypted using Kenc.

NOTE: An attacker that captures the message in step 3 can succeed on replay attack, so resource exhaustion attack on AIoT devices is possible.

Editor’s Note: How AIoTF knows which AIoT device to send the request is FFS.

4. The AIoT device derives KAIoT from K and RANDAF. The AIoT device derives Kenc and Kint from KAIoT, before decrypting the encrypted AIoT ID. If the decrypted ID is same as the AIoT ID, the AIoT device processes the command.

5. AIoT device responds with AIoT service response. The message includes encrypted AIoT ID, command response, and MAC. MAC is generated from Kint, RANDAIoTF, encrypted ID, and command response.

NOTE: If necessary, command response may be encrypted using Kenc.

6. After AIoTF finds Kenc and Kint from the encrypted AIoT ID, AIoTF checks the authenticity of the message by verifying the MAC. If the verification is successful, AIoTF decrypts the encrypted IE(s) and proceeds to step 7. If the verification fails, AIoTF does not allow the AIoT device to use the network provided by the operator.

7. AIoTF sends AIoT service response to AF. This message includes AIoT ID and command response.

For A-IoT device paging functionality, it is understood that the legacy paging message, legacy paging occasion and legacy DRX from NR are not supported (See TS 38.300 [R2-3] for references for any legacy NR functionality). From RAN2 perspective, it is assumed that the A-IoT device can receive as long as there is enough energy

* * * End of Changes * * * *