3GPP TSG-SA3 Meeting #120	S3-250815

# Athens, Greece, 17 - 21 February 2025

Source:	Ericsson, Thales

Title:	New key issue for secure storage in AIoT devices

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_Ambient_IoT_Sec

Comments

AIoT devices are required to authenticate themselves to the 5G network. Therefore, the credentials used by the devices to authenticate themselves need to be protected from adversaries.

* * * First Change * * * *

## 5.X	Key issue #X: Secure storage in AIoT devices

### 5.X.1	Key issue details

The following is a potential security requirement described in KI#5: "The 5G system shall provide a means to perform mutual authentication between the AIoT device and the network". Designing an authentication solution according to the requirement in KI#5 would require an AIoT device to store a shared secret. The secrecy of the shared secret is paramount to the security of AIoT devices.

If an adversary learns the secret in an AIoT device, then they can completely compromise the security of the device — all confidentiality and integrity protections for the device break down. The secret enables an adversary, with suitable malicious interest, to produce a clone of the device.

A cloned device can impersonate the victim device to the network, which enables an adversary to steal the victim device by replacing it with the cloned device. Since AIoT readers, even for the indoor scenarios, have tens of meters distance from the devices, and not operated by any human agent, the theft will go unnoticed until it is too late. This can cause a great loss for the user of the victim device. Cloning the device also enables unauthorized usage of the connectivity service provided by the MNO.

### 5.X.2	Security threats

-	An adversary can break the confidentiality and integrity of all the communication between an AIoT device and the network

-	Cloning the device also enables unauthorized usage of the connectivity service provided by the MNO.

-	An adversary can steal an AIoT device by replacing the device with a cloned device, and the theft will go unnoticed until it is too late. This can cause a great loss for the user of the device.

### 5.X.3	Potential security requirements

The following requirements apply for the storage and usage of the credentials used for authentication of the AIoT device:

-	The credential(s) shall be integrity protected within the AIoT device using a tamper resistant secure hardware component.

-	The long-term shared secret key shall be confidentiality protected within the UE using a tamper resistant secure hardware component.

-	The long-term key(s) of the subscription credential(s) shall never be available in the clear outside of the tamper resistant secure hardware component.

-	The authentication algorithm(s) that make use of the authentication credentials shall always be executed within the tamper resistant secure hardware component.

NOTE:	The security assessment scheme used for the security evaluation of the tamper resistant secure hardware component is outside the scope of 3GPP specifications.

* * * End of Changes * * * *