3GPP TSG-SA3 Meeting #120	S3-250840

# Athens, Greece, 17 - 21 February 2025

Source:	Xiaomi Technology Spain S.L

Title:	Protection mechanism for PC5 discovery messages with ProSe Message Content Type extensions

Document for:	Approval

Agenda item:	4.15

Spec:	3GPP TS 33.503

Version:	18.5.0

Work Item:	5G_ProSe_Ph3_Sec

Comments

CT1 has concluded to introduce a new IE representing the ProSe direct discovery PC5 message content type extensions in the PC5 discovery messages, as mentioned in C1-247001. With this new IE, the existing security method needs to be accommodated. Therefore, this pCR propose to define the security protection mechanism for multi-hop UE-to-network relay discovery.

* * * First Change * * * *

6.1.3.X.Y	Protection mechanism for 5G ProSe Multi-hop UE-to-Network Relay Discovery

For the discovery messages that do not include HPLMN ID, the protection mechanisms specified in clause 6.1.3.2.3 are reused with the following changes:

-	Message-specific confidentiality mechanisms as specified in clause A.7 in the present document with the following changes:

-	The input parameter LENGTH is set to LEN(discovery message) - (LEN(Message Type) + LEN(Message content type extensions) + LEN(UTC-based counter LSB) + LEN(MIC)), where LEN(x) is the length of x in number of bits.

-	The KEYSTREAM is XORed with the discovery message for message-specific confidentiality protection excluding Message Type, Message content type extensions, UTC-based counter LSB and MIC.

-	In A.5 of TS 33.303 [4], the time-hash-bitsequence keystream is set to L least significant bits of the output of the KDF, where L is the bit length of the discovery message to be scrambled and set to Min (the length of discovery message – 24, 256).

-	Step 3 of clause 6.1.3.4.3.5 of TS 33.303 [4] becomes:

XOR (0xFFFFFF || time-hash-bitsequence) with the most significant (L + 24) bits of discovery message.

NOTE 1:	24 is the size of Message Type, Message content type extensions and UTC-based counter LSB in bit length.

The discovery messages that include HPLMN ID are protected using the protection mechanism described above with the following changes:

-	Message-specific confidentiality mechanisms as specified in clause A.7 in the present document with the following changes:

-	The input parameter LENGTH is set to LEN(discovery message) - (LEN(Message Type) + LEN(Message content type extensions) + LEN(UTC-based counter LSB) + LEN(HPLMN ID) + LEN(MIC)), where LEN(x) is the length of x in number of bits.

-	The KEYSTREAM is XORed with the discovery message for message-specific confidentiality protection excluding Message Type, Message content type extensions, UTC-based counter LSB, HPLMN ID and MIC.

-	In A.5 of TS 33.303 [4], the time-hash-bitsequence keystream is set to L least significant bits of the output of the KDF, where L is the bit length of the discovery message to be scrambled and set to Min (the length of discovery message – 24 – the length of HPLMN ID, 256).

-	Step 3 of clause 6.1.3.4.3.5 of TS 33.303 [4] becomes:

XOR (0xFF..FF || time-hash-bitsequence) with the most significant (L + 24 + the length of HPLMN ID) bits of discovery message, where 0xFF..FF is (24 + the length of HPLMN ID) bits of length.

* * * End of Changes * * * *