3GPP TSG-SA3 Meeting #120	S3-250843

Athens, Greece, 17 - 21 February 2025

Title:	[draft] Reply LS on UE behaviour in case of SUCI calculation failure

Response to:	Reply LS S3-250321/C6-240721 on UE behaviour in case of SUCI calculation failure from CT6

Release:	Rel-19

Work Item:	5GProtoc19

Source:	Xiaomi [to be SA3]

To:	CT1, CT6

Cc:

Contact person:	Wei Lu

luwei10(at)xiaomi(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

SA3 thanks CT6 for the reply LS (C6-240721) on UE behaviour in case of SUCI calculation failure.

SA3 rediscussed the error case described by CT1 in the LS C1-245039 and analysed the possible UE behaviours based on the response in the LS C6-240721. SA3 concluded on the following:

- Using null-scheme is not recommended, because:

  - The information required for SUCI calculation includes SUPI type, HN ID and RID among others. Misconfiguration of any of the SUPI type or HN ID or RID would fail the SUCI calculation with null-scheme. In this case, SUCI calculation not only fails in the USIM but also fails in the ME even using null-scheme.

  - SUCI calculation using null-scheme that is not made aware by the subscriber may violate the privacy of the subscriber.

  - The SUCI calculation indication stored in the USIM is based on home operator's decision on calculation in either USIM or ME. The error status word from the USIM implies to the ME that the SUCI calculation was already performed by the USIM though failed. SUCI calculation again in the ME using null-scheme is not compliant with the original decision made by the home operator.

- Fallback to 4G is not recommended, because:

  - The subscriber would lose all privacy gains provided by 5G network, which is even worse than using null-scheme.

  - Configuration error remains without being detected by home operator.

- The main purpose of using null-scheme in 5G or fallback to 4G is for preserving user experience with available 5G or 4G services. However, as such configuration error is considered exceptional and rare, preserving user experience while violating user privacy in such a rare case may not be necessary.

- Recommendations in bullets 1 and 2 can avoid exposing attack surface for, e.g. man-in-the-middle attack or bidding-down attack.

- As the probability of such misconfigurations is as low as the risk that the information required for network authentication (e.g. K and OpC) is not correctly configured to the USIM, it would be recommended that UE behaviours for all these cases are treated in the similar ways, e.g. considering the USIM invalid.

2	Actions

To CT1 and CT6

ACTION: 	SA3 kindly asks CT1 to take the above response into account.

3	Dates of next TSG SA WG 3 meetings

SA3#121	7 – 11 April 2025		Goteborg, Sweden

SA3#122	19 - 23 May 2025		Fukuoka, Japan