3GPP TSG-SA3 Meeting #120	S3-250881

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Adding principle on ID verification to draftCR of MEC ph3

Document for:	Approval

Agenda item:	4.17

Spec:	3GPP TS 33.558

Version:	18.2.0

Work Item:	EDGE_Ph3

Comments

This contribution proposes to add the priciple on ID verification concluded in the clause 7.2 of TR 33.749

* * * First Change * * * *

5.1.X Secure retrieval of 5G system UE Ids  requirements

5GC should support a mechanism to verify the UE Id provided by EEC, and verify that the ECS/EES/EAS functions are authorized to retrieve the 5G system UE Id.

The procedures of calling UE identifier API and Nnef_UEId API should not compromise the privacy related information.

* * * Second Change * * * *

6.Y	Secure retrieval of 5G system UE Ids and privacy related information

The following design principles are proposed for the UE Id verification and privacy protection:

- The information used in Nnef_UEId API call invocation and UE Identifier API call invocation needs to prevent the use of permanent or semi-permanent UE identifiers subject to be abused/compromised by malicious applications.

- The entity responsible for the generation of the information needs to be part of the 5G system in HPLMN, and needs to be verified by the 5G system in HPLMN. Alternatively, the information can be generated by both the UE and the 5G system.

- The information may be used by the AF invoking the NEF APIs, so that the NEF APIs can identify the UE by using the information.

NOTE 1: The API invoker does not need to obtain the permanent 5G UE ID, which improves the privacy.

Alternative mechanisms having the design principles above are captured in an informative Annex X of this document.