3GPP TSG-SA3 Meeting #120	S3-251099

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0199 | rev | 1 | Current version: | 19.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Note to TC_BVT_PORT_SCANNING on external interfaces |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current test requires port scans to be performed on all IP-based interfaces. In modern NF deployments e.g. In kubernetes cluster, the network product could be composed of multiple containers with several inwards pointing IP-based interfaces for internal communication, but only externally accessible interfaces should be tests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added note to pre-conditions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the tester performs the test literally for a e.g. Containerized network product, they need to perform several unnecessary port scans on internal IP-based interfaces. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

### 4.4.2	Port scanning

Requirement Name: Port scanning

Requirement Reference: In accordance with industry best practice

Requirement Description:

It shall be ensured that on all network interfaces, only documented ports on the transport layer respond to requests from outside the system.

The test for this requirement can be carried out using a suitable tool or manually performed as described below. If a tool is used then the tester needs to provide evidence, e.g. by referring to the documentation of the tool, that the tool actually provides functionality equivalent to the steps described below.

Threat References: TR 33.926 [4], clause 5.3.6.10, Insecure Network Services

Test Case:

Test Name: TC_BVT_PORT_SCANNING

Purpose:

To ensure that on all network interfaces, only documented ports on the transport layer respond to requests from outside the system

Procedure and execution steps:

Pre-Conditions:

A list of all available network services containing at least the following information shall be included in the documentation accompanying the Network Product:

1.	all interfaces providing IP-based protocols;

2.	the available transport layer protocols on these interfaces;

3.	their open ports and associated services per transport layer protocol;

4.	and a free-form description of their purposes.

The port scanning tool that is used shall be configured to scan all the transport layer ports and detect open ports for each network interface as identified in steps 1 to 4. above.

NOTE1: 	It might not be possible for certain transport layer protocols (like UDP) to unambiguously detect whether a port is open or not by means of external port scanning. Also, in some circumstances it might not be efficient to do external port scanning, e.g. if there are security measures to limit the rate a system can be probed. In those cases, the tester determines another means suitable to verify which ports are open.

NOTE2:	An interface providing IP-based protocols in the context of this test case is any interface, which the network product vendor implements with the intent to be accessible by any other network function or entity.

Execution Steps

The tester is required to execute the following steps:

1.	Verification of the compliance to the prerequisites:

a.	Verification that the list of available network services is available in the documentation of the Network Product

b.	Validation that all entries in the list in Pre-Conditions are necessary and it is justified in the documentation of the Network Product for the operation of the Network Product class

2.	Execution of the port scanning tool against all the interfaces providing IP-based protocols to identify open ports

3.	Verification that the list of open ports identified by the tool, matches the list of available network services in the documentation of the Network Product

Expected Results:

The used tool(s) name, their unambiguous version (also for plug-ins if applicable), used settings, and the relevant output containing all the technically relevant information about test results is evidence and shall be part of the testing documentation.

All discrepancies between the list of identified open ports and the list of available network services in the documentation shall be highlighted in the testing documentation.

Expected format of evidence:

Output of port scan and list of identified discrepancies.

********** END OF 1st CHANGE **********