3GPP TSG-SA3 Meeting #122																			S3-252240

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.700-22 | CR | 0003 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing clarifications and typos correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_CAPIF_Ph3-sec |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Missing clarifications, typos, formatting issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | Typo corrections, text alignments, formatting (6.18.3) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Typos in TR, misunderstandings |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2, 6.4.3, 6.6.3, 6.7.3, 6.12.2.1, 6.17.1, 6.18.3, 6.20.1, 6.25.1, 6.29.2.1, 6.32.1, 6.35.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********* START OF CHANGES

#### 6.4.2.2	Revocation procedure

The procedure for revoking API invoker authorization initiated by the resource owner through the resource owner function (ROF) is given below extended from the procedure specified in clause 6.5.3.4 of TS 33.122 [4]:

1. The resource owner may trigger token revocation through the ROF. The ROF sends an authorization revocation request message to the CCF. The message shall include the service API ID, the GPSI, and other information related to the revoked token (e.g., the scope info).

NOTE: 	The GPSI is the identifier of the resource owner. It is stored at the ROF through configuration or the authorization procedure.

2.	With reference to step 2 in clause 8.23.4 of TS 23.222 [2], the request message includes in addition the GPSI of the UE, on which the ROF resides.

3. With reference to step 3 in clause 8.23.4 of TS 23.222 [2], the AEF may additionally determine whether to update the resource due to revocation, e.g., the API invoker is using the resource (i.e. QoS) that should be revoked after token revocation for the QoS service API, the AEF may inform PCF/SMF to modify the QoS level of corresponding PDU sessions after revocation.

4. The same as the step 4 in clause 8.23.4 of TS 23.222 [2].

5. Similar to the step 5 in clause 8.23.4 of TS 23.222 [2], the difference is invalidated authorization here is API invoker authorization for the resource owner/UE corresponding to the GPSI.

6. Similar to the step 6 in clause 8.23.4 of TS 23.222 [2], the difference is that the message is sent to the ROF.

Figure 6.4.2-1: Revocation procedure for initiated through ROF

************ NEXT CHANGE

### 6.4.3	Evaluation

This solution addresses the requirements of Key Issue #1.2.

This solution has two separate procedures for authorization and revocation.

This solution reuses the authorization procedure specified in clause 6.5.3 in the TS 33.122 [4] and extends the revocation procedure in clause 6.5.3.4 in TS 33.122 [4] to include steps related to CAPIF-8 interface. Therefore, no impact is introduced by the authorization procedure. As to the revocation procedure, the ROF will send a revocation request message to the CCF and receives the corresponding revocation response message. In addition, AEF may determine whether to update the resource due to revocation, if yes, the AEF may send a message to the PCF/SMF to modify the QoS level. The impact to the AEF in this aspect is that the AEF needs to store information of original resources in order to update related operations.

The revocation procedure is to prevent token being misused by an attacker. Although a short-lived token can potentially mitigate the issue, it has restrictions for its applicability or use cases. For example, setting token lifetime may be challenging in many scenarios. On the one hand, a large number of tokens or frequent token refreshing may be needed if the token lifetime is short, which constrains the system with complexity and overhead. On the other hand, if the token lifetime is short, the token may be misused or compromised.

A short-lived token can be an alternative solution. As comparison with the alternative solution, this solution can provide revocation immediately and avoid frequent refreshing of token.

************ NEXT CHANGE

### 6.6.3	Evaluation

This solution only addresses the 2nd and 3rd requirement of Key issue #1.2 and provide the security procedures for resource owner authorization management.

This solution proposes that the CCF and the ROF establish a secure session over CAPIF-8 reference point. The resource owner provides the resource owner authorization information to the CCF via ROF. The CCF stores the resource owner authorization information along with the resource owner ID.

This solution proposes that the CCF check whether the API invoker is entitled to consume the API and allowed to access the resources of the resource owner, by using the resource owner authorization information.

This solution propose that the resource owner sends resource owner authorization revocation request to the CCF via ROF to trigger the revocation procedure as specified in clause 6.5.3.4 of TS 33.122 [4]. The resource owner authorization revocation information in request message includes the Resource Owner ID, service API information, API invoker ID.

************ NEXT CHANGE

### 6.7.3	Evaluation

This solution covers a management aspect necessary for the RO authorization/permission process, i.e., how to ensure reachability of the ROF for authentication and authorization. In particular, how can the resource owner subscribe for the notifications.

In ROF and CCF new APIs need to be supported for subscription notifications.

For NAS based notifications, AMF and UE need to support new NAS IEs to support wakeup.

The NAS based solution has access network impact.

The solution needs to keep ROF (e.g., the web browser on UE) online to provide authorization information to CCF.

************ NEXT CHANGE

#### 6.12.2.1	Summary

The API invoker wants to consume APIs offered at an AEF outside of its trust domain. In its Security Method Request message to its local CCF (CCF-B) (via CAPIF-1/1e) it needs to include AEF details along with the security capability information to allow the local CCF to find the CCF where the AEF is registered.

NOTE: With the current CAPIF framework the API invoker cannot get details from its own CCF (CCF-B) about the APIs offered at the requested AEF in the other security domain.

Based on the AEF details received from the API invoker, CCF-B identifies the CCF responsible (i.e. CCF-A) where the AEF is registered and forwards the request to this CCF-A.

CCF-A selects a security method for the API service requested from its AEF, considering the information received from CCF-B, access scenarios and AEF capabilities. CCF-A sends a Security Method Response message back to CCF-B, providing the selected security method that allows to get authorization to access the AEF, and any security information related to the security method.

CCF-B sends the response to the API Invoker.

************ NEXT CHANGE

### 6.17.1	Introduction

This solution addresses the requirements identified in key issue#2 "CAPIF interconnection security".

It is proposed to reuse clauses 6.6 and 6.10 of TS 33.122 [4] for securing CAPIF-6 and CAPIF-6e reference points respectively.

The solution 6.17.2.1 enhances the Authentication and Authorization using Method 3 specified in subclause 6.5.2.3 of TS 33.122 [4] for CAPIF interconnection scenario. It enables the API invoker onboarded to the CCF B to obtain the service API invocation authorization from the CCF A via CCF B in CAPIF interconnection. And the AEF registered to the CCF A can obtain security information from the CCF B via the CCF A to perform authentication and authorization with the API invoker. The enhancements are bolded.

The solution 6.17.2.2 addresses offboarding from CCF-B. If the API invoker previously accessed the AEF service APIs published by CCF A, when the API invoker is offboarding from the CCF B, the CCF B notifies the CCF A to delete the security information of the API invoker. If the CCF A and the AEF do not delete the security information, the API invoker offboarded from the CCF B may access the AEF unexpectedly using the obtained security information.

************ NEXT CHANGE

### 6.18.3	Evaluation

This solution addresses the following part of KI#2: the CAPIF should support mutual authentication between API invoker and AEF when AEF service APIs are published via CAPIF-6/6e reference point in CAPIF interconnection scenarios).

The benefit of this solution is that it realizes API invoker authentication via enhancing existing mechanisms.

Impacts to API  invoker:

The API invoker is required to send CCF-A ID to the AEF.

The API invoker is required to send CCF-A ID to the AEF.

Note: 	How API invoker can determine to send the CCF-A ID is not addressed.

Impacts to AEF:

AEF is required to send AEF ID, API invoker ID and CCF-A ID to the CCF-B.

The AEF ID and API invoker ID is used by the CCF-A to retrieve security information related to the specific AEF and API invoker. The CCF-A ID is used by the CCF-B to which the AEF registers to identify the CCF A and send the request to the CCF-A.

AEF is required to send AEF ID, API invoker ID and CCF-A ID to the CCF-B.

The AEF ID and API invoker ID is used by the CCF-A to retrieve security information related to the specific AEF and API invoker. The CCF-A ID is used by the CCF-B to which the AEF registers to identify the CCF A and send the request to the CCF-A.

Impacts to CCF-B:

CCF-B is required to request AEFPSK/root CA certificate from the CCF-A.

CCF-B is required to request AEFPSK/root CA certificate from the CCF-A.

Impacts to CCF-A:

CCF-A is required to send AEF the AEFPSK/root CA certificate via CCF-B.

CCF-A is required to send AEF the AEFPSK/root CA certificate via CCF-B.

************ NEXT CHANGE

### 6.20.1	Introduction

This solution addresses the following part of KI#2: The API invoker should support retrieval of the security method needed for accessing service APIs when these AEF service APIs are published via CAPIF-6/6e reference point in CAPIF interconnection scenarios.

The existing secure method negotiation procedure defined in clause 6.3.1.2 of TS 33.122 [4] is enhanced to support the CAPIF inter-connection scenarios.

************ NEXT CHANGE

### 6.25.1	Introduction

This solution addresses the requirements identified in key Issue #3 (Authorizing API invoker on one UE accessing resources related to another UE).

In the real word application deployments, it is very common practice to have a backend server in addition to the application clients running/installed on end point devices such as mobile phones. For example, when the use case of gaming application is considered, even if the game application client instances are running on different devices and communicating with each other directly, there is always a backend server serving these clients. Thus, utilization of backend server can bring some benefits in terms of handling access permission and user identifiers. Taking these benefits into account, this solution proposes that when an application client instance running on a UE wants to access resources related to another UE which hosts another application client instance, the requester client can send the request to the backend server and then the backend server can consume the related APIs and returns the result back to the requester client.

From 3GPP point of view, only permission check whether the backend server as AF is allowed to access the resources of the UE is needed. The remaining permission check such as which users' resource can be accessed by which users can be handled in the application layer out of 3GPP scope.

************ NEXT CHANGE

#### 6.29.2.1	Summary

The solution proposes a way to enhance current OAuth2 based authorization mechanisms, both at CCF and AEF, to allow a finer granularity access control.

NOTE 1: 	A finer granularity can include: Resource Owner ID, operations (e.g. retrieve, create, etc.), features (e.g. feature 1, feature 2, etc.) and resources (e.g. resource 1, resource 2, etc.).

After authentication between the CCF and the API Invoker, the latter will include the required additional information to CCF during the Access token Request. The API Invoker will include in the scope parameter more authorization details that allow to distinguish both the resource owner and the resources that API Invoker intends to access. This will allow the CCF to verify if the resource owner allowed this specific API Invoker to access the requested resources.

When the verification is completed, the CCF will include the authorization details, with the new granularity, into the access token returned to the API Invoker.

The previously provided access token will allow the AEF to correctly authorize, or deny, the request by the mechanism already available to AEF. AEF need to be able to verify the details for finer granularity access control.

NOTE 2: The inclusion of resource owner identifier is proposed to be a scope parameter while in Rel-18 RO id is part of claim. Granularity is not specified in Rel-18. The solution proposes that a Rel-19 RNAA token is including the RO id and granularity details in scope and leaving empty the Rel-18 resOwenrId. This allows to distinguish between Rel-19 and Rel-18. It is done by purpose to distinguish between RO id in claims in Rel-18 and to avoid backward compatibility problems.

************ NEXT CHANGE

### 6.32.1	Introduction

This solution addresses KI#6 of the present document and relates to KI#5 of TR 23.700-22 [3].

GPSI provided by the APIinvoker in an onboarding or modification request needs to be confirmed to be associated to the UE on which the APIinvoker is running. The solution enables the CCF to validate apiInvokerInformation details by UE interaction.

********* NEXT CHANGE

### 6.35.3	Solution evaluation

This solution addresses the requirement on how UE hosted APIs can be onboarded. It proposes the following changes to the existing onboarding procedure.

-	The onboarding credential access token can include the Application ID.

-	The onboarding request can include the token issuer's certificate.

-	The CCF needs to store the Application ID in the API invoker profile.

-	The CCF can authenticate the UE hosting the API invoker and can store the UE ID in the API invoker profile.

Impacts on the API invoker:

-	Since obtaining of the enrolment information is out of scope, there is no impact on the API invoker whether it obtains the enrolment information from API provider domain or its own backend server.

-	There is impact on the API invoker for sending the token issuer's certificate to the CCF.

-	There can be impact on the API invoker for authentication of the UE towards the CCF.

Impacts on the CCF:

-	There is impact on the CCF to store the Application ID in the API invoker profile and use this information in the authorization decision.

-	There is impact on the CCF to authenticate the UE hosting the API invoker.

There is no impact on the AEF.

There can be business relation between the CCF and API provider domain as in the existing system, so the certificate or necessary info of API provider domain to perform token validation can be same as existing system. So, why an access token is not sufficient and why additionally token issuer's certificate and API invoker's public key need to be sent in Onboard API invoker Request is not addressed.

************ END OF CHANGES