3GPP TSG-SA3 Meeting #123	S3-252651

# Goteborg, Sweden, 25 – 28 August 2025

Source:	China Telecom, CAICT, ZTE

Title:	Pseudo-CR on topology hiding in N9 interface

Document for:	Approval

Agenda item:	6.1.5

Spec:	3GPP <TS/TR><TS/TR number>

Version:	<TS/TR version>

Work Item:	<Work Item>

Comments

This contribution proposes KI on topology hiding in N9 interface for SID on security for PLMN hosting a NPN phase 2.

Proposed Changes

* * * First Change * * * *

6.X	Key Issue #X: Topology hiding in N9 interface

6.X.1	Key issue details

Figure 6.x-1 Example of N9 interface across PLMN operation domain and PNI-NPN operation domain

Considering the scenario depicted in Figure6.x-1, the dedicated UPF is controlled by SMF in service area B, and customers may access the DN through the UPF in service area A or the UPF in service area B depending on customers’ location. Attackers in PNI-NPN operational domain (e.g., a misbehaving employee in PNI-NPN or an external attacker gaining unauthorized access to the PNI-NPN networks) may collect the topology information of UPFs in the PLMN operation domain and launch further attacks to PLMN.

An firewall at the border between PLMN and NPN can do topology hiding at the IP layer. But the NFs in the PLMN operational domain don’t know the IP address after topology hiding, which would affect the operation of services. For example, suppose that the original IP address of the PLMN UPF in service area B is IP1, the firewall does topology hiding and the UPF in PNI-NPN operation domain will see the the IP address of PLMN UPF as IP2. When the PLMN SMF sends FAR to the UPF in PNI-NPN operation domain, since PLMN SMF doesn’t know IP2, tha IP address included in FAR will be IP1. When the UPF in PNI-NPN operation domain tries to forward packets to IP1, since it cannot reach IP1, the operation will fail.

IPUPS is supposed to enforce GTP-U security on the N9 interface between UPFs of the visited and home PLMNs. It is unknown if IPUPS can be used on the N9 interface between PLMN operation domain and PNI-NPN operation domain to provide topology hiding.

6.X.2	Security threats

When there is no mechanism to do topology hiding on the N9 interface between PLMN operation domain and PNI-NPN operation domain, attackers in the PNI-NPN operational domain may collect the topology information of UPFs in the PLMN operation domain and launch further attacks to PLMN.

6.X.3	Potential security requirements

The 5G system shall support a mechanism to support topology hiding on the N9 interface between PLMN operation domain and PNI-NPN operation domain.

* * * End of Changes * * * *