3GPP TSG-SA3 Meeting #123	S3-252673

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Huawei, HiSilicon

Title:	New key issue on security between Sensing Function(s) and Sensing Entities

Document for:	Approval

Agenda item:	6.1.8

Spec:	3GPP TS/TR <TS/TR number>

Version:	<TS version>

Work Item:	FS_Sensing_SEC

Comments

This contribution aims to provide a potential Key Issue for the Sensing study. The security between Sensing Function(s) and Sensing Entities needs to be protected otherwise cause attacks.

* * * First Change * * * *

Y.X	Key Issue #X: Security between Sensing Function(s) and Sensing Entities

Y.X.1	Key issue details

The connection between SF(s) and Sensing Entities (i.e. RAN) can be used to convey the control plane (CP) messages from SF to manage the Sensing tasks, as well as the Sensing data transmission from Sensing Entities. Failure to provide the security protection between SF(s) and Sensing entities may lead to various attacks by attackers or unauthorised entities, e.g. unauthorised Sensing control message from malicious entities, Sensing data manipulation, or potential leakage of privacy sensitive information. Therefore, the security aspects of the connection need to be assured and studied.

Y.X.2	Security threats

Without mutual authentication and authorisation are provided between SF(s) and Sensing Entities, the sensing operations can be triggered by an unauthenticated and/or authorised SF which may be a malicious entity, or an attacker can send forged sensing data to SF.

If the Sensing control signals are not integrity and replay protected, the parameters included can be modified, or replayed by an attacker. Consequently, the sensing may be executed with incorrect purpose/service requirements, or unauthorised triggered.

The Sensing data may be eavesdropped by an attacker if the Sensing data from Sensing entity to SF is not confidentiality protected.

Y.X.3	Potential security requirements

The 5G System shall support mutual authentication and authorisation between the Sensing Entity and SF(s).

The 5G System shall provide a means for confidentiality protection, integrity protection and replay protection of the connection between Sensing Entity and SF(s).

* * * End of Changes * * * *