3GPP TSG-SA3 Meeting #123	S3-252726

Goteborg, Sweden, 25 – 29 August 2025

Source:	Nokia

Title:	Resolution of EN’s concerning AIOT authentication and command protection.

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

This pCR proposes a resolution to the authentication procedure. The rational for the proposal is as follows. The asset to protect in case of inventory only, is a non-repudiation requirement of the presence of the device – the device owner shall not be able to claim that the device was not there. This implies that the network shall authenticate the device which ensures non-repudiation of the device’s presence. Therefore, device authentication is sufficient in the case of inventory only. In the case of inventory plus command, non-repudiation of the device and authenticity of the command are required to secure the procedure. This implies that, after the insurance of non-repudiation of the device, the device also needs to ensure the authenticity of the command. This is achieved by implicit network authentication through integrity protection of the command, using a derived key based on the root key and freshness parameters sent from network and device – the freshness parameters are included to ensure anti-reply of both authentication and command procedures.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TS 23.369: “Architecture support for Ambient power-enabled Internet of Things”.

[3]	3GPP TS 38.300: “NR; NR and NG-RAN Overall description; Stage-2”.

[4]	3GPP TS 22.369 “Service requirements for Ambient power-enabled IoT”.

[5]	3GPP TS 33.501 “Security architecture and procedures for 5G System”.

[6]	3GPP TS 38.391: "Ambient IoT Medium Access Control Protocol specification".

[b]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

* * * Next Change * * * *

### 4.2.2	Requirements on the AIOTF

#### 4.2.2.1	Requirement on Authentication

The AIOTF shall support device authentication for inventory only.

The AIOTF shall support mutual authentication for inventory plus command procedure.

* * * Next Change * * * *

#### 4.2.1.3	Requirements for command protection

The AIoT device shall support confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF.

Confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF is optional to use.

The AIoT device shall support the following ciphering algorithms:

NEA0 and 128-NEA2 as specified in Annex D of TS 33.501 [5].

The AIoT device shall support integrity protection of AIoT NAS messages between the AIoT device and the AIOTF.

Integrity protection of AIoT NAS messages between the AIoT device and the AIOTF is mandatory to use.

The AIoT device shall support the following integrity algorithms:

128-NIA2 as specified in Annex D of TS 33.501 [5].

Editor’s Note: For this clause privacy related requirements are FFS.

* * * Next Change * * * *

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed.

1. ADM shall generate RANDAIOT_n.

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN.

3. NG-RAN shall send the paging request message including RANDAIOT_n to the AIoT device.

4. Upon receiving the paging request message, AIoT device shall derive RESAIOT using KAIoT and RANDAIOT_n for network authenticating AIoT Device.

5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device.

NOTE: RANDAIOT_d is not required for inventory only but replied as it’s required for command procedure. As the device doesn’t have state awareness and it’s the network who initiates the command procedure, the device cannot decide when the RANDAIOT_d is required, therefore it’s always replied but for inventory only cases not used.

6.  NG-RAN sends Inventory report message to AIOTF, including the RESAIOT and RANDAIOT_d.

7. AIOTF sends device identifier and RANDAIOT_d to ADM.

8. ADM derives XRESAIOT using the same method as in AIoT device. If requested by the AIOTF, the ADM will derive the KAIOT_ses used during the command procedure.

9. ADM sends XRESAIOT, and if requested, KAIOT_ses to AIOTF.

10. AIOTF verifies RESAIOT. If the verification is successful, the steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 [2] continues.

NOTE: Network authentication prior to command procedure, is explicitly achieved through integrity protection of command messages utilizing the KAIOT_ses key.

### 5.2.3	Input parameters to RESAIOT and XRESAIOT calculation

When deriving RESAIOT and XRESAIOT from KAIOTF, the following parameters shall be used to form the input S to the KDF described in TS 33.220 [b].

-	FC = 0xcc,

-	P0 = RANDAIoT_n,

-	L0 = length of RANDAIoT_n

The input key KEY shall be KAIOT.

### 5.2.4	Input parameters to derive KAIOT_ses from KAIOT

When deriving KAIOT_ses from KAIOT, the following parameters shall be used to form the input S to the KDF described in TS 33.220 [b].

-	FC = 0xbb,

-	P0 = RANDAIoT_n,

-	L0 = length of RANDAIoT_n

-	P1 = RANDAIoT_d,

-	L1 = length of RANDAIoT_d

The input key KEY shall be KAIOT.

## 5.3	Protection of information during AIoT service communication

### 5.3.1	General

This clause describes the security procedures for the information protection in command message. The protection of information is provided as part of the AIoT NAS protocol between AIoT device and AIOTF. The AIOTF acts as the security termination point for AIoT information protection.

### 5.3.2	Security procedure on information protection during command procedure

Figure 5.3.2-1:  Security procedure on the information protection during command procedure

1.	The command procedure is initiated as specified in step 1-6 of clause 6.2.3 of TS 23.369 [2].

2.	The procedure as described in clause 5.2.2 shall be performed. The device and AIOTF acquire the KAIOT_ses key to be used for command protection from the ADM.

3.	The AIoTF shall construct a NAS Command Request and protect the message based on the KAIoT_ses, the confidentiality and integrity algorithms for the AIoT device. The AIoTF shall send the Command Request containing the protected NAS Command Request to NG-RAN. The selection of integrity and confidentiality protection algorithm is done by the AIoTF.

4.	The NG-RAN shall send a R2D message containing the protected NAS Command Request as specified in as specified in TS 38.300 [3] and TS 38.391 [6].

5.	The device shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. The AIoT device shall construct a NAS Command Response and protect the message based on the KAIoT_ses key using the same algorithms.

6.	The AIoT device shall send a D2R message containing the protected NAS Command Response to the NG-RAN as specified in as specified in TS 38.300 [3] and TS 38.391 [6].

7.	The NG-RAN shall forward the Command Response containing the protected NAS Command Response to the AIoTF.

8-9.	The AIoTF shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. Then, the AIOTF shall continue the procedure as specified in clause 6.2.3 of TS 23.369 [2].

### 5.3.3	Input parameters to integrity algorithm

The input parameters to the integrity algorithm as described in Annex D.3 in TS 33.501[5] shall be set as follows.

The KEY input is equal to the KAIOT_ses key.

The DIRECTION bit is set to 0 for uplink and 1 for downlink.

The BEARER is set to all zeros.

The COUNT is set to all zeros.

* * * End of Change * * * *