3GPP TSG-SA3 Meeting #123	S3-252819

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.938 | CR | 0006 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minor Typos and Consistency-related Edits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_CryptoInv |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current version of the TR has some typos and consistency related issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fixed a typo in 4.2.12. Made the different entries in Table 4.3.1-1 and Table 4.3.2-1 consistent with each other. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.12, 4.3.1, 4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

### 4.2.12	NAS security

The NAS security mechanisms are to protect NAS signaling and data between the UE and the AMF over the N1 reference point in 5G system:

-	NAS signaling integrity and confidentiality protection between UE and AMF (see clause 6.4.3 and 6.4.4 of TS 33.501 [4]).

-	User plane data (SMS over NAS) integrity and confidentiality protection between UE and AMF (see clause 6.4.7 and 6.16 of TS 33.501 [4]).

NAS security protocol employs symmetric cryptography for confidentiality and integrity protection.

* * * Next Change * * * *

### 4.3.1	3GPP Symmetric Cryptographic Algorithms

The following table summarizes the security related protocols used in 3GPP employing symmetric cryptographic algorithms including hash functions (5G System).

Table 4.3.1-1: Protocols Used in 3GPP Employing Symmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| MIKEY-SAKKE (IETF RFC 6509) [14] | IETF RFC 6509 [14], Appendix A | SHA256 | Hash Function |
| KDF(TS 33.220, Clause B.2 [11]) | TS 33.220 [11], Clause B.2.0 | HMAC-SHA-256 | Session Key Derivation |
|  | TS 33.501 [4], Clause C.3 | ANSI-X9.63-KDF | Session Key Derivation |
| ECIES ([7], [8]) | TS 33.501 [4], Clause C.3 | SHA-256, HMAC-SHA-256, | Session Key Derivation |
|  |  | HMAC–SHA-256 | Integrity Protection |
|  |  | AES-128-CTR | Confidentiality Protection |
| PKI | TS 33.310 [3], Clause 6.1.1 | SHA-256 SHA-384 | Hash Function |
| OCSP (IETF RFC 6960 [22]) | TS 33.310 [3], Clause 6.1b | SHA-256 SHA-384 | Hash Function |
| COSE (IETF RFC 8152[10]) | TS 33.220 [11], Clause P.3.3 | HMAC-based KDF with SHA-256 [31] | Session Key Derivation / Hash Function |
|  |  | AES-CCM-16-64-128 | Confidentiality and Integrity Protection |
| EAP-TLS (IETF RFCs 9190 [5], 5216 [6]) | TS 33.501 [4], Clause B.2.1 | AEAD_AES_128_GCM | Confidentiality and Integrity Protection |
|  |  | HKDF (RFC5869 [31] | Session Key Derivation |
| EAP TTLS (IETF RFC 5281 [39]) | TS 33.501 [4], Annex U TS 33.210 [2] clause 6.2 for TLS | See TLS in this table | Confidentiality and Integrity Protection |
|  |  |  | Session Key Derivation |
| OAuth 2.0 (IETF RFC 6749 [40], 6750 [41]) | TS 33.210 [2] clause 6.2 for TLS | See TLS 1.2 and TLS 1.3 in this table | Confidentiality and Integrity Protection |
|  |  |  | Hash Function |
|  | TS 33.210 [2] clause 6.3 for JWE/JWS | See JWE and JWS in this table | Confidentiality and Integrity Protection |
|  |  |  | Hash Function |
| IKEv2 (IETF RFC 7296 [23]) | TS 33.210 [2] clause 5.4 | 128-AES GCM SHA256 (IETF RFC 8442 [x2]). 256-AES GCM SHA384 (IETF RFC 8442 [x2]). | Confidentiality and Integrity Protection |
|  | TS 33.310 [3] clauses 5,6,7 | SHA2-256/384 [x4] | Hash Function |
| PDCP security (TS 38.323 [x1]) | TS 33.501 [4], Annex D | 128-NEA1, 128-NIA1 128-NEA2, 128-NIA2 128-NEA3, 128-NIA3 | Confidentiality and Integrity Protection |
| NAS security (TS 33.501 [4]) | TS 33.501 [4], Annex D | 128-NEA1, 128-NIA1 128-NEA2, 128-NIA2 128-NEA3, 128-NIA3 | Confidentiality and Integrity Protection |
| IPsec ESP (IETF RFCs 4303 [32], 8221 [25], 8750 [26]) | TS 33.210 [2] | ENCR_AES_CBC (IETF RFC 3602 [33]) | Confidentiality Protection |
|  |  | ENCR_AES_GCM_16 (IETF RFC 4106 [34]) ENCR_AES_GCM_16_IIV (IETF RFC 8750 [26]) | Confidentiality and Integrity Protection |
|  |  | AUTH_AES_128_GMAC (IETF RFC 4543 [35]) AUTH_HMAC_SHA2_256_128 (IETF RFC 4868 [36]) | Authentication |
| DTLS 1.2 (IETF RFC 6347 [37]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.2 in this table | Confidentiality and Integrity Protection |
| DTLS 1.3 (IETF RFC 9147 [20]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.3 in this table | Confidentiality and Integrity Protection |
| TLS 1.2 (IETF RFC 5246 [38]) | TS 33.210 [2] clauses 6.2.1, 6.2.3 | AES_128_GCM, AES_256_GCM | Confidentiality and Integrity Protection |
|  |  | SHA256, SHA384 | Hash Function |
| TLS 1.3 (IETF RFC 8446 [21]) | TS 33.210 [2] clauses 6.2.1, 6.2.2 | AES_128_GCM, AES_256_GCM, CHACHA20_POLY1305 | Confidentiality and Integrity Protection |
|  |  | SHA256, SHA384 | Hash Function |
| JWE (IETF RFC 7516 [27]) | TS 33.210 [2] clauses 6.3.1, 6.3.2 | AES_128_GCM, AES_256_GCM | Confidentiality and Integrity Protection |
| JWS (IETF RFC 7515 [28]) | TS 33.210 [2] clauses 6.3.1, 6.3.3 | SHA256 | Hash Function |

* * * Next Change * * * *

### 4.3.2	3GPP Asymmetric Cryptographic Algorithms

The following table summarizes the security related protocols used in 3GPP employing asymmetric cryptographic algorithms (5G System).

Table 4.3.2-1: Protocols Used in 3GPP Employing Asymmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| MIKEY-SAKKE (IETF RFC 6509) [14] | IETF RFC 6507 [29] | ECCSI | Digital signature |
|  | IETF RFC 6508 [30] | SAKKE | Key agreement |
| ECIES ([7], [8]) | TS 33.501 [4], Clause C.3 | ECDH | Key Agreement |
| PKI | TS 33.310 [3], Clause 6.1.1 | RSA, ECDSA | Authentication / Digital Signature |
| OCSP (IETF RFC 6960 [22]) | TS 33.310 [3], Clause 6.1b | RSA ECDSA | Authentication / Digital Signature |
| EAP-TLS (IETF RFCs 9190 [5], 5216 [6]) | TS 33.501 [4] ], Clause B.2.1 | See TLS in this table. | Authentication / Digital Signature / Confidentiality Protection / Hash Function |
|  | TS 33.501 [4] RFC 9190 (TLS1.3) [5] | ECDHE | Key Agreement |
| EAP-TTLS (IETF RFC 5281 [39]) | TS 33.501 [4], Annex U TS 33.210 [2] clause 6.2 for TLS | See TLS in this table | Key Agreement |
|  |  | See TLS in this table | Authentication / Digital Signature / Confidentiality Protection / Hash Function |
| OAuth 2.0 (IETF RFC 6749 [40], 6750 [41]) | TS 33.210 [2] clause 6.2 for TLS | See TLS 1.2 and TLS 1.3 in this table | Key Agreement |
|  |  |  | Digital Signature |
|  | TS 33.210 [2] clause 6.3 for JWE/JWS | See JWE and JWS in this table | Key Agreement |
|  |  |  | Digital Signature |
| IKEv2 (IETF RFC 7296 [23]) | TS 33.210 [2] clause 5.4 | DH | Key Agreement |
|  | TS 33.310 [3] clauses 5,6,7 | RSA Sha-256/384 (IETF RFC 8017 [x3]) ECDSA SHA-256/384/512 (IETF RFC 4754 [x4]) RSASSA-PSS SHA-256 [x4] | Digital Signature |
|  |  | SHA2-256/384 [x4] | Hash Function |
| DTLS 1.2 (IETF RFC 6347 [37]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.2 in this table | Confidentiality and Integrity Protection |
| DTLS 1.3 (IETF RFC 9147 [20]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.2 in this table | Confidentiality and Integrity Protection |
| TLS 1.2 (IETF RFC 5246 [38]) | TS 33.210 [2] clauses 6.2.1, 6.2.3 | ECDHE | Key Agreement |
|  |  | ECDSA, RSA | Digital Signature |
| TLS 1.3 (IETF RFC 8446 [21]) | TS 33.210 [2] clauses 6.2.1, 6.2.2 | ECDHE | Key Agreement |
|  |  | ECDSA, RSA | Digital Signature |
| JWE (IETF RFC 7516 [27]) | TS 33.210 [2] clauses 6.3.1, 6.3.2 | ECDH-ES | Key Agreement |
| JWS (IETF RFC 7515 [28]) | TS 33.210 [2] clauses 6.3.1, 6.3.3 | ECDSA | Digital Signature |

* * * End of Changes * * * *