# 3GPP TSG SA WG3 (Security) Meeting #67	S3-120552

# 21 – 25 May 2012, Kyoto, Japan

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.828 | CR | 0003 | rev | - | Current version: | 9.1.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MIKEY-IBAKE deferred delivery solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone, Alcatel Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eMEDIASEC |  |  |  |  |  |  | Date: |  | 2012-05-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | The current MIKEY-IBAKE deferred delivery solution imposes additional requirements on functionality of the mailbox to protect against alteration of deposited messages |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of text in section 7.6.2.5 to describe mailbox security provisions that are needed to protect against source spoofing, as well as dependency on IMS security in order to prevent alteration of deposited messages. Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Required security functionality for the mailbox when used with MIKEY-IBAKE is not identified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Start of changes ***

#### 7.6.2.5	Deferred delivery

In this section, deferred delivery is discussed for the case of MIKEY-IBAKE. Deferred delivery is type of service such that the session content cannot be delivered to the destination at the time that it is being sent (e.g., the destination user is not currently online). Nevertheless, the sender expects the network to deliver the message as soon as the recipient becomes available. A typical example of deferred delivery is voicemail.

Below, the basic scenario of deferred delivery for the case of MIKEY-IBAKE is presented. In the scenario (Figure 27) presented, user A and B’s mailbox perform mutual authentication before they agree on the key to be used for decrypting the content of the message intended for deferred delivery.

In the scenario depicted in Figure 27, it is assumed that the user A is trying to reach the user B, who is currently not available, therefore the call is forwarded to the B’s ‘voicemail’ (more generally deferred delivery server). Following the MIKEY-IBAKE protocol, the message received in step 2 by the B’s mailbox is encrypted using B’s identity, therefore B’s mailbox will not be able to decrypt it. B’s mailbox chooses random y and computes yP and send its identity and yP IBE-encrypted to the user A. The user A recognizes that the B did not receive the message and that the actual recipient was not able to decrypt the message sent in step 1 by the lack of its identity and xP in the message received in step 4. Therefore, the user sends a new message containing A’s identity, B’s mailbox identity, xP and yP all IBE-encrypted using B’s mailbox identity. The user A also chooses a random a and includes its identity and aP encrypted using B’s public key,. Upon reception of this message, the B’s mailbox accepts aP as the session key for the message intended for B and returns A’s identity and xP to the user A to complete the authentication. Subsequently, when B is online and checks ‘voicemail’ (checks with the deferred delivery server), B can obtain the encrypted value of aP from the mailbox server.

Note that B may have to authenticate with the mailbox to obtain the key (this is not shown in the figure below) – this could be based on existing authentication mechanisms already in place.

Note also that the mailbox needs to establish a secure communication path to B and communicate the original authenticated user’s identity with every message.

In addition, the mailbox needs to implement a MIKEY-IBAKE client to mutually authenticate with A and B in order to protect against A spoofing the source of deposited messages towards B.

Note further, that in order to prevent alterations of deposited multimedia messages by the man-in-the-middle attacker, the integrity protection of IMS transport is expected. In order to achieve full end-to-end protection at the IBAKE layer for the deferred delivery solution, further modification of the protocol may be needed.

The practicality of addressing these additional requirements on mailbox security and potentially needed modifications to the IBAKE protocol need to be taken into account when deciding the suitability of MIKEY-IBAKE for deferred delivery scenarios.

Figure 27: Deferred Delivery

*** End of changes ***