Third Generation Partnership Project (3GPP™)

Meeting Report
for
TSG SA WG3
meeting: 73

San Francisco, USA, 11/11/2013 to 15/11/2013

Note: Hyperlinks to tdocs will work only if the present report is in a subdirectory named "report" and the tdocs themselves are in a parallel subdirectory names "docs".

TABLE OF CONTENTS

1	Opening of the Meeting	3

2	Approval of Agenda and Meeting Objectives	3

3	IPR Reminder	3

4	Meeting Reports	3

4.1	Approval of the Report from SA3 #72	3

4.2	Report from SA #61	3

4.3	Report from SA3-LI	3

5	Items for early consideration	4

6	Reports and Liaisons from other Groups	9

6.1	3GPP Working Groups	9

6.2	IETF	10

6.3	ETSI SAGE	10

6.4	GSMA	10

6.5	3GPP2	10

6.6	OMA	10

6.7	TCG	10

6.8	oneM2M	11

6.9	Other Groups	11

7	Work Areas	11

7.1	IP Multimedia Subsystem (IMS) Security	11

7.1.1	Media Plane Security	11

7.1.1.1	Study on Extended IMS media plane security features	11

7.1.1.2	Extended IMS media plane security features	11

7.1.2	  Security Aspects of Web Real Time Communication (WebRTC) Access to IMS	13

7.1.3	Other Common IMS Issues	16

7.2	Network Domain Security	16

7.3	UTRAN Network Access Security	16

7.4	GERAN Network Access Security	16

7.5	GAA	16

7.5.1	 Security enhancements for usage of GBA from the browser	16

7.5.2 	GBA extensions for re-use of SIP Digest credentials	17

7.5.3	 Other GAA Issues	17

7.6	Multimedia Broadcast/Multicast Service (MBMS)	20

7.7	SAE/LTE Security	20

7.7.1	TS 33.401 Issues	20

7.7.2	TS 33.402 Issues	20

7.7.3	Other SAE/LTE Security Issues	21

7.8	Security Aspects of Home (e)NodeB	21

7.8.1	TS 33.320 Issues	21

7.8.2	TR 33.820 Issues	21

7.9	Security Aspects related to Machine-Type Communication	21

7.10 	Security Aspects of Public Warning System	33

7.11	Firewall Traversal	38

7.11.1	Study on Firewall Traversal	38

7.11.2	Tunnelling of UE Services over Restrictive Access Networks	39

7.12 	Security Aspects of WLAN Network Selection for 3GPP Terminals	39

7.13	Security Aspects of Proximity-based Services	40

7.14	Security Aspects of Group Communication System Enablers for LTE	54

7.15	Other areas	60

8	Studies	65

8.1 	Study on Security aspects of Integration of Single Sign-On (SSO) frameworks with 3GPP networks	65

8.2 	Security Study on Spoofed Call Detection and Prevention	67

8.3 	Study Item on Security Assurance Methodology for 3GPP Network Elements	68

8.4 	Study on Subscriber Privacy Impact in 3GPP	75

8.5	Other Study Areas	78

10	Elections	78

11	Review and Update of Work Plan	78

12	Future Meeting Dates and Venues	79

13	Any Other Business	80

## 1	Opening of the Meeting

The Chairman opened the meeting at 0900 Monday the 11th November 2013. Adrian Escott gave a few words on behalf of the host and welcomed the delegates to San Francisco.

## 2	Approval of Agenda and Meeting Objectives

The agenda was approved without comments.

S3-130900	Agenda
					Source: WG Chairman

Discussion:

The Chairman presented the agenda and his proposed schedule for the meeting. A few comments recommended efficient meeting procedures, avoiding representing proposals earlier presented. Evening session proposed, to be decided during the meeting.

Decision: 		The document was approved.

## 3	IPR Reminder

The Chairman presented the 3GPP IPR policy:

The attention of the delegates to the meeting of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates were asked to take note that they were thereby invited:

- to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

- to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms

## 4	Meeting Reports

The report from previous meeting was approved without comments.

### 4.1	Approval of the Report from SA3 #72

S3-130903	Report from last SA3 meeting
					Source: ETSI Secretariat

Discussion:

Not presented. No comments.

Decision: 		The document was approved.

### 4.2	Report from SA #61

S3-130901	Report from last SA meeting
					Source: WG Chairman

Discussion:

Presented by the Chairman. For information. No comments.

Decision: 		The document was noted.

### 4.3	Report from SA3-LI

Alex Leadbeater informed from SA3-LI.

Last SA3-LI meeting Burlington 22nd - 24th Oct. Important topic discussed included; Separated delivery of messaging (SMS and MMS), IMS IMEI, WebRTC/Joyn/VoLTE, and Packet header reporting.

CR approval deadline passed 8th Nov with no comments received.

SA3-LI intend to follow the stage 3 deadlines for 33.106,107,108 R12.

Next meeting 4th - 6th Feb ETSI.

## 5	Items for early consideration

S3-130950	LS to SA3 on security aspects of CMPv2 protocol used for certificate enrolment procedures in MvPnP
					Source: S5-131458

Discussion:

SA5 would like to get guidance on whether SA3 believes providing the possibility to protect the CA/RA server behind a SeGw would be worthwhile for less trusted networks.

Decision: 		The document was noted.

S3-131004	Deployment option for CA/RA
					Source: Ericsson

Discussion:

Document concludes that it  does seem worthwhile to allow the deployment option of using a SEG in front of the CA/RA when enrolling certificates to entities connected to an untrusted network such as the internet.

Decision: 		The document was noted.

S3-131005	Draft LS reply security aspects of CMPv2 protocol used for certificate enrolment procedures in MvPnP
					Source: Ericsson

Discussion:

Attack scenario where someone gets access to eNB with valid IPSec certificate to be clarified.

Decision: 		The document was revised to S3-131116.

S3-131116	Draft LS reply security aspects of CMPv2 protocol used for certificate enrolment procedures in MvPnP
					Source: Ericsson

(Replaces S3-131005)

Decision: 		The document was approved.

S3-130952	LS to SA3 on necessary information for certificate enrolment procedures in MvPnP
					Source: S5-131789

Discussion:

SA5 would like to get guidance on whether SA3 believes it’s sufficient for eNB to know the IP address or the FQDN of the CA/RA in order to be able to perform the certificate enrolment. If any other additional information about CA/RA is needed at the eNB for certificate enrolment, SA5 kindly ask SA3 to list it

Decision: 		The document was replied-to.

S3-131071	Draft reply to SA5 on 'necessary information for certificate enrolment procedures in MvPnP'
					Source: Nokia Corporation, NSN, Huawei, Hisilicon

Discussion:

Presented by Guenter Horn, NSN.

A few clarifications were required.

Decision: 		The document was revised to S3-131118.

S3-131118	Draft reply to SA5 on 'necessary information for certificate enrolment procedures in MvPnP'
					Source: Nokia Corporation, NSN, Huawei, Hisilicon

(Replaces S3-131071)

Discussion:

SA3 would like to reply as follows:

The following parameters are mandatory to provide to the eNB for performing an automated certificate enrolment:

1.	IP address or FQDN of the CA/RA

2.	Port number of the CA/RA

3.	Path to the CA/RA directory

4.	Subject name of the CA/RA

The following parameter is optional to provide to the eNB:

5. An indication of the protocol (HTTP or HTTPS).

Approved with modification.

Decision: 		The document was approved.

S3-130940	LS on new work item on 'IMS support for RTP / RTCP transport multiplexing
					Source: C4-131546

Discussion:

CT4 has started work on the introduction of support for RTP / RTCP transport multiplexing, (i.e. using a single IP transport port for RTP and RTCP traffic). Usage of RTP / RTCP transport multiplexing allows to enhance NAT traversal (NAT-T) support. The number of underlying IP transport connections for RTP needed to traverse NAT/FW devices is reduced

Decision: 		The document was replied to.

S3-131133	Reply to: LS on new work item on 'IMS support for RTP / RTCP transport multiplexing
					Source: Unknown

Discussion:

Not available

Decision: 		The document was withdrawn.

S3-130948	Reply LS to CT WG4 LS on new work item on 'IMS support for RTP / RTCP transport multiplexing
					Source: S2-133825

Discussion:

3GPP SA WG2 would like to thank CT WG4 for the LS.  SA WG2 takes note of the start of the new work item and requests CT WG4 to keep SA WG2 informed on the progress of work in this area to ensure any stage 2 impacts are captured by SA WG2 appropriately.

Decision: 		The document was noted.

S3-131114	Reply LS on new work item on IMS support for RTP / RTCP transport multiplexing
					Source: S4-131309

Discussion:

Presented by Oscar/Ericsson.

For information only. No impact expected.

Decision: 		The document was noted.

S3-130943	LS on security aspects of protocol architectures for small cell enhancements
					Source: R2-133018

Discussion:

RAN2 is currently investigating the potential higher layer enhancements for small cell deployment scenarios according to Study Item description in RP-122033. This activity involves the Radio Access work area of the studies and has impacts both on the Mobile Equipment and Access Network of the 3GPP systems.

One area of study is the provision of dual connectivity to the UE. In dual connectivity, the UE consumes radio resources provided by at least two different network points (Master and Secondary eNBs) connected via a non-ideal backhaul. The Master eNB (MeNB) is the eNB which hosts the RRC layer, terminates S1-MME and therefore acts as mobility anchor towards the CN. The Secondary eNB (SeNB) is an eNB providing additional radio resources for the UE. The eNB configured as SeNB for a given UE could also be operated as a normal cell for standalone UEs.

RAN2 is currently working with the baseline assumption for C-plane architecture that there is no termination for the RRC protocol in the SeNB towards the UE with dual connectivity.

With regards to U-plane architecture, a number of protocol architecture options are under discussion for small cell higher layer enhancements and are captured in TR 36.842. They mainly differ in how S1-U terminates: at MeNB always (Option 2 and 3) or at SeNB for offloaded bearers (Option 1).

Decision: 		The document was noted.

S3-130944	Reply LS on security aspects of protocol architectures for small cell enhancements
					Source: R2-133650

Decision: 		The document was noted.

S3-130945	Reply LS on security aspects of protocol architectures for small cell enhancements
					Source: R3-131919

Decision: 		The document was noted.

S3-130956	Proposed Reply LS on security aspects of protocol architectures for small cell enhancements
					TS 33.401 v..
					Source: Alcatel-Lucent

Abstract:

SA3 response to the LSs (R2-133018, R2-133650) on requesting input on security aspects of protocol architecture alternatives 1A and 3C for small cell enhancements.

Discussion:

Dealt with during offline drafting session.

Issue A: Both architectural alternatives 1A and 3C are technically feasible. Architecture 1A requires slight enhancement to the current security framework in terms of key management in the UE.

Option 3C imposes additional requirement on security of the Xn interface. This may add an IPSec processing and transport delays. Option 1A doesn’t have this latency issue on the Xn interface.

Based on the above, alternative 1A and 3C are technically feasible with different security impacts.

Issue B: In option 1A, user traffic doesn’t traverse Xn interface hence doesn’t add any transport and processing delay.

In option 3C, when Xn interface is protected using the IPSec as specified in TS 33.210, additional transport and processing delays can be expected

Decision: 		The document was noted.

S3-131121	Merger contribution of S3-130965 and S3-131106, GCSE_LTE security architecture
					TS 33.401 v..
					Source: Huawei, HiSilicon, NSN

(Replaces S3-130965, S3-131106)

Discussion:

Merger of 0965 and 1106.

Agreed for inclusion into draft TR.

Decision: 		The document was approved.

S3-130957	Discussion on security aspects of protocol architectures for small cell enhancements
					TS 33.401 v..
					Source: Alcatel-Lucent

Abstract:

RAN2 sent an LS (R2-133018) followed up with another LS (R2-133650) to SA3 informing SA3 about selection of architectures (architecture 1A and 3C) for small cell enhancements. This paper discusses potential security impact of these architectural enhancement

Discussion:

Not presented. Dealt with during offline drafting session.

Decision: 		The document was noted.

S3-131017	Small cell enhancements security aspects
					Source: NEC Corporation

Discussion:

Not presented. Dealt with during offline drafting session.

Decision: 		The document was noted.

S3-131001	Draft reply LS on security aspects of protocol architectures for small cell enhancements
					Source: NEC Corporation

Discussion:

Not presented. Dealt with during offline drafting session.

Decision: 		The document was noted.

S3-130971	draft Response LS on security aspects of protocol architectures for small cell enhancements
					Source: Huawei, HiSilicon

Discussion:

Not presented. Dealt with during offline drafting session.

Decision: 		The document was noted.

S3-130989	Discussion on Security Aspects of SCE UP Architecture
					Source: Samsung

Discussion:

Not presented. Dealt with during offline drafting session.

Decision: 		The document was noted.

S3-130990	Discussion on Reply LS on Security Aspects of Protocol Architectures for SCE
					Source: Samsung

Discussion:

Not presented. Dealt with during offline drafting session.

Decision: 		The document was noted.

S3-131006	Security analysis of small cell split PDCP architectures
					Source: Ericsson

Discussion:

Not presented. Dealt with during offline drafting session.

Decision: 		The document was noted.

S3-131007	Draft LS reply on small cell split PDCP architectures
					Source: Ericsson

Discussion:

Not presented. Dealt with during offline drafting session.

Decision: 		The document was noted.

S3-131117	Reply LS on security aspects of protocol architectures for small cell enhancements
					Source: SA3

Discussion:

Reply to S3-130943.

Substantial debate on a draft version regarding phrasing along lines of "alternative 3C prohibits local break out..." and talk about no show stoppers. Multiple draft updates during the meeting.

Decision: 		The document was approved.

## 6	Reports and Liaisons from other Groups

### 6.1	3GPP Working Groups

S3-131000	Small cell enhancements security aspects
					Source: NEC Corporation

Decision: 		The document was withdrawn.

S3-131169	Reply LS on FQDN formats used in MvPnC procedures
					Source: C4

Discussion:

Erroneously address to S3

Decision: 		The document was noted.

S3-131170	LS on FQDNs to support Multi-vendor Plug and Play (MvPnP) feature
					Source: CT4

Discussion:

Erroneously address to S3

Decision: 		The document was noted.

### 6.2	IETF

Chairman reported on the work in IETF. Tracking issues are ongoing, link to be provided. Discussion on contentious VO codec.

### 6.3	ETSI SAGE

### 6.4	GSMA

It was noted there was incoming LS.

### 6.5	3GPP2

The document S.S0152 (Security Framework for Binding of Access Subscription with Devices), available @ http://www.3gpp2.org/Public_html/Misc/SP0152v0.3-V&V-version-clean.pdff, that specifies a network based enforcement solution based on MEID authentication is currently undergoing V&V (verification & validation) process. The document is expected to be published at the 3GPP2 December 2013 meeting.   The necessary stage 3 enhancements for 1x air interface and network enhancements has also been base lined.

### 6.6	OMA

### 6.7	TCG

S3-130936	TCG progress report in the areas of TMS and MPWG
					n/a v..
					Source: Interdigital

Abstract:

This contribution provides a brief summary of the progress in TCG Trusted Mobility Solutions (TMC) and Mobile Platform (MPWG) working groups.

Discussion:

Approved for inclusion in meeting report

Decision: 		The document was approved.

### 6.8	oneM2M

### 6.9	Other Groups

## 7	Work Areas

### 7.1	IP Multimedia Subsystem (IMS) Security

#### 7.1.1	Media Plane Security

##### 7.1.1.1	Study on Extended IMS media plane security features

##### 7.1.1.2	Extended IMS media plane security features

S3-130938	LS on SIP/SDP Usage for TCP/TLS session establishment
					Source: C4-131508

Discussion:

Presented by Richard.

CT4 is currently specifying media security for MSRP and BFCP that use TCP transport. In preparation of the normative work, CT4 works on TR 29.828 (Study on Extended IMS media plane security features and TCP-related NAT traversal support; IMS H.248 profiles aspects).

CT4 identified that TLS endpoints require knowledge whether to act as TLS client or server and sees a need to clarify some related aspects.

Decision: 		The document was noted.

S3-130935	LS on TLS protocol profile for eMediasec
					Source: C4-131868

Discussion:

CT4 is progressing its study on the IMS H.248 profiles requirements and procedures to support the stage 2 requirements specified in 3GPP TS 33.328 for Extended IMS media plane security features.

As part of this work, it has come to the attention of CT4 that while TS 33.328 specifies a TLS profile for the Mikey-Ticket case in Annex H, it does not contain a detailed description of the TLS profile for extended IMS media plane security in general (TLS versions, cipher suites, keys, compression methods, certificates, supported TLS procedures and extensions...).

CT4 kindly asks SA3 to consider updating their specification(s) with the description of the TLS profile applicable for extended IMS media plane security.

Decision: 		The document was noted.

S3-130928	Updated reference for secure fax
					33.328	  CR-0056  (-) v..
					Source: Ericsson

Discussion:

Revised before presentation.

Decision: 		The document was revised to S3-131099.

S3-130951	Addition of TLS profile for eMediaSec
					33.328	  CR-0057  (Rel-12) v..
					Source: Alcatel-Lucent

Abstract:

Missing TLS profile details in 33.328.  1) In 33.328 Sec 4.1.2.3 and 5.5.4 References to general TLS profile in 33.310 are added.  2) Annex H is expanded for TLS eMediaSec specific considerations

Discussion:

Revised before presentation.

Decision: 		The document was revised to S3-131102.

S3-131099	Updated reference for secure fax
					33.328	  CR-0056  rev 1 (Rel-12) v12.4.0
					Source: Ericsson

(Replaces S3-130928)

Discussion:

No comments.

Decision: 		The document was agreed.

S3-131102	Addition of TLS profile for eMediaSec
					 33.328	  CR-0057  rev 1 (Rel-12) v..
					Source: Alcatel-Lucent

(Replaces S3-130951)

Abstract:

Updated CR S3-130951

Discussion:

Revised before presentation.

Decision: 		The document was revised to S3-131127.

S3-131127	Addition of TLS profile for eMediaSec
					33.328	  CR-0057  rev 2 (Rel-12) v12.4.0
					Source: Alcatel-Lucent

(Replaces S3-131102)

Discussion:

No comments. Auto agreed

Decision: 		The document was agreed.

#### 7.1.2	  Security Aspects of Web Real Time Communication (WebRTC) Access to IMS

S3-130979	Discussion: Work plan and prioritization for the WebRTC security work
					Source: Ericsson

Discussion:

Presented by Ericsson. Comment that of the three scenarios, one is scheduled for Rel-13 and two for Rel-12.

Decision: 		The document was noted.

S3-130980	Update of WebRTC WID
					Source: Ericsson

Discussion:

Comment that Rel-12 is an unrealistic release target for this WID.

Decision: 		The document was approved.

S3-130981	(Draft) LS on WebRTC Security Responsibility
					Source: Ericsson

Discussion:

According to the WebRTC IMS Client access to IMS WID (SP-130459), SA3 has the responsibility to drive the security aspects including:

•	WebRTC client authentication mechanisms

•	Control plane security aspects

•	Enhancements to IMS media plane security

For this reason SA3 has updated the WebRTC IMS Client access to IMS WID to include a new SA3 TR. The work will follow the architectural decisions made by SA2. As SA2 has already done some security related work, SA3 has incorporated the SA2 work into the SA3 TR.

Attachments:	S3-xxxx (Updated WebRTC WID)

S3-13xxxx (TR 33.abc)

Decision: 		The document was revised to S3-131193.

S3-131193	(Draft) LS on WebRTC Security Responsibility
					Source: Ericsson

(Replaces S3-130981)

Discussion:

According to the WebRTC IMS Client access to IMS WID (SP-130459), SA3 has the responsibility to drive the security aspects including:

•	WebRTC client authentication mechanisms

•	Control plane security aspects

•	Enhancements to IMS media plane security

For this reason SA3 has updated the WebRTC IMS Client access to IMS WID to include a new SA3 TR. The work will follow the architectural decisions made by SA2. As SA2 has already done some security related work, SA3 has incorporated the SA2 work into the SA3 TR.

Attachments:	S3-xxxx (Updated WebRTC WID)

S3-13xxxx (TR 33.abc)

Decision: 		The document was approved.

S3-130982	WebRTC TR 33.abc skeleton
					Source: Ericsson

Discussion:

Only outstanding issue is a comment that it was premature to include 6.1.3 in the skeleton. Alcatel-Lucent, China Mobile  prefers it to stay there. Telecom Italia suggest to make progress on technical contributions and decide the faith of 6.1.3 afterwards.

Decision: 		The document was revised to S3-131192.

S3-131192	WebRTC TR 33.abc skeleton
					Source: Ericsson

(Replaces S3-130982)

Discussion:

Updated skeleton. Approved as basis for further work.

Decision: 		The document was approved.

S3-131026	pCR 33.abc Overview
					Source: Ericsson

Discussion:

Comment that server function is missing from figure 2. Need to be aligned with outcome of SA2.

IETF references to be added to clarify bullets in 4.1.3.

Decision: 		The document was revised to S3-131122.

S3-131122	pCR 33.abc Overview
					Source: Ericsson

(Replaces S3-131026)

Discussion:

Presented as draft. Editorial comments.

Decision: 		The document was approved.

S3-131013	pCR to TR 33.abc: [Study on Security for WebRTC IMS Client access to IMS]
					TR 33.abc: [Study on Security for WebRTC IMS Client access to IMS] v..
					Source: Alcatel-Lucent

Abstract:

This pCR proposes the addition of solution text for the three types of authentication in clause 6.1.

Discussion:

Presented by 	Richard.	This pCR proposes the addition of solution text for the three types of authentication in clause 6.1.

To include 1027 and 1028 and notes reflecting the discussion.

Decision: 		The document was revised to S3-131123.

S3-131123	pCR to TR 33.abc: [Study on Security for WebRTC IMS Client access to IMS]
					TR 33.abc: [Study on Security for WebRTC IMS Client access to IMS] v..
					Source: Alcatel-Lucent

(Replaces S3-131013)

Discussion:

Presented as draft. Discussion on editors note on SIP Digest. Approved with modifications.

Decision: 		The document was approved.

S3-131027	pCR 33.abc Authentication of WebRTC client re-using existing IMS authentication mechanisms
					Source: Ericsson

Discussion:

Presented by Oscar.

Decision: 		The document was revised to S3-131123.

S3-131028	pCR 33.abc Authentication of WebRTC client using web credentials
					Source: Ericsson

Discussion:

Presented by Oscar.

Decision: 		The document was revised to S3-131123.

S3-131029	pCR 33.abc Use of DTLS-SRTP for SRTP key management
					Source: Ericsson

Discussion:

Presented by Oscar.

Richard: flow figure need to be updated.

Rel-12 vs. Rel-13 doubts. So far go for Rel-12.

Discussion on exception sheet (align with SA2)

Decision: 		The document was revised to S3-131124.

S3-131124	pCR 33.abc Use of DTLS-SRTP for SRTP key management
					Source: Ericsson

(Replaces S3-131029)

Abstract:

Adds text to clause 6.2 Enhancements to IMS media plane security describing the use of DTLS-SRTP for SRTP key management

Decision: 		The document was approved.

S3-131125	Draft TR WebRTC security
					Source: Rapporteur

Discussion:

Study on Security for WebRTC IMS Client access to IMS.

Approved as basis for further work.

Decision: 		The document was approved.

S3-131126	Exception sheet for TR WebRTC security
					Source: Rapporteur

Decision: 		The document was left for email approval and approved.

#### 7.1.3	Other Common IMS Issues

### 7.2	Network Domain Security

### 7.3	UTRAN Network Access Security

### 7.4	GERAN Network Access Security

### 7.5	GAA

#### 7.5.1	 Security enhancements for usage of GBA from the browser

S3-130921	CR to TR 33.823 Update of W3C references
					33.823	  CR-0004  (Rel-12) v12.1.0
					Source: Nokia Corporation, NSN

Abstract:

Work in W3C has progressed and therefore the references in this TR have to be updated.

Discussion:

No comments.

Decision: 		The document was agreed.

#### 7.5.2 	GBA extensions for re-use of SIP Digest credentials

#### 7.5.3	 Other GAA Issues

S3-130919	CR to TR 33.980 Correction of reference
					33.980	  CR-0023  (-) v..
					Source: Nokia Corporation, NSN

Abstract:

For ID-WSF the version number is missing.

Discussion:

Revised before presentation.

Decision: 		The document was revised to s3-131171.

S3-131171	CR to TR 33.980 Correction of reference
					33.980	  CR-0023  rev 1 (Rel-11) v11.0.0
					Source: Nokia Corporation, NSN

(Replaces S3-130919)

Decision: 		The document was agreed.

S3-130920	CR to TS 33.221 Correction of Reference
					33.221	  CR-0012  (-) v..
					Source: Nokia Corporation, NSN

Abstract:

For PKCS#10 reference RFC number is missing.

Decision: 		The document was revised to S3-131172.

S3-131172	CR to TS 33.221 Correction of Reference
					33.221	  CR-0012  rev 1 (Rel-11) v11.0.1
					Source: Nokia Corporation, NSN

(Replaces S3-130920)

Abstract:

For PKCS#10 reference RFC number is missing.

Decision: 		The document was agreed.

S3-131059	Clarification of NAF key negotiation on Ua
					33.222	  CR-0050  (-) v..
					Source: Ericsson, Nokia Corporation, Nokia Solutions and Networks

Abstract:

A CR to CT1 TS 24.109  (C1-133912 and its mirrors) at CT#84bis identified that the negotiation of NAF keys over Ua in case of GBA_U is underspecified for "Shared key-based UE authentication with certificate-based NAF authentication". The CT1 CRs were postponed as guidance from SA3 was needed, but the SA3 TS 33.222 was referring back to CT4 TS saying that " The exact definition of header fields and key selection logic in case of GBA_U (i.e., whether Ks_ext_NAF or Ks_int_NAF shall be used) in the NAF is part of TS 29.109 [19] and TS 24.109 [18]". However, this mechanism should be specified in SA3 TS.

In addition, the term "GBA mode" was not clearly specified, and SA3 TS is not unambiguous whether the UE can indicate support for GBA Digest at the same time with AKA-based "GBA modes".

Discussion:

Revised for minor editorials.

Decision: 		The document was revised to S3-131173.

S3-131173	Clarification of NAF key negotiation on Ua
					33.222	  CR-0050  rev 1 (Rel-11) v11.2.2
					Source: Ericsson, Nokia Corporation, Nokia Solutions and Networks

(Replaces S3-131059)

Abstract:

A CR to CT1 TS 24.109  (C1-133912 and its mirrors) at CT#84bis identified that the negotiation of NAF keys over Ua in case of GBA_U is underspecified for "Shared key-based UE authentication with certificate-based NAF authentication". The CT1 CRs were postponed as guidance from SA3 was needed, but the SA3 TS 33.222 was referring back to CT4 TS saying that " The exact definition of header fields and key selection logic in case of GBA_U (i.e., whether Ks_ext_NAF or Ks_int_NAF shall be used) in the NAF is part of TS 29.109 [19] and TS 24.109 [18]". However, this mechanism should be specified in SA3 TS.

In addition, the term "GBA mode" was not clearly specified, and SA3 TS is not unambiguous whether the UE can indicate support for GBA Digest at the same time with AKA-based "GBA modes".

Discussion:

No comments.

Decision: 		The document was agreed.

S3-131060	Clarification of NAF key negotiation on Ua
					33.222	  CR-0051  (-) v..
					Source: Ericsson, Nokia Corporation, Nokia Solutions and Networks

Discussion:

Mirror to S3-131059. Revised for minor editorials.

Decision: 		The document was revised to S3-131174.

S3-131174	Clarification of NAF key negotiation on Ua
					33.222	  CR-0051  rev 1 (Rel-12) v12.2.0
					Source: Ericsson, Nokia Corporation, Nokia Solutions and Networks

(Replaces S3-131060)

Discussion:

Mirror to S3-131059.

Decision: 		The document was agreed.

S3-131061	Checking that GBA types over Ua and Zn match
					33.220	  CR-0178  (Rel-12) v12.1.0
					Source: Ericsson

Abstract:

The Ua application protocol may have a negotiation of the used GBA keys, especially referring to used GBA bootstrapping type. The NAF should check the negotiation over Ua matches the NAF key received over Zn. Otherwise a malicious UE could be able to downgrade the security of Ua application protocol.

Decision: 		The document was agreed.

S3-131064	Deletion of Ks in ME in GBA_ME
					33.223	  CR-0023  (-) v..
					Source: Ericsson

Abstract:

GBAPush is using the Disposable-Ks model described in Annex A in TS 33.223:

‘GBA-Push utilizes the Disposable-Ks model in which a Ks is only used once to derive a single set of NAF-keys. This means that after a NAF-key derivation, the used Ks is erased or its further usage is blocked.’

In section 5.1.3 ‘BSF processing of NAF GPI request’, its explicitly stated that the BSF shall delete the Ks in step 8 after it has derived the NAF key and sent the response to the NAF,  but there is no corresponding requirement in GBA_ME on the ME in section 5.1.4 ‘UE processing of GPI’.

In GPI there is a NAF-key lifetime delivered to the UE but there is no Ks lifetime. This could imply that for GBA_ME, the ME stores the Ks until it powers off. If the network has pushed down several GPI’s to the UE while attached, then the ME has to store Ks’s keys (from GBAPush) which it is not allowed to re-use any longer if NAF keys have been derived from respectively Ks. Therefore in step 10 in section 5.1.4, it should be clarified for GBA_ME that the ME shall delete the Ks after it has derived and delivered the NAF keys to the application in the ME.

In addition, it has been clarified that when the UE checks if it has received the same GPI earlier, then this can be achieved by comparing the RAND value in the new GPI with the RAND value in an existing NAF SA.

Discussion:

Nokia proposed add comment that GBA and GBA PUSH is not the same thing.

Decision: 		The document was revised to S3-131175.

S3-131175	Deletion of Ks in ME in GBA_ME
					33.223	  CR-0023  rev 1 (Rel-12) v11.0.0
					Source: Ericsson

(Replaces S3-131064)

Discussion:

GBAPush is using the Disposable-Ks model described in Annex A in TS 33.223:

‘GBA-Push utilizes the Disposable-Ks model in which a Ks is only used once to derive a single set of NAF-keys. This means that after a NAF-key derivation, the used Ks is erased or its further usage is blocked.’

In section 5.1.3 ‘BSF processing of NAF GPI request’, its explicitly stated that the BSF shall delete the Ks in step 8 after it has derived the NAF key and sent the response to the NAF,  but there is no corresponding requirement in GBA_ME on the ME in section 5.1.4 ‘UE processing of GPI’.

In GPI there is a NAF-key lifetime delivered to the UE but there is no Ks lifetime. This could imply that for GBA_ME, the ME stores the Ks until it powers off. If the network has pushed down several GPI’s to the UE while attached, then the ME has to store Ks’s keys (from GBAPush) which it is not allowed to re-use any longer if NAF keys have been derived from respectively Ks. Therefore in step 10 in section 5.1.4, it should be clarified for GBA_ME that the ME shall delete the Ks after it has derived and delivered the NAF keys to the application in the ME.

In addition, it has been clarified that when the UE checks if it has received the same GPI earlier, then this can be achieved by comparing the RAND value in the new GPI with the RAND value in an existing NAF SA.

Decision: 		The document was agreed.

### 7.6	Multimedia Broadcast/Multicast Service (MBMS)

### 7.7	SAE/LTE Security

S3-130929	Work Item proposal: 3G Security: Specification of the TUAK algorithm set
					Source: BlackBerry, China Mobile, Ericsson, Deutsche Telekom, Morpho Cards, Orange, Vodafone

Abstract:

-

Decision: 		The document was withdrawn.

#### 7.7.1	TS 33.401 Issues

S3-131018	Correction of a typo
					33.401	  CR-0523  (Rel-12) v12.9.0
					Source: Nokia Corporation, NSN, NEC Corporation

Abstract:

There is a typo in 7.2.4.4.

Discussion:

no comments

Decision: 		The document was agreed.

#### 7.7.2	TS 33.402 Issues

S3-131176	User profile fetching before authentication in non3GPP access
					33.402	  CR-0115  rev 1 (Rel-12) v12.1.0
					Source: Ericsson, Nokia Corporation, Nokia Solutions and Networks

(Replaces S3-131062)

Discussion:

no comments

Decision: 		The document was agreed.

S3-131062	User profile fetching before authentication in non3GPP access
					33.402	  CR-0115  rev 2 (-) v..
					Source: Ericsson, Nokia Corporation, Nokia Solutions and Networks

Discussion:

Revised before presentation.

Decision: 		The document was revised to S3-131176.

S3-131063	User profile fetching before authentication in WLAN interworking
					33.234	  CR-0122  (-) v..
					Source: Ericsson, Nokia Corporation, Nokia Solutions and Networks

Abstract:

-

Discussion:

Related to 1063.

Decision: 		The document was revised to S3-131177.

S3-131177	User profile fetching before authentication in WLAN interworking
					33.234	  CR-0122  rev 1 (Rel-12) v11.4.0
					Source: Ericsson, Nokia Corporation, Nokia Solutions and Networks

(Replaces S3-131063)

Discussion:

No comments

Decision: 		The document was agreed.

#### 7.7.3	Other SAE/LTE Security Issues

### 7.8	Security Aspects of Home (e)NodeB

#### 7.8.1	TS 33.320 Issues

#### 7.8.2	TR 33.820 Issues

### 7.9	Security Aspects related to Machine-Type Communication

S3-130942	LS on interactions of oneM2M with Underlying Networks
					Source: OneM2M

Abstract:

-

Discussion:

1.	An M2M Service provider may request broadcasting/multicasting to multiple devices in the Operator Network (e.g. addressing a group of devices within a specified area for broadcast/multicasting of identical M2M data).

2.	An M2M Service provider may request QoS and Prioritization for M2M communications to/from individual devices or groups of devices. A device may request QoS and Prioritization for M2M communications to/from the M2M Service Provider.

3.	An M2M Service provider and a Network Operator may exchange information related to characteristics of M2M Devices or M2M Gateways, such as indications for small data, transmission scheduling parameters, mobility characteristics, etc..

4.	A Network Operator may request the M2M Service Provider to schedule traffic via the Operator Network (e.g. to delay specific M2M traffic when the 3GPP Network experiences high traffic load).

5.	It should be possible to dynamically (e.g. to enable plug and play) correlate the oneM2M Service Enablement Framework identifier of M2M Devices with the External Identifier used by the 3GPP network for the same MTC client.

6.	It should be possible to provide the oneM2M Service Enablement Framework with information regarding whether a M2M Device is authorized to access the Operator Network.

7.	An M2M Service provider and a Network Operator may exchange information on charging and subscriptions to support interworking with M2M Service providers.

8.	It should be possible to leverage on 3GPP security capabilities such as GBA for the benefit of oneM2M Services and Applications. Alternatively it could be possible to leverage oneM2M security capabilities for the benefit of Operator Network security.

9.	An M2M Service provider and a Network Operator may exchange information related to location information of M2M Devices or M2M Gateways.

Decision: 		The document was noted.

S3-131155	Reply to: LS on interactions of oneM2M with Underlying Networks
					Source: Ericsson

Discussion:

Offline session sorted out last details.

Decision: 		The document was approved.

S3-130946	Answer LS to oneM2M on interactions of oneM2M with Underlying Networks
					Source: S1-134175

Abstract:

-

Discussion:

SA1 would like to thank oneM2M for their LS on interactions of oneM2M with Underlying Networks.

SA1 would like to answer the questions from oneM2M as follows:

1.	Are there plans to specify capabilities for the 3GPP MTC features to interwork with a oneM2M Service Enablement Framework?

2.	oneM2M requests that SA1 considers the features from the list in section 1 as it plans requirements for Rel-13.

Answer to 1 and 2: SA1 has agreed a work item on “Service Exposure and Enablement Support” (attached to this LS) to create requirements for the scenarios indicated by oneM2M. Approval by SA plenary for this work item is pending but expected at SA#61 in September.

It should be noted that

•	The first scenario (on broadcast/multicast) is not referenced in the new work item because related requirements already exist in TS 22.368, however stage 2/3 work has been stopped.

•	Other scenarios are partially addressed in current requirements in TS 22.368 and expected to be completed in future.

3.	Has SA1 considered requirements for features beyond the features from the list in section 1?

Answer to 3: Since M2M services and other Application services often have the same or similar requirements on the 3GPP System these are addressed jointly in the new work item. oneM2M may therefore be interested on this part of the new work item.

General 3GPP requirements on M2M can be found in TS 22.368.

Decision: 		The document was noted.

S3-130930	Requirement of Network based filtering solution in SMS router
					Source: ZTE Corporation, China Unicom

Abstract:

In last meeting, S3-130863 which addressed the security requirement that operator can enforce filtering the trigger messages on the SMS router if SMS router has been deployed for TS23.682 has been approved. This contribution proposes to add same security requirement into the TS33.187.

Discussion:

Some contradiction was noted between proposed inserted text and existing NOTE 2. To be reviewed until next meeting.

Decision: 		The document was noted.

S3-130931	Correction to network based solution for filtering SMS-delivered device trigger messages
					Source: ZTE Corporation, China Unicom

Abstract:

This document is associated with network based solution for filtering SMS-delivered device trigger messages described in Sec.6.1.1 of TS33.187. Two solutions have been analysed, and some modifications are presented.

Discussion:

No support for the proposal.

Decision: 		The document was noted.

S3-130967	Device trigger over T5 interface
					Source: Huawei, HiSilicon

Abstract:

-

Discussion:

In TS33.187 the current device trigger solution is a SMS-based and uses a filtering infrastructure to block fake device trigger SMSs, but from the MTC-IWF point of view, the specification only covers the operation and security in T4 interface. However, as referred in SA2’s TS23.682, the device trigger can not only be delivered through T4 interface but  also through T5 interface, in order to make the device trigger solution complete and clear, it is necessary to add a description of device trigger over T5 interface in SA3’s TS33.187.

No support.

Decision: 		The document was noted.

S3-130998	MTC device triggering editors notes in solution 6
					Source: NEC Corporation

Abstract:

This document is to replace the editor’s note in TR 33.868, Section 5.1.4.2.7 “Solution 6: Secure Trigger Delivery with Security Association between MTC-IWF and UE” with clear text.

Discussion:

Approved for inclusion in the draft TR.

Decision: 		The document was approved.

S3-130968	Deletion of editor notes
					Source: Huawei, HiSilicon

Abstract:

-

Discussion:

Left for email approval

Decision: 		The document was revised to S3-131209.

S3-131209	Deletion of editor notes
					Source: Huawei, HiSilicon

(Replaces S3-130968)

Decision: 		The document was approved.

S3-130969	Detail the description of partly ciphering SDT solution
					Source: Huawei, HiSilicon

Abstract:

-

Discussion:

left for email approval.

Decision: 		The document was revised to S3-131206.

S3-131206	Detail the description of partly ciphering SDT solution
					Source: Huawei, HiSilicon

(Replaces S3-130969)

Decision: 		The document was approved.

S3-130997	MTC-IWF based solution editors notes
					Source: NEC Corporation

Abstract:

-

Discussion:

Left for email approval.

Decision: 		The document was revised to S3-131210.

S3-131210	MTC-IWF based solution editors notes
					Source: NEC Corporation

(Replaces S3-130997)

Decision: 		The document was approved.

S3-130970	Considerations on the MTC-IWF based SDT security solution
					Source: Huawei, HiSilicon

Abstract:

-

Discussion:

Left for email approval.

Decision: 		The document was revised to S3-131207.

S3-131207	Considerations on the MTC-IWF based SDT security solution
					Source: Huawei, HiSilicon

(Replaces S3-130970)

Decision: 		The document was approved.

S3-130985	Evaluation on Security of Small Data Transmission
					Source: China Mobile

Abstract:

-

Discussion:

Left for email approval.

Decision: 		The document was revised to S3-131208.

S3-131208	Evaluation on Security of Small Data Transmission
					Source: China Mobile

(Replaces S3-130985)

Decision: 		The document was approved.

S3-130962	Security for UE Power Consumption Optimisation (UEPCOP)
					Source: Huawei, HiSilicon

Abstract:

-

Discussion:

SA2#99 has approved UEPCOP CRs in S3-133647, 648, 649 of TS23.682, 23.401 and 23.060 with the description of UE Power Saving Mode (PSM). There is a conclusion for the feature of UE Power Consumption Optimisation in SA2. In this method, a UE may adopt a PSM. When the UE wants to use the PSM it shall request an Active Time and may request a Periodic TAU/RAU Timer value during every Attach and TAU procedures. When the Active timer expires the MME knows that the UE entered PSM and is not available for paging. The MME handles mobile reachable/Active timer and availability for paging. In PSM the UE is in ECM_IDLE. On UE side the PSM complies with some substates of EMM_REGISTERED. The MME considers the UE to be EMM_REGISTERED, but not reachable. The UE’s Access Stratum functions are considered as deactivated during PSM. The same is for UMTS system.

As we analyzed in TR33.868v0.15.0 clause 5.6.4.1, there is no security impact of solution 3 which is as the same principle as PSM. The periodic timer value is protected by NAS security. No new security procedure is needed.

It was decided to remove the section instead.

Decision: 		The document was noted.

S3-131054	Secure Connection details on GBA based solutions
					Source: Ericsson

Abstract:

This contribution proposes to add more details to the GBA and GBAPush solutions for establishing key material for the Secure Connection between UE and SCS/MTC Application Server in TR 33.868 Rel-12.

Discussion:

Approved for inclusion in the draft TR.

Decision: 		The document was approved.

S3-131056	GBA for constrained devices
					Source: Ericsson

Abstract:

Contribution S3-130658 raised concerns that using GBA requires the support of HTTP and TCP which may be problematic for constrained M2M devices. This contribution provides an analysis of protocol dependency aspects of GBA and constrained devices.

Discussion:

Revision needed. Some parts of text not required.

Decision: 		The document was revised to S3-131158.

S3-131158	GBA for constrained devices
					Source: Ericsson

(Replaces S3-131056)

Abstract:

TS 22.368 state the following requirement on secure connection:

The network operator shall be able to efficiently provide network security for connection between MTC Device and a MTC Server or between MTC Device and a MTC Application Server in case there is a direct connection with the MTC Application Server. This applies even when some of the devices are roaming i.e. connected via a VPLMN.

This contribution provides more evaluation on EAP-AKA and GBA-based Secure connection proposals.

Discussion:

No further comments.

Decision: 		The document was approved.

S3-131065	Evaluation of secure connection proposals
					Source: Ericsson

Abstract:

TS 22.368 state the following requirement on secure connection:

The network operator shall be able to efficiently provide network security for connection between MTC Device and a MTC Server or between MTC Device and a MTC Application Server in case there is a direct connection with the MTC Application Server. This applies even when some of the devices are roaming i.e. connected via a VPLMN.

This contribution provides more evaluation on EAP-AKA and GBA-based Secure connection proposals.

Discussion:

Update required. Table to be removed. Further offline discussion required.

Decision: 		The document was revised to S3-131159.

S3-131159	Evaluation of secure connection proposals
					Source: Ericsson,China Mobile

(Replaces S3-131065)

Abstract:

TS 22.368 state the following requirement on secure connection:

The network operator shall be able to efficiently provide network security for connection between MTC Device and a MTC Server or between MTC Device and a MTC Application Server in case there is a direct connection with the MTC Application Server. This applies even when some of the devices are roaming i.e. connected via a VPLMN.

This contribution provides more evaluation on EAP-AKA and GBA-based Secure connection proposals.

Discussion:

Approved with modification for inclusion in draft TR.

Decision: 		The document was approved.

S3-131020	pCR on the evaluation of MTC Secure Connection solutions
					Source: Nokia Corporation, NSN

Abstract:

-

Discussion:

Not presented, no longer relevant.

Decision: 		The document was noted.

S3-131055	Solution for Secure Connection in draft TS 33.187
					Source: Ericsson

Abstract:

This contribution proposes that the GBA and GBAPush solutions are agreed to be incorporated into the draft TS 33.187 for the MTC Feature Secure Connection.

Discussion:

Offline work required to get phrasing right.

Decision: 		The document was revised to S3-131160.

S3-131160	Solution for Secure Connection in draft TS 33.187
					Source: Ericsson

(Replaces S3-131055)

Abstract:

This contribution proposes that the GBA and GBAPush solutions are agreed to be incorporated into the draft TS 33.187 for the MTC Feature Secure Connection.

Discussion:

No further comments

Decision: 		The document was approved.

S3-131069	Solution of restricting the USIM to specific UEs
					Source: Nokia, NSN, Ericsson, ST-Ericsson, Samsung

Abstract:

-

Discussion:

Revised before presentation

Decision: 		The document was revised to S3-131146.

S3-131146	Solution of restricting the USIM to specific UEs
					Source: Nokia, NSN, Ericsson, ST-Ericsson, Samsung

(Replaces S3-131069)

Abstract:

As analyzed in TR33.868 the solution of IMSI-IMEI binding in HSS can fulfil the SA1’s requirements, doesn’t require new functionality on the UE, doesn’t require new UICCs, and requires minimal new functionality in the network. So it’s the feasible solution of restricting the USIM to specific UEs.

The current contribution proposes the needed functionality in draft TS 33.187.

Accompanying contributions S3-131058 and S3-131057 show the needed changes as SA2 CRs for TS 23.060 and 23.401.  Contributions S3-131058 and S3-131057 are submitted for information to SA3. Respective CRs have also been submitted to SA2 meeting #100 (11-15 November).

If SA3 agrees on the solution in the current contribution, it is proposed to send an LS to SA2 so that SA2 could agree on the respective CRs to TS 23.060 and 23.401 at SA2 #100.

The supporting companies believe that the current proposed solution should be agreed in SA3 as a first step.

Discussion:

Available as draft.

Decision: 		The document was noted.

S3-131057	pCR to 23.401: Solution of restricting the USIM to specific UEs	
					Source: Ericsson

Abstract:

This contribution shows the impacts the solution described in S3-131069 for IMSI-IMEI binding in HSS would have on SA2 TS 23.401 v. 12.2.0.

Discussion:

Await outcome on 1146.

Decision: 		The document was noted.

S3-131058	pCR to 23.060: Solution of restricting the USIM to specific UEs
					Source: Ericsson

Abstract:

This contribution shows the impacts the solution described in S3-131069 for IMSI-IMEI binding in HSS would have on SA2 TS 23.060 v12.2.0.

Discussion:

Await outcome on 1146.

Decision: 		The document was noted.

S3-131090	Updating the proposed KME management procedures
					Source: Qualcomm Incorporated, Alcatel-Lucent

Abstract:

This contribution proposes a couple of enhancements to the KME provisioning described in clause 5.4.4.2.3.3.1.

Discussion:

Approved for inclusion into the draft TR.

Decision: 		The document was approved.

S3-131091	Propose pCR to add symmetric key based solution to the MTC TS
					Source: Qualcomm Incorporated, Alcatel-Lucent

Abstract:

This contribution is a companion contribution to S3-131089 and contain a pCR in to implement the conclusion proposed in that document.

Discussion:

Proposal for inclusion in the MTC TS.

Decision: 		The document was noted.

S3-130986	Working way and evaluation on restricting the USIM to specific MEs/MTC Devices
					Source: China Mobile, TeliaSonera

Abstract:

Abstract of the contribution: this contribution discusses the working way and evaluation on restricting the USIM to specific MEs/MTC Devices and gives a way forward.

Discussion:

In order to help the selection of the binding method, we think first of all we need to decide the working way and then compare the evaluation result and the last select the appropriate solution. This contribution discusses the working way based on s3-120408 in 67 meeting and integrates the previous evaluation results. We kindly propose we consider this contribution and give the appropriate working way and select the appropriate solution.

Decision: 		The document was noted.

S3-131107	Comments on S3-130986
					Source: Orange

Abstract:

-

Decision: 		The document was noted.

S3-131085	Conclusion for USIM restriction to specific MTC Devices
					3GPP TR 33.868 v..
					Source: Morpho Cards, Gemalto, Vodafone

Abstract:

Conclusion for USIM restriction to specific MTC Devices

Decision: 		The document was revised to S3-131115.

S3-131115	Conclusion for USIM restriction to specific MTC Devices
					3GPP TR 33.868 v..
					Source: Morpho Cards, Gemalto, Vodafone

(Replaces S3-131085)

Abstract:

This contribution proposes the Conclusion on Solution choice for USIM restriction to specific MTC Devices

Discussion:

Following the decision of SA3#71 and the conclusion of the discussion at SA3#72, this contribution aims to finalize the selection of a solution for UICC pairing with specific MTC Devices.

The discussion at SA3#72 has arrived to the point that each solution described in the TR provides a specific solution addressing the sub-set of requirements and depends on the deployment context. Due to the fact that selecting one solution is not possible, the best way forward is to select one UE-based solution and one network-based solution to address as much as possible MTC scenarios.

Supported by BT as well

Decision: 		The document was withdrawn.

S3-131089	Proposed conclusion for restricting USIM to specific UEs
					Source: Qualcomm Incorporated, Alcatel-Lucent

Abstract:

-

Discussion:

There are several proposed solutions to satisfy the requirements for restricting the use of a USIM to specific devices. When reflecting on the discussion for this problem over the last meetings, it seems that the main issues that have been preventing the discussion reaching a conclusion are the following:

•	The level of security that needs to be achieved by this feature

•	The ability to manage this feature when it is deployed

•	Getting a suitable amount of benefit from the implementation effort

•	The desire to select only one solution

Discussion on the group of related documents showed that there is no consensus on the preferred way forward. There are four solutions on the table (plus their combinations), and most companies indicated they were looking towards bundled solutions, though not necessarily the same bundle. An informal show of hands was proposed, but even the way to do that was fiercely discussed.

10 companies prefer two solutions.

7 companies prefer one solution.

This result did not resolve the issue.

Decision: 		The document was noted.

S3-131105	conditional pCR TS 33.187: USAT-based pairing for USIM restriction
					TS 33.187 v..
					Source: Gemalto

Abstract:

pseudo-CR to TS 33.187 on USAT-based pairing description under condition that SA3 would conclude to specify this solution.

Discussion:

Not agreed, further work until later meeting.

Decision: 		The document was noted.

S3-131108	Comments on S3-130986
					Source: Orange

Abstract:

-

Decision: 		The document was withdrawn.

S3-131112	Comments on S3-130985
					Source: NEC Corporation

Abstract:

-

Discussion:

Email approval.

Decision: 		The document was left for email approval and noted

S3-131144	S2MTC
					Source: Unknown

Decision: 		The document was withdrawn.

S3-131156	Draft TR 33.868
					Source: Rapporteur

Discussion:

Email approval.

Decision: 		The document was left for email approval and approved.

S3-131157	Draft TS 33.187
					Source: Rapporteur

Discussion:

Email approval.

Decision: 		The document was left for email approval and approved.

S3-131199	Exception sheet for MTC
					Source: Rapporteur

Discussion:

Three solutions are on the table. The companies supported different packs as follows:

Simple check in the HSS plus pin pairing

China mobile, TeliaSonera, NSN, Nokia, China Unicom

Simple check plus USAT pairing

BT, Samsung, Huawei, Orange, Telecom Italia

Full check on Pairing based on symmetric shared secret on network side plus USAT pairing

Ericsson, Deutche Telecom, Qualcomm, Vodafone, Morpho, Gemalto, Alcatel-Lucent

SA3 is nowhere near consensus on this issue. Decision is urgently needed if this shall be released in Rel-12 (and exception sheet will be required). The Chairman noted there opinions appear to be too spread for a technical voting to make sense, as 71% will obviously not be met by any proposal. Issue was postponed until next meeting with companies being urged to do their homework and lobbying until then, to allow the open question reach final decision at next meeting. The possibility of a vote was NOT excluded, but the Chairman recommended to reach agreement without vote at next meeting.

Email approval.

Decision: 		The document was left for email approval and approved.

### 7.10 	Security Aspects of Public Warning System

Way forward: to clean up the 800 series TR and upgrade it to become  a 900 series TR. First draft 900 should be available at next meeting. SA3 agreed that no conclusion will be contained in the TR 900 that aims to either already recommend or rule out a particular solution

S3-130963	Solving EN for terminology used to identify keys
					Source: Huawei, HiSilicon

Abstract:

SA3 made agreement that PKID is an identifier to distinguish public keys for PWS. So the editor’s note in section 6.2.8 for terminology used to identify keys should be deleted.

Discussion:

Approved for inclusion in draft TR PWS.

Decision: 		The document was approved.

S3-130964	Corrections on NAS based solution
					Source: Huawei, HiSilicon

Abstract:

-

Discussion:

Approved for inclusion in draft TR PWS.

Decision: 		The document was approved.

S3-131022	pCR: Clarifications on Implicit Certificate approach
					Source: BlackBerry UK Ltd.

Abstract:

The current PWS study details on the implicit certificate approach as a means of providing security for PWS. This pCR proposes several editorial corrections to the current text on the implicit certificate approach in addition to adding details on using the PWS CA update message to revoke an existing CA.

Discussion:

Approved for inclusion in draft TR PWS with two modified editors notes.

Decision: 		The document was approved.

S3-130932	Restrictions on signature length in PWS security
					33.869 v..
					Source: Ericsson, Nokia Corporation, NSN

Abstract:

This pCR proposes to add a section in the TR which captures the information received in the LS replies. The new text makes Clause 6.2.6 redundant and it is therefore removed/voided.

Decision: 		The document was noted.

S3-131189	Restrictions on signature length in PWS security
					33.869 v..
					Source: Ericsson, Nokia Corporation, NSN

(Replaces S3-130932)

Decision: 		The document was noted.

S3-131072	PWS security: notes on PWS signing proxies
					Source: Nokia Corporation, NSN

Abstract:

A PWS signing proxy is an entity in the domain of a PWS regulator that signs PWS warning messages on behalf of CBEs. Signing proxies were first described, without much detail, in the context of the NAS-based solution.  The present discussion paper motivates two companion pCRs in S3-131073 and S3-131074. These pCRs describe the use of signing proxies independent of the root key distribution scheme and compare it to the use of implicit certificates, respectively.

Decision: 		The document was noted.

S3-131073	PWS security: The use of signing proxies
					Source: Nokia Corporation, NSN, TeliaSonera

Abstract:

This contribution elaborates on the use of signing proxies. This is not a new solution, rather the use of signing proxies was first described when evaluating the NAS-based solution, cf. clause 8.3.4. In other parts of the TR, the possibility of their use is only briefly mentioned in clauses 6.1, 6.2, and Annex B. It is, however, important to understand that the use of signing proxies is by no means limited to a particular root key distribution solution. Signing proxies are therefore described in more detail, and in a way independent of the root key distribution solution, below. The companion pCR in S3-131074 compares their use to the implicit certificate approach in solution 6.

Decision: 		The document was revised to S3-131190.

S3-131190	PWS security: The use of signing proxies
					Source: Nokia Corporation, NSN, TeliaSonera

(Replaces S3-131073)

Abstract:

This contribution elaborates on the use of signing proxies. This is not a new solution, rather the use of signing proxies was first described when evaluating the NAS-based solution, cf. clause 8.3.4. In other parts of the TR, the possibility of their use is only briefly mentioned in clauses 6.1, 6.2, and Annex B. It is, however, important to understand that the use of signing proxies is by no means limited to a particular root key distribution solution. Signing proxies are therefore described in more detail, and in a way independent of the root key distribution solution, below. The companion pCR in S3-131074 compares their use to the implicit certificate approach in solution 6.

Discussion:

Almost consensus, however one editors note might be subject to further discussion at future meetings.

Decision: 		The document was approved.

S3-131074	PWS security: Comparison of the use of signing proxies and implicit certificates
					Source: Nokia Corporation, NSN, TeliaSonera

Abstract:

This contribution complements the companion pCR in S3-131073 that gave an overview of the use of signing proxies. Here, we compare the use of signing proxies with that of implicit certificates as both approaches are suitable to satisfy the most stringent length requirements.

Decision: 		The document was revised to S3-131191.

S3-131191	PWS security: Comparison of the use of signing proxies and implicit certificates
					Source: Nokia Corporation, NSN, TeliaSonera

(Replaces S3-131074)

Decision: 		The document was withdrawn.

S3-131075	PWS security: review of solution 7 (generalised certificate-based approach)
					Source: Nokia Corporation, NSN

Abstract:

This contribution resolves Editor’s notes and makes an update to the introduction. The first Editor’s note is resolved through additional text in clause 7.7.4. Three more Editor’s notes are resolved by text close to the location of the Editor’s notes. The update to the introduction takes into account the working assumption that 2G subscriber need not be supported.

Decision: 		The document was approved.

S3-131076	PWS security: proposed skeleton for clause 9 'Conclusions'
					Source: Nokia Corporation, NSN, Ericsson

Abstract:

PWS security has been studied in SA3 in TR 33.869. It is about time to draw conclusions from the study. This contribution proposes a skeleton for the clause ‘Conclusions’ in the TR. Editor’s notes show the expected content for each sub clause. In a few instances, regular text has been provided instead of Editor’s notes when it was felt that agreement had already been reached on the issue. By providing this skeleton, the contribution also shows what decisions need to be taken on which issues.

Discussion:

Not yet agreed.

Decision: 		The document was noted.

S3-131077	PWS security: conclusions on Public Key Infrastructure
					Source: Nokia Corporation, NSN

Abstract:

A skeleton for clause 9 on Conclusions was provided in S3-131076. The present contribution proposes text for clause 9.3.3 from S3-131076 on digital signature schemes.

Discussion:

Not yet agreed.

Decision: 		The document was noted.

S3-131078	PWS security: conclusions on Public Key Infrastructure
					Source: Nokia Corporation, NSN

Abstract:

A skeleton for clause 9 on Conclusions was provided in S3-131076. The present contribution proposes text for clause 9.3.4 from S3-131076 on public key infrastructure issues to enable digital signature verification by UEs. We believe it makes sense to leave the decision between the implicit certificate approach and the signing proxy approach currently open in 3GPP because

•	the differences between the two approaches largely affect the regulator’s domain and not the operator’s domain;

•	there are no requirements for the regulator’s domain today;

•	different regulators may want to make different choices in the future.

Discussion:

Not yet agreed.

Decision: 		The document was noted.

S3-131079	PWS security: High-level conclusion on PWS root key distribution
					Source: Nokia Corporation, NSN

Abstract:

A skeleton for clause 9 on Conclusions was provided in S3-131076. The present contribution proposes text for clause 9.4 from S3-131076 on mechanism for distributing PWS root keys to UEs. The text does not make a final selection among the mechanisms that were studied in the present TR. It only proposes to restrict the selection to those mechanisms that do not place root key distribution functionality in the core network.

Discussion:

Not yet agreed.

Competitive conclusion to the one in 0959.

Decision: 		The document was noted.

S3-130959	pCR: Conclusion section of TR 33.869
					Source: BlackBerry, BT, Huawei, Vodafone

Abstract:

TR33.869 discusses several potential approaches for adding security to PWS including NAS-based solution, Implicit Certificate, GBA, Generalised Certificate and UICC OTA based approaches.

Given the advantages already highlighted in this contribution, and the limited time remaining in Rel’12, it is proposed SA3 conclude to proceed with the implicit certificate approach and agree to include the following in the conclusion section of  TR 33.869.

Discussion:

Not yet agreed.

Competitive conclusion to the one in 1079.

The two conclusions differ significantly. It was emphasized that in the PWS case, the regulator alone decide when it is to be used. Level of protection unclear, ranging from protecting of individuals against advertisements to protection against nation scale cyber attacks.

A large minority of companies indicated they preferred a normative solution. Thus no consensus so far.

Decision: 		The document was noted.

S3-131025	Solution for PWS TS
					Source: Blackberry,Huawei, HiSilicon,Vodafone

Abstract:

-

Decision: 		The document was noted.

S3-131188	Draft TR  on PWS
					Source: Rapporteur

Decision: 		The document was left for email approval and approved

### 7.11	Firewall Traversal

#### 7.11.1	Study on Firewall Traversal

S3-131019	Proposed the solution of media over TCP based on ICE  for TR 33.830
					Source: participant

Abstract:

-

Decision: 		The document was withdrawn.

S3-131024	Proposed the solution of media over TCP based on ICE for TR 33.830
					Source: Huawei, Hisilicon

Abstract:

-

Decision: 		The document was withdrawn.

S3-131096	PCR_TR33.830Firewall traversal for IMS media plane using
					TR33.830 v..
					Source: Huawei

Abstract:

-

Discussion:

Revised before presentation.

Decision: 		The document was revised to S3-131154.

S3-131154	PCR_TR33.830Firewall traversal for IMS media plane using
					TR33.830 v..
					Source: Huawei

(Replaces S3-131096)

Abstract:

TR33.803 has concluded that the existing ICE/TURN/STUN solution for NAT traversal can be reused for IMS firewall traversal.   In SA3 #72 meeting, Huawei proposed an IMS media traversal solution based on ICE/STUN and media over TCP for TS 33.203(S3-130725).   The SA3 suggested that this solution should be submitted to TR 33.830 first.  This pCR follows the discussion in SA3 #72 meeting and proposes to add the ICE/STUN solution to TR 33.830 for more study.

Discussion:

Approved for inclusion into the draft TR.

Decision: 		The document was approved.

S3-131181	Draft TR33.830 Firewall traversal
					Source: Rapporteur

Decision: 		The document was approved.

#### 7.11.2	Tunnelling of UE Services over Restrictive Access Networks

S3-130927	Incorrect reference for keep-alive mechanism
					33.203	  CR-0204  (-) v..
					Source: Ericsson

Abstract:

-

Decision: 		The document was revised to S3-131098.

S3-131098	Incorrect reference for keep-alive mechanism
					33.203	  CR-0204  rev 1 (Rel-12) v12.3.0
					Source: Ericsson

(Replaces S3-130927)

Abstract:

The reference to RFC 5626 (Outbound) in the ICE based firewall traversal solution is incorrect. The correct reference is RFC 6223.

Decision: 		The document was agreed.

### 7.12 	Security Aspects of WLAN Network Selection for 3GPP Terminals

S3-130975	Correction and Deletion of the Editor notes
					Source: Huawei, HiSilicon

Abstract:

There are some inaccurate details and editor’ notes in TR33.865. So it is necessary to clarify the problems and deletes the corresponding content.

Decision: 		The document was approved.

S3-130976	Details of solution 2 supporting security parameters of selection policies
					Source: Huawei, HiSilicon

Abstract:

This contribution provides more details for the solution 2 of clause 5.2 in TR33.865 and solves the following editor’s note.

Discussion:

Add reference pointing to SA2 spec.

Approved with modifications

Decision: 		The document was approved.

S3-131080	WLAN  Proposal for clause 6 Framework for Analysis of Solutions.doc
					3GPP TR 33.865 Security aspects of WLAN network selection for 3GPP terminals v..
					Source: BT Group

Abstract:

It had previously been agreed that due to the clear text transmission between WLAN selection information and UE, this information can be used by the attackers/malicious WLAN networks to attract user join in their WLAN network.

Discussion:

The importance of this was clarified, and also the key issue, which is how to communicate availability of secure, authenticated access points to UEs in an efficient manner (think very crowded environment). Doing things over TCP is too slow.

Decision: 		The document was noted.

S3-131179	Draft TR 33.865; Security aspects of WLAN network selection for 3GPP terminals
					3GPP TR 33.865 Security aspects of WLAN network selection for 3GPP terminals v..
					Source: Rapporteur

Discussion:

Approved as basis for further work.

Decision: 		The document was approved.

S3-131180	Exception sheet for WLAN Network Selection for 3GPP Terminals
					Source: Rapporteur

Discussion:

Email approval

Decision: 		The document was left for email approval and approved.

### 7.13	Security Aspects of Proximity-based Services

S3-130905	Proposed changes to the Definitions section for ProSe
					Source: Telecom Italia, Orange

Abstract:

In order to avoid possible misunderstandings due to different terminology adopted across SA WGs, this contribution proposes to align and complete the definitions in the SA3 ProSe TR to those already agreed within SA1.

Discussion:

Approved with modifications

Decision: 		The document was approved.

S3-131081	Security architecture and requirements for proximity based services
					Source: samsung

Abstract:

-

Discussion:

Added sentence ''interface..'' after Figure X to be removed. For PC7 and PC8, we should have exact copy from SA2.

Approved with modifications.

Decision: 		The document was approved.

S3-131095	Alignment and editorial changes to the ProSe TR
					Source: Qualcomm Incorporated

Abstract:

-

Discussion:

Discussion if definitions from SA1/2 and TR21.905 should be copied or referred. Decision to copy, and add note explaining need to constantly check if source definitions change.

approved for inclusion in TR.

With these changes, approved for inclusion in TR.

Decision: 		The document was approved.

S3-130907	Proposed changes to Clause 4
					Source: Telecom Italia

Abstract:

This pseudo-CR proposes some changes to Clause 4 Proximity Services of SA3 draft-TR on ProSe.

Discussion:

It was checked that the proposal was in line with SA2.

Decision: 		The document was approved.

S3-130906	Adding General Requirements on ProSe Security, Authorization and Privacy
					Source: Telecom Italia

Abstract:

This pseudo-CR proposes to add a new clause to the SA3 draft-TR on ProSe on General Requirements on ProSe Security, Authorization and Privacy.

Discussion:

Proposal to refer to 22.278 instead of copying. Decision to replace with reference. It was noted that SA1 should not make security requirements, and better would be to remove such requirements from SA1 documents and only have them in SA3. To be investigated if the SA1 security requirements be moved to SA3 documents.

Decision: 		The document was revised to S3-131194.

S3-131194	Adding General Requirements on ProSe Security, Authorization and Privacy
					Source: Telecom Italia

(Replaces S3-130906)

Abstract:

This pseudo-CR against the SA3 draft-TR on ProSe implements the compromise agreed offline after the plenary discussion of TD S3-130906

Decision: 		The document was approved.

S3-131012	ProSe security using a combination of LTE security and an overlay
					Source: Ericsson

Abstract:

There are two layers in the ProSe architecture, application layer and connectivity layer. Regular users should be able to expect the same level of security from the connectivity layer as in normal LTE. NSPS organizations may, on the other hand, have stronger requirements. To ensure that the connectivity layer does not become unnecessarily complex, this pCR proposes that the stronger security requirements are resolved by the ProSe application layer. This results in a more modular design which will reduce needed standardization work, and may reduce the complexity of the design.

Discussion:

UE-UE is not rel-12

Decision: 		The document was revised to S3-131134.

S3-131134	ProSe security using a combination of LTE security and an overlay
					Source: Ericsson

(Replaces S3-131012)

Abstract:

There are two layers in the ProSe architecture, application layer and connectivity layer. Regular users should be able to expect the same level of security from the connectivity layer as in normal LTE. NSPS organizations may, on the other hand, have stronger requirements. To ensure that the connectivity layer does not become unnecessarily complex, this pCR proposes that the stronger security requirements are resolved by the ProSe application layer. This results in a more modular design which will reduce needed standardization work, and may reduce the complexity of the design.

Discussion:

Approved for inclusion in draft TR.

Decision: 		The document was approved.

S3-130955	LS on Group Communication Security in LTE
					Source: ETSI TC TCCE

Abstract:

-

Discussion:

The TETRA LS summarises to the following points:

- 	The scope of the new TR on GCSE security should cover modifications of the existing key management for IMS and MBMS media security and the addition of new key management mechanisms independent of IMS and MBMS.

- 	The existing key management options for IMS media security may not be appropriate for large groups when used with bilateral or group keys.

- 	The current key management in MBMS may not offer a sufficient level of security for public safety users.

- 	The best solution may be to introduce a new key management option that is suitable for use in both IMS and MBMS media plane security.

Decision: 		The document was replied to in S3-131128.

S3-131128	Reply to: LS on Group Communication Security in LTE
					Source: EADS

Discussion:

Reply to S3-130995.

Decision: 		The document was approved.

S3-130933	ProSe: Key issues on Group Communications
					Source: Home Office (UK)

Abstract:

Aspects of ProSe relevant to security for group communications, especially for public safety usage, are highlighted as key issues for SA3 to consider.

Decision: 		The document was revised to S3-131137.

S3-131137	ProSe: Key issues on Group Communications
					Source: Home Office (UK)

(Replaces S3-130933)

Decision: 		The document was approved.

S3-130972	key issue for ProSe communications in Group Owner mode
					Source: Huawei, HiSilicon

Abstract:

-

Decision: 		The document was revised to S3-131138.

S3-131138	key issue for ProSe communications in Group Owner mode
					Source: Huawei, HiSilicon

(Replaces S3-130972)

Decision: 		The document was approved.

S3-131087	ProSe: new key Issue on key establishment
					3GPP TR 33.cde on Security Aspects of ProSe v..
					Source: Gemalto, Morpho Cards

Abstract:

Addition of key issue related to key establishment

Discussion:

Several scenarios of SA2 TR 23.703, e.g. solutions C1 and C2, require the presence of a security mechanism to generate and distribute a key that could be shared by different members of a ProSe Group. This shared key could be used for one-to-many communication when the ProSe-enabled UEs are in or out of network coverage. This key establishment has to be secure and represents a Key Issue to be described in 3GPP TR on security issues to support ProSe.

Decision: 		The document was revised to S3-131142.

S3-131142	ProSe: new key Issue on key establishment
					3GPP TR 33.cde on Security Aspects of ProSe v..
					Source: Gemalto, Morpho Cards

(Replaces S3-131087)

Decision: 		The document was approved.

S3-131094	Proposed key issue on one-to-many communication for ProSe
					Source: Qualcomm Incorporated

Abstract:

-

Discussion:

Debate on details attempting to debug for ambiguities.

Offline studies. To be merged into 1137.

Decision: 		The document was revised to S3-131137.

S3-130973	Security for PrSe UE communication in group owner mode
					Source: Huawei, HiSilicon

Abstract:

-

Decision: 		The document was revised to S3-131143.

S3-131143	Security for PrSe UE communication in group owner mode
					Source: Huawei, HiSilicon

(Replaces S3-130973)

Decision: 		The document was approved.

S3-131010	IDENTITY Solution for Public Safety ProSe
					Source: CESG (UK)

Abstract:

A presentation introducing the IDENTITY solution.

Decision: 		The document was noted.

S3-131034	IDENTITY Solution for ProSe group communications
					Source: CESG (UK)

Abstract:

This contribution contains a proposed solution for security for ProSe group communications.

Discussion:

It was found that some description or explanation is required. Details on options on key management are needed. Editors note to be inserted.

Decision: 		The document was approved.

S3-131088	ProSe: solution for key issue on key establishment
					3GPP TR 33.cde on Security Aspects of ProSe v..
					Source: Gemalto, Morpho Cards

Abstract:

ProSe: Solution for key issue on key establishment

Discussion:

Add editors note explaining incompleteness of procedures.

Approved with modifications

Decision: 		The document was approved.

S3-130961	pCR to TR 33.cde Sec.6.3.4.3.2
					TR 33.cde [ProSe] v..
					Source: Alcatel-Lucent

Abstract:

This pCR proposes descriptive changes to sec 6.3.4.3.2 of TR 33.cde to accommodate key establishment schemes pending their approval.

Discussion:

This pCR proposes descriptive changes to sec 6.3.4.3.2 of TR 33.cde.

In sec.6.3.4.3.2 the UE-to-UE key exchange protocol for the KD is described for the Network-assisted direct communications use case.

The proposed change makes the text in the section sufficiently general pending acceptance of the specific key establishment scheme.

Approved for inclusion

Decision: 		The document was approved.

S3-131008	Using IMS e2e solutions for direct one-to-one communications
					Source: CESG (UK)

Abstract:

This contribution contains a solution for 1-1 communication security.

Discussion:

This contribution contains a solution for direct one-to-one communication security in a commercial context.

The intention of the proposal is to demonstrate that security solutions that have already been standardised to provide end-to-end security for the IMS media plane can be simply adapted to meet the commercial requirement for direct one-to-one communications.

Though it is not a requirement, it is noted that having a similar approach to e2e security across IMS and ProSe may be beneficial. By minimising the number of security solutions, the number of network elements and protocols involved in securing UE connections will be reduced. The advantages will be particularly evident should SA2 decide to use IMS as part of the ProSe architecture.

Approved for inclusion in the TR.

Decision: 		The document was approved.

S3-131009	A security context for direct one-to-one communications
					Source: CESG (UK)

Abstract:

This contribution contains a mechanism for deriving traffic keys to create a security context for 1-1 communication security.

Discussion:

Relate to S3-131008.

Decision: 		The document was noted.

S3-131011	IDENTITY Solution for ProSe one-to-one communications
					Source: CESG (UK)

Abstract:

This contribution contains a proposed solution for security for ProSe 1-to-1 communications.

Discussion:

Relate to S3-131008.

It was decided to add necessary definitions.

Approved for inclusion in the draft TR.

Decision: 		The document was approved.

S3-130974	Security for ProSe communication through UE-to-Network relay with network authorization
					Source: Huawei, HiSilicon

Abstract:

-

Discussion:

Security for ProSe communication through UE-to-Network relay with network authorization	Huawei, HiSilicon	-	-	-	-	-	"SA2 has defined the ProSe relay solutions for public safety use case. This solution is the security part of solution 12 of ProSe Relay in TR23.703.

More editors notes added.

Decision: 		The document was revised to S3-131147.

S3-131147	Security for ProSe communication through UE-to-Network relay with network authorization
					Source: Huawei, HiSilicon

(Replaces S3-130974)

Abstract:

This solution is the security part of solution 12 of ProSe Relay in TR23.703.

Decision: 		The document was approved.

S3-130908	Proposed changes on Key Issue #1
					Source: Telecom Italia

Abstract:

This pseudo-CR proposes some changes to the Key Issue #1: Configuration of ProSe-enabled UEs of SA3 draft-TR on ProSe

Discussion:

Configuration data may include e.g. proximity criteria as well as PLMN sensitive radio resource configuration. For this reason the server sending the configuration needs to be part of the HPLMN/VPLMN offering the ProSe service. Configuration data impacting the network operations shall be provided only by Operators (either HPLMN or VPLMN). 3rd party are not allowed to provide such parameters.

Moreover, it is crucial that configuration data are not prone to modification/eavesdropping also when stored on the UE.

Decision: 		The document was revised to S3-131145.

S3-131145	Proposed changes on Key Issue #1
					Source: Telecom Italia

(Replaces S3-130908)

Abstract:

This is a proposed merge of TD S3-130908 and TD S3-130960, both pseudo-CRs related to the Key Issue #1: Configuration of ProSe-enabled UEs of SA3 draft-TR on ProSe.

Discussion:

Merger of 908 and 960.

Decision: 		The document was approved.

S3-130960	pCR to TR 33.cde: [Study on security issues to support Proximity Services]
					TR 33.cde v..
					Source: Alcatel-Lucent

Abstract:

This pCR proposes descriptive changes to sec 5.1 of TR 33.cde

Decision: 		The document was revised to S3-131145.

S3-131083	security solution for configuration data transfer
					Source: Samsung

Abstract:

-

Discussion:

The several security solutions for Proximity based Services are discussed in SA3 72 meeting. Among security solutions, the security for configuration of ProSe enabled UE is important to protect UEs and ProSe Function. However, the security solution for protect PC3 or PC8 is not sufficient and has limited assumptions. Therefore, in this contribution security solution is suggested to protect configuration data for UE.

Approved for inclusion in TR.

Decision: 		The document was approved.

S3-131084	security solution for prose service
					Source: Samsung

Abstract:

-

Discussion:

The following text is proposed for inclusion in the TR 33.cde to support security protection for Proximity based Services.   Alternative solution one and two are available, and after SA3 discussion, one of them will be selected as proposed solution.

Alternative Solution 1)

Between UE and Application server can be protected by the derived Key which is derived from IK, CK or KASME.

Alternative Solution 2)

Between UE and Application server can be protected using GBA/GAA mechanism and the Prose Function can support verification.

Phrasing need to be improved.

More work required. Not ready for inclusion in the TR.

Decision: 		The document was noted.

S3-131086	ProSe: new solution to address key issue 1
					3GPP TR 33.cde on ProSe v..
					Source: Gemalto, Morpho Cards, Orange

Abstract:

New solution to address key issue 1

Discussion:

Nokia expressed concern that this is not needed for interworking. Questioned the need to support all of the four reference specifications.		New solution to address key issue

Decision: 		The document was revised to S3-131148.

S3-131148	ProSe: new solution to address key issue 1
					3GPP TR 33.cde on ProSe v..
					Source: Gemalto, Morpho Cards, Orange

(Replaces S3-131086)

Discussion:

Approved for inclusion in draft TR,.

Decision: 		The document was approved.

S3-130909	Proposed changes on Key Issue #2
					Source: Telecom Italia

Abstract:

This pseudo-CR proposes some changes to the Key Issue #2: Security analysis for restricted ProSe discovery of SA3 draft-TR on ProSe.

Discussion:

No support for the proposed change. However, the underlying issue was found to be a Rel-12/13 issue, which might be resolved by modifying the editors note.

Update expected.

Decision: 		The document was noted.

S3-130910	Proposed change on Security requirements drawn for the Key Issue #3
					Source: Telecom Italia

Abstract:

This pseudo-CR proposes a change to the security requirements drawn for the Key Issue #3.

Discussion:

Comment that absolute prevention might not be possible, and that was the reason for the original "minimal" wording. Discussion led to new, more accurate, phrasing.

Approved with modification for inclusion in the draft TR.

Decision: 		The document was approved.

S3-130999	Security in ProSe Discovery
					Source: NEC Corporation

Abstract:

The key issue and solution of ProSe discovery has been studied in SA2 TR 23.703 v070. This document studies threats, security requirements and potential security solutions for ProSe discovery.

Discussion:

Clarified that in case on network interaction, UE ID forms part of its unauthenticated initial message. Heading to be modified. Editors notes to be added.

Approved with modifications.

Decision: 		The document was approved.

S3-131041	Security Aspects of ProSe Direct Discovery
					Source: Samsung

Abstract:

SA#66 endorsed the SA2 Prioritization Output in SP-130506, which includes ProSe Direct E-UTRA Discovery (Direct Discovery) as one of the component and mentioned that this component has SA3 dependency. SA2 is working on the solutions for Direct discovery in SA2 TR 23.703. This contribution discusses the security threats and detailed the security requirements and relevant solutions for Open discovery.

Discussion:

Clarifications and few additions needed.

Decision: 		The document was approved.

S3-130984	[ProSe]Security analysis for registration in Network based ProSe discovery
					Source: China Mobile, Intel

Abstract:

This document analysis the security threats in the registration process of ProSe, and gives the security requirement of the registration process in ProSe discovery.

Discussion:

Approved with clarifications for inclusion in the draft TR.

Decision: 		The document was approved.

S3-131030	[ProSe] Additional ProSe key issue
					Source: Intel

Abstract:

A key issue for Proximity Registration Procedure is added to the TR.

Discussion:

Some update required.

Decision: 		The document was revised to S3-131150.

S3-131150	[ProSe] Additional ProSe key issue
					Source: Intel

(Replaces S3-131030)

Discussion:

Approved with editorial modification.

Decision: 		The document was approved.

S3-131092	Updates to solution 2 on protection for restricted discovery information
					Source: Qualcomm Incorporated

Abstract:

-

Discussion:

Lots of clarifications. Some defacto requirements were identified and shall be stripped of any options. Extended discussion. Offline discussion required.

Decision: 		The document was revised to S3-131151.

S3-131151	Updates to solution 2 on protection for restricted discovery information
					Source: Qualcomm Incorporated

(Replaces S3-131092)

Decision: 		The document was approved.

S3-131093	[DRAFT] LS on timing synchronization
					Source: Qualcomm Incorporated

Abstract:

-

Discussion:

Questions need to be rephrased.

Decision: 		The document was revised to S3-131152.

S3-131152	 LS on timing synchronization
					Source: Qualcomm Incorporated

(Replaces S3-131093)

Decision: 		The document was approved.

S3-130911	Adding a new Key Issue on Ensuring a trusted and reliable accounting
					Source: Telecom Italia

Abstract:

This pseudo-CR proposes to add a new Key Issue on Ensuring a trusted and reliable accounting to the SA3 draft-TR on ProSe

Discussion:

Work in progress, open issues.

Decision: 		The document was revised to S3-131153.

S3-131153	Adding a new Key Issue on Ensuring a trusted and reliable accounting
					Source: Telecom Italia

(Replaces S3-130911)

Abstract:

This is a proposed merge of TD S3-130911 and TD S3-131042, both pseudo-CRs related to add a new key Issue on the same aspect to the SA3 draft-TR on ProSe.

Decision: 		The document was approved.

S3-131042	Security Aspects of charging in ProSe Direct communication
					Source: Samsung

Abstract:

-

Decision: 		The document was revised to S3-131153.

S3-130912	Adding a new Key Issue on Enforcing proper behaviour for non-Public Safety ProSe UEs
					Source: Telecom Italia

Abstract:

This pseudo-CR proposes to add a new Key Issue on Enforcing proper behaviour for non-Public Safety ProSe UEs to the SA3 draft-TR on ProSe.

Discussion:

Nokia: problematic to make normative something which cannot be tested. Key issue is that only authorized and non-compromised UEs shall be able to use ProSe feature. Particular concern that one-to-many communication is used in a fraudulent manner.

Decision: 		The document was noted.

S3-130913	Adding a new Key Issue on ProSe enabled UE security aspects
					Source: Telecom Italia

Abstract:

This pseudo-CR proposes to add a new Key Issue on ProSe enabled UE security aspects to the SA3 draft-TR on ProSe.

Discussion:

Debate on what requirements to add. Should a requirement be left out because it is unlikely that SA3 will find a solution? ADRIAN COMMENT? Support for the view that not all security issues need to be addressed with solutions, but risk assessment need to be done before neglecting a security issue.

More focused on the actual paper, the first threat listed were questioned and in open discovery not be threat at all.

Key issue details were accepted for inclusion.

Security threats need further discussion.

Security requirements are not yet agreed for inclusion.

Partially approved for inclusion in the draft TR.

Decision: 		The document was approved.

S3-131135	Draft TR ProSe
					Source: Rapporteur

Discussion:

Email approval.

Decision: 		The document was left for email approval and approved.

S3-131136	Exception sheet DraftTS ProSe
					Source: Rapporteur

Discussion:

Email approval

Decision: 		The document was left for email approval and approved.

S3-131195	Updated WID for ProSE
					Source: Rapporteur

Decision: 		The document was approved.

### 7.14	Security Aspects of Group Communication System Enablers for LTE

S3-131043	pCR GCSE skeleton
					Source: NSN

Abstract:

-

Discussion:

Agreed as foundation for the draft TR.

Decision: 		The document was approved.

S3-131044	pCR GCSE references
					Source: NSN

Abstract:

-

Discussion:

Agreed for inclusion in the draft TR.

Decision: 		The document was approved.

S3-131045	pCR GCSE Scope
					Source: NSN

Abstract:

-

Discussion:

Agreed for inclusion in the draft TR.

Suggestion to weaken the statement on what will be included in Rel-12. Anja: not all the solutions are to be included in the normative work.

This TR will look at security from system perspective and define functional security requirements for all entities. It should be noted that not all will be covered in Rel.12.

Decision: 		The document was approved.

S3-131046	pCR GCSE introduction
					Source: NSN

Abstract:

-

Discussion:

Agreed for inclusion in the draft TR.

Decision: 		The document was approved.

S3-131047	pCR GCSE architecture intro
					Source: NSN

Abstract:

-

Discussion:

Agreed for inclusion in the draft TR.

Decision: 		The document was approved.

S3-130965	GCSE_LTE security architecture
					Source: Huawei, HiSilicon

Abstract:

-

Discussion:

Merged into 1121.

Decision: 		The document was revised to S3-131121.

S3-131106	Comment on S3-130965-GCSE_LTE security architecture
					Source: NSN

Abstract:

-

Discussion:

merged into 1121.

Decision: 		The document was revised to S3-131121.

S3-130966	Security requirements for GCSE_LTE
					Source: Huawei, HiSilicon

Abstract:

-

Decision: 		The document was revised to S3-131120.

S3-131120	Merger of S3-130966 and S3-131052, GCSE Key issue – GC2 interface
					Source: Huawei, HiSilicon, NSN

(Replaces S3-130966)

Abstract:

Communication between GCSE AS and BM-SC needs to be secured.

Discussion:

Merger of 0966 and 1052.

Decision: 		The document was approved.

S3-131052	pCR GCSE key issue - GC2 interface
					Source: NSN

Abstract:

-

Discussion:

Merged into 1120

Decision: 		The document was revised to S3-131120.

S3-130994	GCSE_LTE: Key issue on performance and scalability
					Source: Home Office (UK)

Abstract:

Public safety usage of GCSE_LTE enabled group communications will require high levels of call setup performance, and potentially very large group sizes.  A key issue is proposed to highlight the relevance of this to the design of security mechanisms.

Discussion:

Three bullets under 5X3 to be replaced by a reference to SA requirements.

With these modification, the proposal is agreed.

Decision: 		The document was approved.

S3-130995	GCSE_LTE: Key issue on unicast/multicast modes
					Source: Home Office (UK)

Abstract:

Group communications are likely to be delivered over a mix of unicast and multicast bearers.  A key issue is proposed to highlight the importance of a consistent set of security mechanisms across the two modes.

Discussion:

Alex: interception might breach security for all members of a multicast community. The key mechanism need to allow security to be restored after lawful interception.

With modifications, the proposal is agreed for inclusion in the draft TR.

Decision: 		The document was approved.

S3-131038	GCSE: Key issue- Decision point for using PtP and/or PtM
					Source: China Unicom, Ericsson

Abstract:

-

Discussion:

Separate authentication and privacy issues. Some editorial modifications too. Clarify bullets are copied text. Agreed for inclusion into the draft TR.

Decision: 		The document was approved.

S3-131039	GCSE: Key issue- Enhanced Security in Critical Scenarios
					Source: China Unicom, Ericsson

Abstract:

-

Discussion:

X3 requirements to be replaced with "ffs" in some form. Title of section to be changed. Suggestion to clarify transmitting group members. Consensus that integrity is essential.

Decision: 		The document was approved.

S3-130996	GCSE_LTE: Key issue on calling party authentication
					Source: Home Office (UK)

Abstract:

There is a need in some forms of GCSE-based communication to be able to identify the transmitting party.  A key issue is proposed to highlight this.

Decision: 		The document was revised to S3-131131.

S3-131131	GCSE_LTE: Key issue on calling party authentication
					Source: Home Office (UK)

(Replaces S3-130996)

Abstract:

There is a need in some forms of GCSE-based communication to be able to identify the transmitting party.  A key issue is proposed to highlight this.

Discussion:

No further comments.

Decision: 		The document was approved.

S3-131049	pCR GSCE key issue - compromised UE
					Source: NSN

Abstract:

-

Discussion:

List of threats to be added to the draft TR. Tiny modifications agreed. Much longer debate about the type of threats to include in the list.

Decision: 		The document was revised to S3-131132.

S3-131132	pCR GSCE key issue - compromised UE
					Source: NSN

(Replaces S3-131049)

Abstract:

A Public Safety UE is illegitimately used. Threats are listed for this key issue.

Discussion:

No further comments.

Decision: 		The document was approved.

S3-131051	pCR GCSE key issue - securing keys in UE
					Source: NSN

Abstract:

-

Discussion:

Copied text from old specs appear a bit outdated. It was clarified that the references to MBMS serve to highlight existing security solutions which might be updated/reused. Agreed for inclusion in the draft TR.

Decision: 		The document was approved.

S3-131053	pCR GCSE key issue - join and leave
					Source: NSN

Abstract:

-

Discussion:

Merged into S3-131119

Decision: 		The document was revised to S3-131119.

S3-130934	GCSE: Key issues for public safety usage
					Source: Home Office (UK)

Abstract:

Aspects of the Stage 1 requirements for GCSE_LTE relevant to security for public safety issues are highlighted as key issues for SA3 to consider.

Discussion:

Includes 1053.

Decision: 		The document was revised to S3-131119.

S3-131119	GCSE_LTE: Key issue on adding/removing group members
					Source: Home Office (UK)

(Replaces S3-130934)

Discussion:

Merger of 0934 and 1053.

Agreed for inclusion in draft TR.

Decision: 		The document was approved.

S3-130993	GCSE_LTE: Key issue on interaction with ProSe security
					Source: Home Office (UK)

Abstract:

Public safety users of GCSE-enabled Group Communications are likely to also be using ProSe Group Communications.  A key issue is proposed to highlight the importance of considering this interaction in the design of security mechanisms.

Discussion:

Colin: does this accommodate use of same key, a master? That might require enhancements; foreseen for revision at next meeting.

Approved for inclusion in the draft TR.

Decision: 		The document was approved.

S3-131050	pCR GCSE key issue - MBMS security
					Source: NSN

Abstract:

Abstract of the contribution:

It needs to be analysed whether GCSE risks can be addressed by the mechanisms of MBMS security or whether additional mechanisms are needed.

Discussion:

Clarification that this is for SA3 to check the level of security for applying mitigation.

Approved for inclusion in draft TR.

Decision: 		The document was approved.

S3-131023	GCSE discussion paper on the way forward for eMBMS security
					Source: Intel

Abstract:

Regarding a way forward with eMBMS security for GCSE_LTE three possible options have been indentified.

Discussion:

Regarding a way forward with eMBMS security for GCSE_LTE we indentified three possible options:

1.  Do nothing at eMBMS level, since all confidentiality and integrity protection could be provided at the application layer (e.g. via SRTP).

2.  Use eMBMS security as defined in 33.246 (GBA + MSK distribution via HTTP/MIKEY).

3.  Use a subset of 33.246 (GBA for bootstrapping of MUK), plus MSK distribution via GC1. This is the current approach in OMA PoC.

GC2 is impacted only in the latter case.

Further analysis required. Subject partly beyond  the scope of Rel-12. Comment that unless this is done in Rel-12, it will be done by other organisation before 3GPP turns out Rel-13. Outgoing LS shall await further analysis.

Decision: 		The document was noted.

S3-131129	TR System Enablers for LTE
					Source: Rapporteur

Decision: 		The document was left for email approval and approved.

S3-131130	Exception sheet for GCSE_LTE
					Source: Rapporteur

Decision: 		The document was left for email approval and approved.

### 7.15	Other areas

S3-130949	LS on DASH Authentication
					Source: S4-130867

Abstract:

-

Decision: 		The document was replied to in S3-131187.

S3-130987	[DRAFT] Reply LS on DASH Authentication
					Source: Intel, Huawei

Abstract:

-

Discussion:

Numerous comments. Offline drafting required.

Decision: 		The document was revised to S3-131187.

S3-131187	Reply LS on DASH Authentication
					Source: Intel, Huawei

(Replaces S3-130987)

Discussion:

No further comments.

Decision: 		The document was approved.

S3-130947	Wi-Fi Roaming Task Force Signalling Optimisation (forwarded by SA2)
					Source: GSMA Wi-Fi Roaming Task Force

Abstract:

The Task Force concluded that one approach to control WLAN authentication signalling is for the operator to be able to set the aggressiveness of the authentication/association attempts of the UE to operator controlled AP/networks or AP/networks of roaming partners with operator policies e.g. provisioned from an ANDSF server. The task force further recognised the need for operators to be able to control WLAN authentication signalling more dynamically (depending on the WLAN signalling load situation) and for specific areas generating WLAN signalling load (e.g. signalling ‘hotspots’).

Discussion:

It was pointed out that only issue 2 and 3 in this documents appear to be in the scope of SA3 work.

Decision: 		The document was noted.

S3-130953	LS Announcing TUAK, an alternative to Milenage
					Source: SAGE (13) 08

Abstract:

-

Discussion:

A SAGE developed new 3G authentication and key agreement algorithm has been completed.  This new algorithm can be used by any operator, but is aimed particularly at the embedded UICC, where it may be sensible to have two strong algorithms installed on the platform and available for selection by subsequently loaded USIM applications.  This provides choice to operators; it also provides resilience against future cryptanalysis of either algorithm.

At the end of the handling of the TUAK documents, TeliaSonera commented that no design rationale had been provided by ETSI SAGE this time and that this felt somewhat unsatisfactory. NSN agreed that an Evaluation Report (or similar) would have been good to have available, also for the public. Vodafone replied that this request will asap be brought to ETSI SAGE chair attention

Decision: 		The document was noted.

S3-130954	LS on Full specification set for TUAK, an alternative to Milenage
					Source: SAGE (13) 09

Abstract:

-

Discussion:

For information.

Decision: 		The document was noted.

S3-130958	Work Item proposal: 3G Security: Specification of the TUAK algorithm set
					Source: AT&T, BlackBerry, China Mobile, Ericsson, Deutsche Telekom, Morpho Cards, NTT DOCOMO, Orange, Vodafone

Abstract:

-

Discussion:

Revised to include changes proposed in 1103 and offline comments.

Decision: 		The document was revised to S3-131182.

S3-131182	Work Item proposal: 3G Security: Specification of the TUAK algorithm set
					Source: AT&T, BlackBerry, China Mobile, Ericsson, Deutsche Telekom, Morpho Cards, NTT DOCOMO, Orange, Vodafone

(Replaces S3-130958)

Decision: 		The document was approved.

S3-131066	DRAFT TS: Specification of the TUAK Algorithm Set Document 1: Algorithm Specification
					Source: Vodafone

Abstract:

-

Decision: 		The document was revised to S3-131139.

S3-131139	DRAFT TS: Specification of the TUAK Algorithm Set Document 1: Algorithm Specification
					Source: Vodafone

(Replaces S3-131066)

Abstract:

The enclosed document 35.xxx is a first draft TS that copies of the Document 1 Algorithm Specification of the ETSI SAGE technical specification of the TUAK Algorithm Set into the 3GPP Technical Specification Template.

Discussion:

Improved Note 1 text proposed and accepted). Concern that method for switching between multiple security mechanisms will need to be defined, likely left proprietary.

Decision: 		The document was revised to S3-131196.

S3-131196	DRAFT TS: Specification of the TUAK Algorithm Set Document 1: Algorithm Specification
					Source: Vodafone

(Replaces S3-131139)

Discussion:

Agreed as basis for further work.

Decision: 		The document was revised to S3-131203.

S3-131203	DRAFT TS: Specification of the TUAK Algorithm Set Document 1: Algorithm Specification
					Source: Vodafone

(Replaces S3-131196)

Discussion:

revised to include changes agreed during the email approval

Decision: 		The document was left for email approval and approved.

S3-131067	DRAFT TS: Specification of the TUAK Algorithm Set, Document 2: Implementers Test Data
					Source: Vodafone

Abstract:

-

Decision: 		The document was revised to S3-131140.

S3-131140	DRAFT TS: Specification of the TUAK Algorithm Set, Document 2: Implementers Test Data
					Source: Vodafone

(Replaces S3-131067)

Decision: 		The document was revised to S3-131197.

S3-131197	DRAFT TS: Specification of the TUAK Algorithm Set, Document 2: Implementers Test Data
					Source: Vodafone

(Replaces S3-131140)

Discussion:

Agreed as basis for further work.

Decision: 		The document was agreed.

S3-131068	DRAFT TS: Specification of the TUAK Algorithm Set, Document 3: Design Conformance Test Data
					Source: Vodafone

Abstract:

-

Decision: 		The document was revised to S3-131141.

S3-131141	DRAFT TS: Specification of the TUAK Algorithm Set, Document 3: Design Conformance Test Data
					Source: Vodafone

(Replaces S3-131068)

Decision: 		The document was revised to S3-131198.

S3-131198	DRAFT TS: Specification of the TUAK Algorithm Set, Document 3: Design Conformance Test Data
					Source: Vodafone

(Replaces S3-131141)

Discussion:

Agreed as basis for further work.

Decision: 		The document was agreed.

S3-130941	LS/o on draft Recommendation ITU-T X.sap-8, Efficient multi-factor authentication mechanisms using mobile devices [to ISO/IEC JTC1 SC27/WG2 and 3GPP SA3]
					Source: ITU-T Study Group 17

Abstract:

-

Discussion:

BT noted that some comments sent to them a year ago has not been taken into account (a.o. virtual SIM card).

Does not seem to refer sufficiently to existing SA3 work.

Decision: 		The document was noted.

S3-131103	Comments on S3-130958, Work Item proposal: 3G Security: Specification of the TUAK algorithm set
					Source: Nokia, NSN

Abstract:

-

Discussion:

Justification section of the WID made more detailed. Additional rephrasing proposed. Add ISIM.

Decision: 		The document was noted.

S3-131104	Comments on S3-131066 (attachment), TUAK algorithm set, Document 1: Algorithm Specification
					Source: Nokia, NSN

Abstract:

-

Discussion:

Taken into account in the new version.

Decision: 		The document was noted.

S3-131186	Exception sheet for TS: Specification of the TUAK
					Source: SA3

Discussion:

Email approval

Decision: 		The document was left for email approval and approved.

## 8	Studies

### 8.1 	Study on Security aspects of Integration of Single Sign-On (SSO) frameworks with 3GPP networks

S3-130939	PCR to 33.895 - Sections 3 and  8.3.X
					TR 33.895 v..
					Source: Interdigital

Abstract:

This PCR proposes additional definitions in Section 3 as well as a new text in Section 8.3.X with considerations on multi-factor and multi-step authentication

Discussion:

Email approval.

Decision: 		The document was revised to S3-131205.

S3-131205	PCR to 33.895 - Sections 3 and  8.3.X
					TR 33.895 v..
					Source: Interdigital

(Replaces S3-130939)

Decision: 		The document was approved.

S3-130937	PCR to 33.895: Section 8.4.2.2  generalization of local authentication
					TR 33.895 v..
					Source: Interdigital

Abstract:

This PCR proposes modifications in Section 8.4.2.2  to allow generalized local user authentication and authorization

Discussion:

Left for Email approval.

Decision: 		The document was revised to S3-131204.

S3-131204	PCR to 33.895: Section 8.4.2.2  generalization of local authentication
					TR 33.895 v..
					Source: Interdigital

(Replaces S3-130937)

Decision: 		The document was approved.

S3-131031	pCR TR 33.895: Terminology correction
					Source: Ericsson

Abstract:

-

Discussion:

Email approval.

Decision: 		The document was left for email approval and approved.

S3-131082	pCR TR33.895: GBA_U with user consent
					3GPP TR 33.895 v..
					Source: Gemalto, Ericsson, Morpho Cards

Abstract:

pCR to TR 33.895: GBA_U with user consent

Discussion:

Email approval.

Decision: 		The document was left for email approval and approved

S3-131200	Draft TR SSO study
					Source: Rapporteur

Decision: 		The document was left for email approval and approved.

### 8.2 	Security Study on Spoofed Call Detection and Prevention

S3-131109	Information of spoof call in real networks
					Source: China Mobile

Abstract:

-

Discussion:

Email approval.

Decision: 		The document was left for email approval and noted.

S3-131110	Conclusion of FS_SPOOF
					Source: China Mobile

Abstract:

-

Discussion:

Revised following offline discussion.

Decision: 		The document was revised to S3-131149.

S3-131149	Conclusion of FS_SPOOF
					Source: China Mobile

(Replaces S3-131110)

Discussion:

Email approval.

Decision: 		The document was left for email approval and approved.

S3-131111	Information of spoof call in real networks
					Source: China Mobile

Abstract:

-

Discussion:

Email approval.

Decision: 		The document was left for email approval and noted.

S3-131201	Draft TR spoofed call study
					Source: Rapporteur

Decision: 		The document was left for email approval and approved.

### 8.3 	Study Item on Security Assurance Methodology for 3GPP Network Elements

S3-131048	LS on Security Assurance Methodology for 3GPP Network Products
					Source: GSMA Security Group

Abstract:

-

Discussion:

SG would like to inform SA3 of the creation of a new subgroup called NESAG (Network Equipment Security Assurance Group) with the intention of contributing to the standardisation effort led by SA3 on the security assurance work (SECAM), particularly on the administrative framework and Accreditation Body aspects. SG proposes that NESAG defines the administrative framework to implement in practice the SECAM methodology chosen by SA3 in TR33.805 v1.2.1.

SG proposes that the administrative framework should study the following aspects:

•	administrative structure and organisation

•	accreditation process including scheduling, criteria and associated acceptable evidences to obtain accreditation (for development process and for testing labs)

•	selection of partners and running of trials

•	dispute resolution process

•	funding, operational capabilities and resources to maintain the related documentation

NESAG will be open to both GSMA and 3GPP members. 3GPP members who are not members of the GSMA will be accepted but strongly encouraged to become GSMA members. NESAG will follow the GSMA working group rules: it will have an interim chair and elections will be organised to choose a chair.

Decision: 		The document was noted.

S3-130983	[SECAM] Discussion on pilot node selection
					Source: China Mobile, BT Group,TeliaSonera,Orange, Telecom Italia, China Unicom

Abstract:

-

Discussion:

It is understood from discussion that most companies favour to look at the MME pro’s arguments - that speak for working on the MME as SECAM pilot.

The SA3 decided to take MME as the SECAM pilot.

Decision: 		The document was noted.

S3-130918	[SECAM] Normative WID for SECAM
					Source: Orange, NTT DOCOMO, Deutsche Telekom, Telecom Italia, BT, China Mobile, China Unicom, TeliaSonera, Vodafone, NSN, Juniper Networks, NEC, Huawei, Ericsson

Abstract:

-

Discussion:

Revised before presentation.

Decision: 		The document was revised to S3-130961.

S3-131161	[SECAM] Normative WID for SECAM
					Source: Orange, NTT DOCOMO, Deutsche Telekom, Telecom Italia, BT, China Mobile, China Unicom, TeliaSonera, Vodafone, NSN, Juniper Networks, NEC, Huawei, Ericsson

(Replaces S3-130918)

Discussion:

Change of "will" to "can" in added text regarding potential regulatory requirements.

Decision: 		The document was approved.

S3-131070	First outline version of 900 series TR describing the general SECAM scheme
					Source: NSN, Orange

Abstract:

-

Discussion:

Revised before presentation.

Decision: 		The document was revised to S3-131162.

S3-131162	First outline version of 900 series TR describing the general SECAM scheme
					Source: NSN, Orange

(Replaces S3-131070)

Discussion:

Approved as baseline for further work.

Decision: 		The document was approved.

S3-130926	[SECAM] LS to GSMA SG
					Source: Orange

Abstract:

-

Discussion:

Revised before presentation.

Decision: 		The document was revised to S3-131163.

S3-131163	[SECAM] LS to GSMA SG
					Source: Orange

(Replaces S3-130926)

Discussion:

No further comments.

Decision: 		The document was approved.

S3-130923	[SECAM] Conclusion section - editorials
					Source: Orange, NSN, Telecom Italia

Abstract:

-

Discussion:

Approved for inclusion in draft TR.

Decision: 		The document was approved.

S3-130922	[SECAM] Conclusion section - comparison table M2/CC
					Source: Orange, NSN, Telecom Italia

Abstract:

-

Discussion:

Merged into 1164

Decision: 		The document was revised to S3-131164.

S3-131164	[SECAM] Conclusion section - comparison table M2/CC
					Source: Orange, NSN, Telecom Italia, Huawei, Hisilicon, China Mobile

(Replaces S3-130922)

Discussion:

Merger of 922, 1021 and 1100.

Approved for inclusion in draft TR.

Decision: 		The document was approved.

S3-131021	comments on S3-130922
					Source: Huawei, Hisilicon

Abstract:

-

Discussion:

Merged into 1164

Decision: 		The document was revised to S3-131164.

S3-131100	comments on S3-130922
					Source: China Mobile

Abstract:

-

Discussion:

Merged into 1164

Decision: 		The document was revised to S3-131164.

S3-130924	[SECAM] TOE/TSF section - clarifications
					Source: Orange

Abstract:

-

Discussion:

Debate on need for and proposals for new definitions.

Approved with modification.

Decision: 		The document was approved with modifications.

S3-131014	[SECAM] Accreditation Validity
					Source: China Mobile

Abstract:

-

Discussion:

No comments.

Decision: 		The document was approved.

S3-131016	[SECAM] Information needed to execute the required tests for the Basic Vulnerability Testing
					Source: China Mobile,Orange,NSN

Abstract:

-

Discussion:

Merged into 1165

Decision: 		The document was revised to S3-131165.

S3-131165	[SECAM] Information needed to execute the required tests for the Basic Vulnerability Testing
					Source: China Mobile,Orange,NSN

(Replaces S3-131016)

Discussion:

Merger of 1016 and 1101.

Decision: 		The document was approved.

S3-131101	[SECAM] Comments on S3-13016
					Source: China Mobile, Orange, NSN

Abstract:

-

Discussion:

Merged into 1165

Decision: 		The document was revised to S3-131165.

S3-131032	clarification on mandatory requirements
					Source: Huawei, Hisilicon, China Unicom

Abstract:

This contribution propose to clarify the meaning of the “mandatory” requirements. The mandatory requirements don’t mean the network product has to fulfil the requirements and pass the test. they just mean the requirements have to be tested. They are different with the “conditional” requirements which are just needed to be tested under certain conditions. so we add the “for testing” after the mandatory and conditional requirements phrase to express it more clear.

Discussion:

Approved with modification

Decision: 		The document was approved.

S3-131015	[SECAM] New security requirements
					Source: China Mobile

Abstract:

-

Discussion:

Approved with modification for inclusion in new TR

Decision: 		The document was approved.

S3-131113	SECAM practice in China Mobile
					Source: China Mobile

Abstract:

This contribution introduces China Mobile’s methodology in security assurance. The assessment path and some analysis are provided according to real network practice.

Discussion:

For information only.

Decision: 		The document was noted.

S3-130925	[SECAM] coversheet for presentation of TR 33.805 to SA
					Source: Orange, Telecom Italia

Abstract:

-

Discussion:

Revised due to editorials.

Decision: 		The document was approved.

S3-131178	[SECAM] coversheet for presentation of TR 33.805 to SA
					Source: Orange, Telecom Italia

(Replaces S3-130925)

Discussion:

No further comments.

Decision: 		The document was approved.

S3-130914	Accreditation Validity
					Source: delegate

Abstract:

-

Decision: 		The document was withdrawn.

S3-130915	New security requirements
					Source: delegate

Abstract:

-

Decision: 		The document was withdrawn.

S3-130916	New security requirements
					Source: delegate

Abstract:

-

Decision: 		The document was withdrawn.

S3-130917	Information needed to execute the required tests for the Basic Vulnerability Testing
					Source: delegate

Abstract:

-

Decision: 		The document was withdrawn.

S3-130988	clarification on mandatory requirements
					Source: Huawei, Hisilicon, China Unicom

Abstract:

-

Decision: 		The document was withdrawn.

S3-131097	comments on S3-130922
					Source: China Mobile

Abstract:

-

Decision: 		The document was withdrawn.

S3-131166	Outcome of evening session.
					Source: Rapperteur

Discussion:

It was requested to have minuted this text:

“In the SECAM normative WID, SA3 will start with a 9xx series TR for the third output document “Security Assurance scheme for 3GPP Network Products”. This document might be upgraded to a TS as needed if the use of normative language becomes necessary”

Decision: 		The document was noted.

S3-131167	Development requirements
					Source: Orange

Discussion:

Approved for inclusion in the draft TR.

Decision: 		The document was approved.

S3-131168	Draft TR on SECAM
					Source: Rapporteur

Discussion:

Presented for approval as draft.

Decision: 		The document was approved.

### 8.4 	Study on Subscriber Privacy Impact in 3GPP

S3-131033	Skeleton of SPI TR
					Source: China Unicom

Abstract:

-

Discussion:

Left for email approval.

Decision: 		The document was revised to S3-131211.

S3-131211	Skeleton of SPI TR
					Source: China Unicom

(Replaces S3-131033)

Discussion:

Merge of 1033 and 992

Decision: 		The document was approved.

S3-130991	Structure of a Privacy Report
					Source: Nokia Corporation, NSN

Abstract:

-

Discussion:

This is a discussion paper motivating 992. 992 was then merged with 1033 into S3-131211

Decision: 		The document was left for email approval and noted.

S3-130992	Draft Skeleton for TR on Study on Subscriber Privacy Impact in 3GPP
					Source: Nokia Corporation, NSN

Abstract:

-

Discussion:

Left for email approval.

Decision: 		The document was revised to S3-131211.

S3-131036	Reference for SPI
					Source: China Unicom

Discussion:

Email approval.

Decision: 		The document was left for email approval and approved.

S3-131040	Scope of SPI TR
					Source: China Unicom, Ericsson

Abstract:

-

Discussion:

Email approval.

Decision: 		The document was left for email approval and approved.

S3-131003	Privacy study purpose
					Source: Ericsson, China Unicom

Abstract:

-

Discussion:

Email approval.

Decision: 		The document was left for email approval and revised.

S3-131212	Privacy study purpose
					Source: Ericsson, China Unicom

(Replaces S3-131003)

Discussion:

Email approval.

Decision: 		The document was approved.

S3-130977	PII Privacy
					Source: Huawei, HiSilicon,CATR

Abstract:

-

Discussion:

Email approval.

Decision: 		The document was left for email approval and approved

S3-130978	User and UE Identity Privacy
					Source: Huawei, HiSilicon,CATR, China Unicom

Abstract:

-

Discussion:

Email approval.

Decision: 		The document was left for email approval and approved.

S3-131002	Inclusion of the OECD privacy principles
					Source: Ericsson, China Unicom

Abstract:

-

Discussion:

Email approval.

Decision: 		The document was left for email approval and approved.

S3-131037	Key issue- Privacy vs. System Functionality and Availability
					Source: China Unicom

Abstract:

-

Discussion:

Email approval.

Decision: 		The document was left for email approval and noted

S3-131035	Scope of SPI TR
					Source: China Unicom

Abstract:

-

Decision: 		The document was withdrawn.

S3-131202	Draft TR on privacy (study)
					Source: Rapporteur

Decision: 		The document was left for email approval and approved.

### 8.5	Other Study Areas

## 10	Elections

Chairman's election:

Dr. Anand PRASAD, NEC Corporation, TTC, 44 votes, 70,97%

Mr. Marcus WONG, HUAWEI TECHNOLOGIES Co. Ltd., 18 votes, 29,03%

Following presentation of this result, Mr. Marcus Wong withdrew his candidature so Anand Prasad became elected Chairman of TSG SA WG3

1st vice chair

Mrs Judy ZHU, China Mobile Com. Corporation, CCSA, 25 votes, 41,4%

Dr. Alf ZUGENMAIER, NTT DOCOMO INC. ARIB, 34 votes, 58,6%.

Following presentation of this result, Mrs Judy ZHU withdraw her candidature, leaving Dr. Alf ZUGENMAIER as elected vice chairman of TSG SA WG3.

2nd vice chair

Mrs Judy ZHU, China Mobile Com. Corporation, CCSA, 36 votes, 62,1%

Mrs Mireille PAULIAC, Gemalto N.V., ETSI, 22 votes, 37,9%

A second voting round for 2nd vice chair was required, with this outcome:

Mrs Judy ZHU, China Mobile Com. Corporation, CCSA, 35 votes, 59,3%

Mrs Mireille PAULIAC, Gemalto N.V., ETSI, 24 votes, 40,7%

Therefore Ms. Judy Zhu was elected for the second vice chair position in SA3.

Following presentation of this result, Alex Leadbeater from BT asked for it to be minuted that the principles for regional balancing of 3GPP officials set out in article 22 of the 3GPP Working Procedures had not been respected with this election at SA3-73.

## 11	Review and Update of Work Plan

Exception sheets for:

- WebRTC

- ProSe

- WLAN-NS

- GCSE

- TUAK

- MTC

S3-130904	SA3 Work Plan
					Source: ETSI Secretariat

Abstract:

-

Discussion:

Presented for information.

Decision: 		The document was noted.

## 12	Future Meeting Dates and Venues

A number of items was sent on email approval:

Deadlines for exception sheets and coversheets.

Deadline for first version: Wednesday nov 20,

comments until friday nov 22

Final version until nov 25.

Draft TRs:

1st version until fri nov 22.

comments until nov 29

final version wed dec 04.

3 studies plus MTC are on email approval start Monday 25 nov. end fri dec 06.

result confirmation wed 11.

Email approval of MTC SSO, spoofed call and privacy TR: from fri dec 13. until dec 18. final version out friday dec 20.

It was proposed to attempt to arrange an extra meeting in first half of 2014. It will be an adhoc with limited agenda.

3 day adhoc mon-wed western Europe (ETSI?) in week 14, 2014.

S3-130902	SA3 meeting calendar
					Source: ETSI Secretariat

Abstract:

-

Discussion:

Presented for information.

Decision: 		The document was noted.

## 13	Any Other Business

Surprise thanks for leaving chairman and leaving delegate (Loic Benjamin from Orange).

## Annex A: List of contribution documents

| Document | Title | Source | Decision | Replaces | Replaced by |
| --- | --- | --- | --- | --- | --- |
| S3-130900 | Agenda | WG Chairman | approved | - | - |
| S3-130901 | Report from last SA meeting | WG Chairman | noted | - | - |
| S3-130902 | SA3 meeting calendar | ETSI Secretariat | noted | - | - |
| S3-130903 | Report from last SA3 meeting | ETSI Secretariat | approved | - | - |
| S3-130904 | SA3 Work Plan | ETSI Secretariat | noted | - | - |
| S3-130905 | Proposed changes to the Definitions section for ProSe | Telecom Italia, Orange | approved | - | - |
| S3-130906 | Adding General Requirements on ProSe Security, Authorization and Privacy | Telecom Italia | revised | - | S3-131194 |
| S3-130907 | Proposed changes to Clause 4 | Telecom Italia | approved | - | - |
| S3-130908 | Proposed changes on Key Issue #1 | Telecom Italia | revised | - | S3-131145 |
| S3-130909 | Proposed changes on Key Issue #2 | Telecom Italia | noted | - | - |
| S3-130910 | Proposed change on Security requirements drawn for the Key Issue #3 | Telecom Italia | approved | - | - |
| S3-130911 | Adding a new Key Issue on Ensuring a trusted and reliable accounting | Telecom Italia | revised | - | S3-131153 |
| S3-130912 | Adding a new Key Issue on Enforcing proper behaviour for non-Public Safety ProSe UEs | Telecom Italia | noted | - | - |
| S3-130913 | Adding a new Key Issue on ProSe enabled UE security aspects | Telecom Italia | approved | - | - |
| S3-130914 | Accreditation Validity | delegate | withdrawn | - | - |
| S3-130915 | New security requirements | delegate | withdrawn | - | - |
| S3-130916 | New security requirements | delegate | withdrawn | - | - |
| S3-130917 | Information needed to execute the required tests for the Basic Vulnerability Testing | delegate | withdrawn | - | - |
| S3-130918 | [SECAM] Normative WID for SECAM | Orange, NTT DOCOMO, Deutsche Telekom, Telecom Italia, BT, China Mobile, China Unicom, TeliaSonera, Vodafone, NSN, Juniper Networks, NEC, Huawei, Ericsson | revised | - | S3-130961 |
| S3-130919 | CR to TR 33.980 Correction of reference | Nokia Corporation, NSN | revised | - | s3-131171 |
| S3-130920 | CR to TS 33.221 Correction of Reference | Nokia Corporation, NSN | revised | - | S3-131172 |
| S3-130921 | CR to TR 33.823 Update of W3C references | Nokia Corporation, NSN | agreed | - | - |
| S3-130922 | [SECAM] Conclusion section - comparison table M2/CC | Orange, NSN, Telecom Italia | revised | - | S3-131164 |
| S3-130923 | [SECAM] Conclusion section - editorials | Orange, NSN, Telecom Italia | approved | - | - |
| S3-130924 | [SECAM] TOE/TSF section - clarifications | Orange | approved | - | - |
| S3-130925 | [SECAM] coversheet for presentation of TR 33.805 to SA | Orange, Telecom Italia | approved | - | - |
| S3-130926 | [SECAM] LS to GSMA SG | Orange | revised | - | S3-131163 |
| S3-130927 | Incorrect reference for keep-alive mechanism | Ericsson | revised | - | S3-131098 |
| S3-130928 | Updated reference for secure fax | Ericsson | revised | - | S3-131099 |
| S3-130929 | Work Item proposal: 3G Security: Specification of the TUAK algorithm set | BlackBerry, China Mobile, Ericsson, Deutsche Telekom, Morpho Cards, Orange, Vodafone | withdrawn | - | - |
| S3-130930 | Requirement of Network based filtering solution in SMS router | ZTE Corporation, China Unicom | noted | - | - |
| S3-130931 | Correction to network based solution for filtering SMS-delivered device trigger messages | ZTE Corporation, China Unicom | noted | - | - |
| S3-130932 | Restrictions on signature length in PWS security | Ericsson, Nokia Corporation, NSN | noted | - | - |
| S3-130933 | ProSe: Key issues on Group Communications | Home Office (UK) | revised | - | S3-131137 |
| S3-130934 | GCSE: Key issues for public safety usage | Home Office (UK) | revised | - | S3-131119 |
| S3-130935 | LS on TLS protocol profile for eMediasec | C4-131868 | noted | - | - |
| S3-130936 | TCG progress report in the areas of TMS and MPWG | Interdigital | approved | - | - |
| S3-130937 | PCR to 33.895: Section 8.4.2.2 generalization of local authentication | Interdigital | revised | - | S3-131204 |
| S3-130938 | LS on SIP/SDP Usage for TCP/TLS session establishment | C4-131508 | noted | - | - |
| S3-130939 | PCR to 33.895 - Sections 3 and 8.3.X | Interdigital | revised | - | S3-131205 |
| S3-130940 | LS on new work item on 'IMS support for RTP / RTCP transport multiplexing | C4-131546 | replied to | - | S3-131133 |
| S3-130941 | LS/o on draft Recommendation ITU-T X.sap-8, Efficient multi-factor authentication mechanisms using mobile devices [to ISO/IEC JTC1 SC27/WG2 and 3GPP SA3] | ITU-T Study Group 17 | noted | - | - |
| S3-130942 | LS on interactions of oneM2M with Underlying Networks | OneM2M | noted | - | - |
| S3-130943 | LS on security aspects of protocol architectures for small cell enhancements | R2-133018 | noted | - | - |
| S3-130944 | Reply LS on security aspects of protocol architectures for small cell enhancements | R2-133650 | noted | - | - |
| S3-130945 | Reply LS on security aspects of protocol architectures for small cell enhancements | R3-131919 | noted | - | - |
| S3-130946 | Answer LS to oneM2M on interactions of oneM2M with Underlying Networks | S1-134175 | noted | - | - |
| S3-130947 | Wi-Fi Roaming Task Force Signalling Optimisation (forwarded by SA2) | GSMA Wi-Fi Roaming Task Force | noted | - | - |
| S3-130948 | Reply LS to CT WG4 LS on new work item on 'IMS support for RTP / RTCP transport multiplexing | S2-133825 | noted | - | - |
| S3-130949 | LS on DASH Authentication | S4-130867 | replied to | - | - |
| S3-130950 | LS to SA3 on security aspects of CMPv2 protocol used for certificate enrolment procedures in MvPnP | S5-131458 | noted | - | - |
| S3-130951 | Addition of TLS profile for eMediaSec | Alcatel-Lucent | revised | - | S3-131102 |
| S3-130952 | LS to SA3 on necessary information for certificate enrolment procedures in MvPnP | S5-131789 | replied-to | - | - |
| S3-130953 | LS Announcing TUAK, an alternative to Milenage | SAGE (13) 08 | noted | - | - |
| S3-130954 | LS on Full specification set for TUAK, an alternative to Milenage | SAGE (13) 09 | noted | - | - |
| S3-130955 | LS on Group Communication Security in LTE | ETSI TC TCCE | replied to | - | - |
| S3-130956 | Proposed Reply LS on security aspects of protocol architectures for small cell enhancements | Alcatel-Lucent | noted | - | - |
| S3-130957 | Discussion on security aspects of protocol architectures for small cell enhancements | Alcatel-Lucent | noted | - | - |
| S3-130958 | Work Item proposal: 3G Security: Specification of the TUAK algorithm set | AT&T, BlackBerry, China Mobile, Ericsson, Deutsche Telekom, Morpho Cards, NTT DOCOMO, Orange, Vodafone | revised | - | S3-131182 |
| S3-130959 | pCR: Conclusion section of TR 33.869 | BlackBerry, BT, Huawei, Vodafone | noted | - | - |
| S3-130960 | pCR to TR 33.cde: [Study on security issues to support Proximity Services] | Alcatel-Lucent | revised | - | S3-131145 |
| S3-130961 | pCR to TR 33.cde Sec.6.3.4.3.2 | Alcatel-Lucent | approved | - | - |
| S3-130962 | Security for UE Power Consumption Optimisation (UEPCOP) | Huawei, HiSilicon | noted | - | - |
| S3-130963 | Solving EN for terminology used to identify keys | Huawei, HiSilicon | approved | - | - |
| S3-130964 | Corrections on NAS based solution | Huawei, HiSilicon | approved | - | - |
| S3-130965 | GCSE_LTE security architecture | Huawei, HiSilicon | revised | - | S3-131121 |
| S3-130966 | Security requirements for GCSE_LTE | Huawei, HiSilicon | revised | - | S3-131120 |
| S3-130967 | Device trigger over T5 interface | Huawei, HiSilicon | noted | - | - |
| S3-130968 | Deletion of editor notes | Huawei, HiSilicon | revised | - | S3-131209 |
| S3-130969 | Detail the description of partly ciphering SDT solution | Huawei, HiSilicon | revised | - | S3-131206 |
| S3-130970 | Considerations on the MTC-IWF based SDT security solution | Huawei, HiSilicon | revised | - | S3-131207 |
| S3-130971 | draft Response LS on security aspects of protocol architectures for small cell enhancements | Huawei, HiSilicon | noted | - | - |
| S3-130972 | key issue for ProSe communications in Group Owner mode | Huawei, HiSilicon | revised | - | S3-131138 |
| S3-130973 | Security for PrSe UE communication in group owner mode | Huawei, HiSilicon | revised | - | S3-131143 |
| S3-130974 | Security for ProSe communication through UE-to-Network relay with network authorization | Huawei, HiSilicon | revised | - | S3-131147 |
| S3-130975 | Correction and Deletion of the Editor notes | Huawei, HiSilicon | approved | - | - |
| S3-130976 | Details of solution 2 supporting security parameters of selection policies | Huawei, HiSilicon | approved | - | - |
| S3-130977 | PII Privacy | Huawei, HiSilicon,CATR | approved | - | - |
| S3-130978 | User and UE Identity Privacy | Huawei, HiSilicon,CATR, China Unicom | approved | - | - |
| S3-130979 | Discussion: Work plan and prioritization for the WebRTC security work | Ericsson | noted | - | - |
| S3-130980 | Update of WebRTC WID | Ericsson | approved | - | - |
| S3-130981 | (Draft) LS on WebRTC Security Responsibility | Ericsson | revised | - | S3-131193 |
| S3-130982 | WebRTC TR 33.abc skeleton | Ericsson | revised | - | S3-131192 |
| S3-130983 | [SECAM] Discussion on pilot node selection | China Mobile, BT Group,TeliaSonera,Orange, Telecom Italia, China Unicom | noted | - | - |
| S3-130984 | [ProSe]Security analysis for registration in Network based ProSe discovery | China Mobile, Intel | approved | - | - |
| S3-130985 | Evaluation on Security of Small Data Transmission | China Mobile | revised | - | S3-131208 |
| S3-130986 | Working way and evaluation on restricting the USIM to specific MEs/MTC Devices | China Mobile, TeliaSonera | noted | - | - |
| S3-130987 | [DRAFT] Reply LS on DASH Authentication | Intel, Huawei | revised | - | S3-131187 |
| S3-130988 | clarification on mandatory requirements | Huawei, Hisilicon, China Unicom | withdrawn | - | - |
| S3-130989 | Discussion on Security Aspects of SCE UP Architecture | Samsung | noted | - | - |
| S3-130990 | Discussion on Reply LS on Security Aspects of Protocol Architectures for SCE | Samsung | noted | - | - |
| S3-130991 | Structure of a Privacy Report | Nokia Corporation, NSN | noted | - | - |
| S3-130992 | Draft Skeleton for TR on Study on Subscriber Privacy Impact in 3GPP | Nokia Corporation, NSN | revised | - | S3-131211 |
| S3-130993 | GCSE_LTE: Key issue on interaction with ProSe security | Home Office (UK) | approved | - | - |
| S3-130994 | GCSE_LTE: Key issue on performance and scalability | Home Office (UK) | approved | - | - |
| S3-130995 | GCSE_LTE: Key issue on unicast/multicast modes | Home Office (UK) | approved | - | - |
| S3-130996 | GCSE_LTE: Key issue on calling party authentication | Home Office (UK) | revised | - | S3-131131 |
| S3-130997 | MTC-IWF based solution editors notes | NEC Corporation | revised | - | S3-131210 |
| S3-130998 | MTC device triggering editors notes in solution 6 | NEC Corporation | approved | - | - |
| S3-130999 | Security in ProSe Discovery | NEC Corporation | approved | - | - |
| S3-131000 | Small cell enhancements security aspects | NEC Corporation | withdrawn | - | - |
| S3-131001 | Draft reply LS on security aspects of protocol architectures for small cell enhancements | NEC Corporation | noted | - | - |
| S3-131002 | Inclusion of the OECD privacy principles | Ericsson, China Unicom | approved | - | - |
| S3-131003 | Privacy study purpose | Ericsson, China Unicom | revised | - | S3-131212 |
| S3-131004 | Deployment option for CA/RA | Ericsson | noted | - | - |
| S3-131005 | Draft LS reply security aspects of CMPv2 protocol used for certificate enrolment procedures in MvPnP | Ericsson | revised | - | S3-131116 |
| S3-131006 | Security analysis of small cell split PDCP architectures | Ericsson | noted | - | - |
| S3-131007 | Draft LS reply on small cell split PDCP architectures | Ericsson | noted | - | - |
| S3-131008 | Using IMS e2e solutions for direct one-to-one communications | CESG (UK) | approved | - | - |
| S3-131009 | A security context for direct one-to-one communications | CESG (UK) | noted | - | - |
| S3-131010 | IDENTITY Solution for Public Safety ProSe | CESG (UK) | noted | - | - |
| S3-131011 | IDENTITY Solution for ProSe one-to-one communications | CESG (UK) | approved | - | - |
| S3-131012 | ProSe security using a combination of LTE security and an overlay | Ericsson | revised | - | S3-131134 |
| S3-131013 | pCR to TR 33.abc: [Study on Security for WebRTC IMS Client access to IMS] | Alcatel-Lucent | revised | - | S3-131123 |
| S3-131014 | [SECAM] Accreditation Validity | China Mobile | approved | - | - |
| S3-131015 | [SECAM] New security requirements | China Mobile | approved | - | - |
| S3-131016 | [SECAM] Information needed to execute the required tests for the Basic Vulnerability Testing | China Mobile,Orange,NSN | revised | - | S3-131165 |
| S3-131017 | Small cell enhancements security aspects | NEC Corporation | noted | - | - |
| S3-131018 | Correction of a typo | Nokia Corporation, NSN, NEC Corporation | agreed | - | - |
| S3-131019 | Proposed the solution of media over TCP based on ICE for TR 33.830 | participant | withdrawn | - | - |
| S3-131020 | pCR on the evaluation of MTC Secure Connection solutions | Nokia Corporation, NSN | noted | - | - |
| S3-131021 | comments on S3-130922 | Huawei, Hisilicon | revised | - | S3-131164 |
| S3-131022 | pCR: Clarifications on Implicit Certificate approach | BlackBerry UK Ltd. | approved | - | - |
| S3-131023 | GCSE discussion paper on the way forward for eMBMS security | Intel | noted | - | - |
| S3-131024 | Proposed the solution of media over TCP based on ICE for TR 33.830 | Huawei, Hisilicon | withdrawn | - | - |
| S3-131025 | Solution for PWS TS | Blackberry,Huawei, HiSilicon,Vodafone | noted | - | - |
| S3-131026 | pCR 33.abc Overview | Ericsson | revised | - | S3-131122 |
| S3-131027 | pCR 33.abc Authentication of WebRTC client re-using existing IMS authentication mechanisms | Ericsson | revised | - | S3-131123 |
| S3-131028 | pCR 33.abc Authentication of WebRTC client using web credentials | Ericsson | revised | - | S3-131123 |
| S3-131029 | pCR 33.abc Use of DTLS-SRTP for SRTP key management | Ericsson | revised | - | S3-131124 |
| S3-131030 | [ProSe] Additional ProSe key issue | Intel | revised | - | S3-131150 |
| S3-131031 | pCR TR 33.895: Terminology correction | Ericsson | approved | - | - |
| S3-131032 | clarification on mandatory requirements | Huawei, Hisilicon, China Unicom | approved | - | - |
| S3-131033 | Skeleton of SPI TR | China Unicom | revised | - | S3-131211 |
| S3-131034 | IDENTITY Solution for ProSe group communications | CESG (UK) | approved | - | - |
| S3-131035 | Scope of SPI TR | China Unicom | withdrawn | - | - |
| S3-131036 | Reference for SPI | China Unicom | approved | - | - |
| S3-131037 | Key issue- Privacy vs. System Functionality and Availability | China Unicom | revised | - | - |
| S3-131038 | GCSE: Key issue- Decision point for using PtP and/or PtM | China Unicom, Ericsson | approved | - | - |
| S3-131039 | GCSE: Key issue- Enhanced Security in Critical Scenarios | China Unicom, Ericsson | approved | - | - |
| S3-131040 | Scope of SPI TR | China Unicom, Ericsson | approved | - | - |
| S3-131041 | Security Aspects of ProSe Direct Discovery | Samsung | approved | - | - |
| S3-131042 | Security Aspects of charging in ProSe Direct communication | Samsung | revised | - | S3-131153 |
| S3-131043 | pCR GCSE skeleton | NSN | approved | - | - |
| S3-131044 | pCR GCSE references | NSN | approved | - | - |
| S3-131045 | pCR GCSE Scope | NSN | approved | - | - |
| S3-131046 | pCR GCSE introduction | NSN | approved | - | - |
| S3-131047 | pCR GCSE architecture intro | NSN | approved | - | - |
| S3-131048 | LS on Security Assurance Methodology for 3GPP Network Products | GSMA Security Group | noted | - | - |
| S3-131049 | pCR GSCE key issue - compromised UE | NSN | revised | - | S3-131132 |
| S3-131050 | pCR GCSE key issue - MBMS security | NSN | approved | - | - |
| S3-131051 | pCR GCSE key issue - securing keys in UE | NSN | approved | - | - |
| S3-131052 | pCR GCSE key issue - GC2 interface | NSN | revised | - | S3-131120 |
| S3-131053 | pCR GCSE key issue - join and leave | NSN | revised | - | S3-131119 |
| S3-131054 | Secure Connection details on GBA based solutions | Ericsson | approved | - | - |
| S3-131055 | Solution for Secure Connection in draft TS 33.187 | Ericsson | revised | - | S3-131160 |
| S3-131056 | GBA for constrained devices | Ericsson | revised | - | S3-131158 |
| S3-131057 | pCR to 23.401: Solution of restricting the USIM to specific UEs | Ericsson | noted | - | - |
| S3-131058 | pCR to 23.060: Solution of restricting the USIM to specific UEs | Ericsson | noted | - | - |
| S3-131059 | Clarification of NAF key negotiation on Ua | Ericsson, Nokia Corporation, Nokia Solutions and Networks | revised | - | S3-131173 |
| S3-131060 | Clarification of NAF key negotiation on Ua | Ericsson, Nokia Corporation, Nokia Solutions and Networks | revised | - | S3-131174 |
| S3-131061 | Checking that GBA types over Ua and Zn match | Ericsson | agreed | - | - |
| S3-131062 | User profile fetching before authentication in non3GPP access | Ericsson, Nokia Corporation, Nokia Solutions and Networks | revised | - | S3-131176 |
| S3-131063 | User profile fetching before authentication in WLAN interworking | Ericsson, Nokia Corporation, Nokia Solutions and Networks | revised | - | S3-131177 |
| S3-131064 | Deletion of Ks in ME in GBA_ME | Ericsson | revised | - | S3-131175 |
| S3-131065 | Evaluation of secure connection proposals | Ericsson | revised | - | S3-131159 |
| S3-131066 | DRAFT TS: Specification of the TUAK Algorithm Set Document 1: Algorithm Specification | Vodafone | revised | - | S3-131139 |
| S3-131067 | DRAFT TS: Specification of the TUAK Algorithm Set, Document 2: Implementers Test Data | Vodafone | revised | - | S3-131140 |
| S3-131068 | DRAFT TS: Specification of the TUAK Algorithm Set, Document 3: Design Conformance Test Data | Vodafone | revised | - | S3-131141 |
| S3-131069 | Solution of restricting the USIM to specific UEs | Nokia, NSN, Ericsson, ST-Ericsson, Samsung | revised | - | S3-131146 |
| S3-131070 | First outline version of 900 series TR describing the general SECAM scheme | NSN, Orange | revised | - | S3-131162 |
| S3-131071 | Draft reply to SA5 on 'necessary information for certificate enrolment procedures in MvPnP' | Nokia Corporation, NSN, Huawei, Hisilicon | revised | - | S3-131118 |
| S3-131072 | PWS security: notes on PWS signing proxies | Nokia Corporation, NSN | noted | - | - |
| S3-131073 | PWS security: The use of signing proxies | Nokia Corporation, NSN, TeliaSonera | revised | - | S3-131190 |
| S3-131074 | PWS security: Comparison of the use of signing proxies and implicit certificates | Nokia Corporation, NSN, TeliaSonera | revised | - | S3-131191 |
| S3-131075 | PWS security: review of solution 7 (generalised certificate-based approach) | Nokia Corporation, NSN | approved | - | - |
| S3-131076 | PWS security: proposed skeleton for clause 9 'Conclusions' | Nokia Corporation, NSN, Ericsson | noted | - | - |
| S3-131077 | PWS security: conclusions on Public Key Infrastructure | Nokia Corporation, NSN | noted | - | - |
| S3-131078 | PWS security: conclusions on Public Key Infrastructure | Nokia Corporation, NSN | noted | - | - |
| S3-131079 | PWS security: High-level conclusion on PWS root key distribution | Nokia Corporation, NSN | noted | - | - |
| S3-131080 | WLAN Proposal for clause 6 Framework for Analysis of Solutions.doc | BT Group | noted | - | - |
| S3-131081 | Security architecture and requirements for proximity based services | samsung | approved | - | - |
| S3-131082 | pCR TR33.895: GBA_U with user consent | Gemalto, Ericsson, Morpho Cards | approved | - | - |
| S3-131083 | security solution for configuration data transfer | Samsung | approved | - | - |
| S3-131084 | security solution for prose service | Samsung | noted | - | - |
| S3-131085 | Conclusion for USIM restriction to specific MTC Devices | Morpho Cards, Gemalto, Vodafone | revised | - | S3-131115 |
| S3-131086 | ProSe: new solution to address key issue 1 | Gemalto, Morpho Cards, Orange | revised | - | S3-131148 |
| S3-131087 | ProSe: new key Issue on key establishment | Gemalto, Morpho Cards | revised | - | S3-131142 |
| S3-131088 | ProSe: solution for key issue on key establishment | Gemalto, Morpho Cards | approved | - | - |
| S3-131089 | Proposed conclusion for restricting USIM to specific UEs | Qualcomm Incorporated, Alcatel-Lucent | noted | - | - |
| S3-131090 | Updating the proposed KME management procedures | Qualcomm Incorporated, Alcatel-Lucent | approved | - | - |
| S3-131091 | Propose pCR to add symmetric key based solution to the MTC TS | Qualcomm Incorporated, Alcatel-Lucent | noted | - | - |
| S3-131092 | Updates to solution 2 on protection for restricted discovery information | Qualcomm Incorporated | revised | - | S3-131151 |
| S3-131093 | [DRAFT] LS on timing synchronization | Qualcomm Incorporated | revised | - | S3-131152 |
| S3-131094 | Proposed key issue on one-to-many communication for ProSe | Qualcomm Incorporated | revised | - | S3-131137 |
| S3-131095 | Alignment and editorial changes to the ProSe TR | Qualcomm Incorporated | approved | - | - |
| S3-131096 | PCR_TR33.830Firewall traversal for IMS media plane using | Huawei | revised | - | S3-131154 |
| S3-131097 | comments on S3-130922 | China Mobile | withdrawn | - | - |
| S3-131098 | Incorrect reference for keep-alive mechanism | Ericsson | agreed | S3-130927 | - |
| S3-131099 | Updated reference for secure fax | Ericsson | agreed | S3-130928 | - |
| S3-131100 | comments on S3-130922 | China Mobile | revised | - | S3-131164 |
| S3-131101 | [SECAM] Comments on S3-13016 | China Mobile, Orange, NSN | revised | - | S3-131165 |
| S3-131102 | Addition of TLS profile for eMediaSec | Alcatel-Lucent | revised | S3-130951 | S3-131127 |
| S3-131103 | Comments on S3-130958, Work Item proposal: 3G Security: Specification of the TUAK algorithm set | Nokia, NSN | noted | - | - |
| S3-131104 | Comments on S3-131066 (attachment), TUAK algorithm set, Document 1: Algorithm Specification | Nokia, NSN | noted | - | - |
| S3-131105 | conditional pCR TS 33.187: USAT-based pairing for USIM restriction | Gemalto | noted | - | - |
| S3-131106 | Comment on S3-130965-GCSE_LTE security architecture | NSN | revised | - | S3-131121 |
| S3-131107 | Comments on S3-130986 | Orange | noted | - | - |
| S3-131108 | Comments on S3-130986 | Orange | withdrawn | - | - |
| S3-131109 | Information of spoof call in real networks | China Mobile | noted | - | - |
| S3-131110 | Conclusion of FS_SPOOF | China Mobile | revised | - | S3-131149 |
| S3-131111 | Information of spoof call in real networks | China Mobile | noted | - | - |
| S3-131112 | Comments on S3-130985 | NEC Corporation | noted | - | - |
| S3-131113 | SECAM practice in China Mobile | China Mobile | noted | - | - |
| S3-131114 | Reply LS on new work item on IMS support for RTP / RTCP transport multiplexing | S4-131309 | noted | - | - |
| S3-131115 | Conclusion for USIM restriction to specific MTC Devices | Morpho Cards, Gemalto, Vodafone | withdrawn | S3-131085 | - |
| S3-131116 | Draft LS reply security aspects of CMPv2 protocol used for certificate enrolment procedures in MvPnP | Ericsson | approved | S3-131005 | - |
| S3-131117 | Reply LS on security aspects of protocol architectures for small cell enhancements | SA3 | approved | - | - |
| S3-131118 | Draft reply to SA5 on 'necessary information for certificate enrolment procedures in MvPnP' | Nokia Corporation, NSN, Huawei, Hisilicon | approved | S3-131071 | - |
| S3-131119 | GCSE_LTE: Key issue on adding/removing group members | Home Office (UK) | approved | S3-130934 | - |
| S3-131120 | Merger of S3-130966 and S3-131052, GCSE Key issue – GC2 interface | Huawei, HiSilicon, NSN | approved | S3-130966 | - |
| S3-131121 | Merger contribution of S3-130965 and S3-131106, GCSE_LTE security architecture | Huawei, HiSilicon, NSN | approved | S3-130965, S3-131106 | - |
| S3-131122 | pCR 33.abc Overview | Ericsson | approved | S3-131026 | - |
| S3-131123 | pCR to TR 33.abc: [Study on Security for WebRTC IMS Client access to IMS] | Alcatel-Lucent | approved | S3-131013 | - |
| S3-131124 | pCR 33.abc Use of DTLS-SRTP for SRTP key management | Ericsson | approved | S3-131029 | - |
| S3-131125 | Draft TR WebRTC security | Rapporteur | approved | - | - |
| S3-131126 | Exception sheet for TR WebRTC security | Rapporteur | approved | - | - |
| S3-131127 | Addition of TLS profile for eMediaSec | Alcatel-Lucent | agreed | S3-131102 | - |
| S3-131128 | Reply to: LS on Group Communication Security in LTE | EADS | approved | - | - |
| S3-131129 | TR System Enablers for LTE | Rapporteur | approved | - | - |
| S3-131130 | Exception sheet for GCSE_LTE | Rapporteur | approved | - | - |
| S3-131131 | GCSE_LTE: Key issue on calling party authentication | Home Office (UK) | approved | S3-130996 | - |
| S3-131132 | pCR GSCE key issue - compromised UE | NSN | approved | S3-131049 | - |
| S3-131133 | Reply to: LS on new work item on 'IMS support for RTP / RTCP transport multiplexing | Unknown | withdrawn | - | - |
| S3-131134 | ProSe security using a combination of LTE security and an overlay | Ericsson | approved | S3-131012 | - |
| S3-131135 | Draft TR ProSe | Rapporteur | approved | - | - |
| S3-131136 | Exception sheet DraftTS ProSe | Rapporteur | approved | - | - |
| S3-131137 | ProSe: Key issues on Group Communications | Home Office (UK) | approved | S3-130933 | - |
| S3-131138 | key issue for ProSe communications in Group Owner mode | Huawei, HiSilicon | approved | S3-130972 | - |
| S3-131139 | DRAFT TS: Specification of the TUAK Algorithm Set Document 1: Algorithm Specification | Vodafone | revised | S3-131066 | S3-131196 |
| S3-131140 | DRAFT TS: Specification of the TUAK Algorithm Set, Document 2: Implementers Test Data | Vodafone | revised | S3-131067 | S3-131197 |
| S3-131141 | DRAFT TS: Specification of the TUAK Algorithm Set, Document 3: Design Conformance Test Data | Vodafone | revised | S3-131068 | S3-131198 |
| S3-131142 | ProSe: new key Issue on key establishment | Gemalto, Morpho Cards | approved | S3-131087 | - |
| S3-131143 | Security for PrSe UE communication in group owner mode | Huawei, HiSilicon | approved | S3-130973 | - |
| S3-131144 | S2MTC | Unknown | withdrawn | - | - |
| S3-131145 | Proposed changes on Key Issue #1 | Telecom Italia | approved | S3-130908 | - |
| S3-131146 | Solution of restricting the USIM to specific UEs | Nokia, NSN, Ericsson, ST-Ericsson, Samsung | noted | S3-131069 | - |
| S3-131147 | Security for ProSe communication through UE-to-Network relay with network authorization | Huawei, HiSilicon | approved | S3-130974 | - |
| S3-131148 | ProSe: new solution to address key issue 1 | Gemalto, Morpho Cards, Orange | approved | S3-131086 | - |
| S3-131149 | Conclusion of FS_SPOOF | China Mobile | approved | S3-131110 | - |
| S3-131150 | [ProSe] Additional ProSe key issue | Intel | approved | S3-131030 | - |
| S3-131151 | Updates to solution 2 on protection for restricted discovery information | Qualcomm Incorporated | approved | S3-131092 | - |
| S3-131152 | LS on timing synchronization | Qualcomm Incorporated | approved | S3-131093 | - |
| S3-131153 | Adding a new Key Issue on Ensuring a trusted and reliable accounting | Telecom Italia | approved | S3-130911 | - |
| S3-131154 | PCR_TR33.830Firewall traversal for IMS media plane using | Huawei | approved | S3-131096 | - |
| S3-131155 | Reply to: LS on interactions of oneM2M with Underlying Networks | Ericsson | approved | - | - |
| S3-131156 | Draft TR 33.868 | Rapporteur | approved | - | - |
| S3-131157 | Draft TS 33.187 | Rapporteur | approved | - | - |
| S3-131158 | GBA for constrained devices | Ericsson | approved | S3-131056 | - |
| S3-131159 | Evaluation of secure connection proposals | Ericsson,China Mobile | approved | S3-131065 | - |
| S3-131160 | Solution for Secure Connection in draft TS 33.187 | Ericsson | approved | S3-131055 | - |
| S3-131161 | [SECAM] Normative WID for SECAM | Orange, NTT DOCOMO, Deutsche Telekom, Telecom Italia, BT, China Mobile, China Unicom, TeliaSonera, Vodafone, NSN, Juniper Networks, NEC, Huawei, Ericsson | approved | S3-130918 | - |
| S3-131162 | First outline version of 900 series TR describing the general SECAM scheme | NSN, Orange | approved | S3-131070 | - |
| S3-131163 | [SECAM] LS to GSMA SG | Orange | approved | S3-130926 | - |
| S3-131164 | [SECAM] Conclusion section - comparison table M2/CC | Orange, NSN, Telecom Italia, Huawei, Hisilicon, China Mobile | approved | S3-130922 | - |
| S3-131165 | [SECAM] Information needed to execute the required tests for the Basic Vulnerability Testing | China Mobile,Orange,NSN | approved | S3-131016 | - |
| S3-131166 | Outcome of evening session. | Rapperteur | noted | - | - |
| S3-131167 | Development requirements | Orange | approved | - | - |
| S3-131168 | Draft TR on SECAM | Rapporteur | approved | - | - |
| S3-131169 | Reply LS on FQDN formats used in MvPnC procedures | C4 | noted | - | - |
| S3-131170 | LS on FQDNs to support Multi-vendor Plug and Play (MvPnP) feature | CT4 | noted | - | - |
| S3-131171 | CR to TR 33.980 Correction of reference | Nokia Corporation, NSN | agreed | S3-130919 | - |
| S3-131172 | CR to TS 33.221 Correction of Reference | Nokia Corporation, NSN | agreed | S3-130920 | - |
| S3-131173 | Clarification of NAF key negotiation on Ua | Ericsson, Nokia Corporation, Nokia Solutions and Networks | agreed | S3-131059 | - |
| S3-131174 | Clarification of NAF key negotiation on Ua | Ericsson, Nokia Corporation, Nokia Solutions and Networks | agreed | S3-131060 | - |
| S3-131175 | Deletion of Ks in ME in GBA_ME | Ericsson | agreed | S3-131064 | - |
| S3-131176 | User profile fetching before authentication in non3GPP access | Ericsson, Nokia Corporation, Nokia Solutions and Networks | agreed | S3-131062 | - |
| S3-131177 | User profile fetching before authentication in WLAN interworking | Ericsson, Nokia Corporation, Nokia Solutions and Networks | agreed | S3-131063 | - |
| S3-131178 | [SECAM] coversheet for presentation of TR 33.805 to SA | Orange, Telecom Italia | approved | S3-130925 | - |
| S3-131179 | Draft TR 33.865; Security aspects of WLAN network selection for 3GPP terminals | Rapporteur | approved | - | - |
| S3-131180 | Exception sheet for WLAN Network Selection for 3GPP Terminals | Rapporteur | approved | - | - |
| S3-131181 | Draft TR33.830 Firewall traversal | Rapporteur | approved | - | - |
| S3-131182 | Work Item proposal: 3G Security: Specification of the TUAK algorithm set | AT&T, BlackBerry, China Mobile, Ericsson, Deutsche Telekom, Morpho Cards, NTT DOCOMO, Orange, Vodafone | approved | S3-130958 | - |
| S3-131186 | Exception sheet for TS: Specification of the TUAK | SA3 | approved | - | - |
| S3-131187 | Reply LS on DASH Authentication | Intel, Huawei | approved | S3-130987 | - |
| S3-131188 | Draft TR on PWS | Rapporteur | Approved | - | - |
| S3-131189 | Restrictions on signature length in PWS security | Ericsson, Nokia Corporation, NSN | noted | S3-130932 | - |
| S3-131190 | PWS security: The use of signing proxies | Nokia Corporation, NSN, TeliaSonera | approved | S3-131073 | - |
| S3-131191 | PWS security: Comparison of the use of signing proxies and implicit certificates | Nokia Corporation, NSN, TeliaSonera | withdrawn | S3-131074 | - |
| S3-131192 | WebRTC TR 33.abc skeleton | Ericsson | approved | S3-130982 | - |
| S3-131193 | (Draft) LS on WebRTC Security Responsibility | Ericsson | approved | S3-130981 | - |
| S3-131194 | Adding General Requirements on ProSe Security, Authorization and Privacy | Telecom Italia | approved | S3-130906 | - |
| S3-131195 | Updated WID for ProSE | Rapporteur | approved | - | - |
| S3-131196 | DRAFT TS: Specification of the TUAK Algorithm Set Document 1: Algorithm Specification | Vodafone | revised | S3-131139 | S3-131203 |
| S3-131197 | DRAFT TS: Specification of the TUAK Algorithm Set, Document 2: Implementers Test Data | Vodafone | agreed | S3-131140 | - |
| S3-131198 | DRAFT TS: Specification of the TUAK Algorithm Set, Document 3: Design Conformance Test Data | Vodafone | agreed | S3-131141 | - |
| S3-131199 | Exception sheet for MTC | Rapporteur | approved | - | - |
| S3-131200 | Draft TR SSO study | Rapporteur | approved | - | - |
| S3-131201 | Draft TR spoofed call study | Rapporteur | approved | - | - |
| S3-131202 | Draft TR on privacy (study) | Rapporteur | approved | - | - |
| S3-131203 | DRAFT TS: Specification of the TUAK Algorithm Set Document 1: Algorithm Specification | Vodafone | approved | S3-131196 | - |
| S3-131204 | PCR to 33.895: Section 8.4.2.2 generalization of local authentication | Interdigital | approved | S3-130937 | - |
| S3-131205 | PCR to 33.895 - Sections 3 and 8.3.X | Interdigital | approved | S3-130939 | - |
| S3-131206 | Detail the description of partly ciphering SDT solution | Huawei, HiSilicon | approved | S3-130969 | - |
| S3-131207 | Considerations on the MTC-IWF based SDT security solution | Huawei, HiSilicon | approved | S3-130970 | - |
| S3-131208 | Evaluation on Security of Small Data Transmission | China Mobile | approved | S3-130985 | - |
| S3-131209 | Deletion of editor notes | Huawei, HiSilicon | approved | S3-130968 | - |
| S3-131210 | MTC-IWF based solution editors notes | NEC Corporation | approved | S3-130997 | - |
| S3-131211 | Skeleton of SPI TR | China Unicom | approved | S3-131033 | - |
| S3-131212 | Privacy study purpose | Ericsson, China Unicom | Approved | S3-131003 | - |

## Annex B: List of change requests

| Document | Title | Source | Spec | CR | Rev | Rel | Cat | WI | Decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S3-130988 | clarification on mandatory requirements | Huawei, Hisilicon, China Unicom | - | - | - | - | - | - | withdrawn |
| S3-131021 | comments on S3-130922 | Huawei, Hisilicon | - | - | - | - | - | - | revised |
| S3-131024 | Proposed the solution of media over TCP based on ICE for TR 33.830 | Huawei, Hisilicon | - | - | - | - | - | - | withdrawn |
| S3-131032 | clarification on mandatory requirements | Huawei, Hisilicon, China Unicom | - | - | - | - | - | - | approved |
| S3-131102 | Addition of TLS profile for eMediaSec | Alcatel-Lucent | 33.328 | 0057 | 1 | Rel-12 | B | eMediaSec | revised |
| S3-130927 | Incorrect reference for keep-alive mechanism | Ericsson | 33.203 | 0204 | - | - | - | - | revised |
| S3-131098 | Incorrect reference for keep-alive mechanism | Ericsson | 33.203 | 0204 | 1 | Rel-12 | F | TURAN | agreed |
| S3-131061 | Checking that GBA types over Ua and Zn match | Ericsson | 33.220 | 0178 | - | Rel-12 | F | TEI12, GBAExt | agreed |
| S3-130920 | CR to TS 33.221 Correction of Reference | Nokia Corporation, NSN | 33.221 | 0012 | - | - | - | - | revised |
| S3-131172 | CR to TS 33.221 Correction of Reference | Nokia Corporation, NSN | 33.221 | 0012 | 1 | Rel-11 | F | SEC1-SC | agreed |
| S3-131059 | Clarification of NAF key negotiation on Ua | Ericsson, Nokia Corporation, Nokia Solutions and Networks | 33.222 | 0050 | - | - | - | - | revised |
| S3-131173 | Clarification of NAF key negotiation on Ua | Ericsson, Nokia Corporation, Nokia Solutions and Networks | 33.222 | 0050 | 1 | Rel-11 | F | TEI11, GBA-ext | agreed |
| S3-131060 | Clarification of NAF key negotiation on Ua | Ericsson, Nokia Corporation, Nokia Solutions and Networks | 33.222 | 0051 | - | - | - | - | revised |
| S3-131174 | Clarification of NAF key negotiation on Ua | Ericsson, Nokia Corporation, Nokia Solutions and Networks | 33.222 | 0051 | 1 | Rel-12 | A | TEI12, GBA-ext | agreed |
| S3-131064 | Deletion of Ks in ME in GBA_ME | Ericsson | 33.223 | 0023 | - | - | - | - | revised |
| S3-131175 | Deletion of Ks in ME in GBA_ME | Ericsson | 33.223 | 0023 | 1 | Rel-12 | F | TEI12,GBAPush | agreed |
| S3-131063 | User profile fetching before authentication in WLAN interworking | Ericsson, Nokia Corporation, Nokia Solutions and Networks | 33.234 | 0122 | - | - | - | - | revised |
| S3-131177 | User profile fetching before authentication in WLAN interworking | Ericsson, Nokia Corporation, Nokia Solutions and Networks | 33.234 | 0122 | 1 | Rel-12 | F | TEI12, WLAN | agreed |
| S3-130928 | Updated reference for secure fax | Ericsson | 33.328 | 0056 | - | - | - | - | revised |
| S3-131099 | Updated reference for secure fax | Ericsson | 33.328 | 0056 | 1 | Rel-12 | F | eMEDIASEC | agreed |
| S3-130951 | Addition of TLS profile for eMediaSec | Alcatel-Lucent | 33.328 | 0057 | - | Rel-12 | B | eMEDIASEC | revised |
| S3-131127 | Addition of TLS profile for eMediaSec | Alcatel-Lucent | 33.328 | 0057 | 2 | Rel-12 | B | eMEDIASEC | agreed |
| S3-131018 | Correction of a typo | Nokia Corporation, NSN, NEC Corporation | 33.401 | 0523 | - | Rel-12 | F | TEI12,SAES | agreed |
| S3-131176 | User profile fetching before authentication in non3GPP access | Ericsson, Nokia Corporation, Nokia Solutions and Networks | 33.402 | 0115 | 1 | Rel-12 | F | TEI12, SAES | agreed |
| S3-131062 | User profile fetching before authentication in non3GPP access | Ericsson, Nokia Corporation, Nokia Solutions and Networks | 33.402 | 0115 | 2 | - | - | - | revised |
| S3-130921 | CR to TR 33.823 Update of W3C references | Nokia Corporation, NSN | 33.823 | 0004 | - | Rel-12 | F | Web_GBA | agreed |
| S3-130919 | CR to TR 33.980 Correction of reference | Nokia Corporation, NSN | 33.980 | 0023 | - | - | - | - | revised |
| S3-131171 | CR to TR 33.980 Correction of reference | Nokia Corporation, NSN | 33.980 | 0023 | 1 | Rel-11 | F | LibSec | agreed |

## Annex C: Lists of liaisons

### C1: Incoming liaison statements

| Document | Original | Title | From | Decision | Reply in |
| --- | --- | --- | --- | --- | --- |
| S3-130935 |  | LS on TLS protocol profile for eMediasec | C4-131868 | noted |  |
| S3-130938 |  | LS on SIP/SDP Usage for TCP/TLS session establishment | C4-131508 | noted |  |
| S3-130940 | C4-131546 | LS on new work item on 'IMS support for RTP / RTCP transport multiplexing | C4-131546 | replied to | S3-130948 |
| S3-130940 | C4-131546 | LS on new work item on 'IMS support for RTP / RTCP transport multiplexing | C4-131546 | replied to | S3-131133 |
| S3-130941 |  | LS/o on draft Recommendation ITU-T X.sap-8, Efficient multi-factor authentication mechanisms using mobile devices [to ISO/IEC JTC1 SC27/WG2 and 3GPP SA3] | ITU-T Study Group 17 | noted |  |
| S3-130942 |  | LS on interactions of oneM2M with Underlying Networks | OneM2M | noted | S3-131155 |
| S3-130943 |  | LS on security aspects of protocol architectures for small cell enhancements | R2-133018 | noted |  |
| S3-130944 |  | Reply LS on security aspects of protocol architectures for small cell enhancements | R2-133650 | noted |  |
| S3-130945 |  | Reply LS on security aspects of protocol architectures for small cell enhancements | R3-131919 | noted |  |
| S3-130946 |  | Answer LS to oneM2M on interactions of oneM2M with Underlying Networks | S1-134175 | noted |  |
| S3-130947 |  | Wi-Fi Roaming Task Force Signalling Optimisation (forwarded by SA2) | GSMA Wi-Fi Roaming Task Force | noted |  |
| S3-130948 |  | Reply LS to CT WG4 LS on new work item on 'IMS support for RTP / RTCP transport multiplexing | S2-133825 | noted |  |
| S3-130949 |  | LS on DASH Authentication | S4-130867 | replied to |  |
| S3-130950 | S5-131458 | LS to SA3 on security aspects of CMPv2 protocol used for certificate enrolment procedures in MvPnP | S5-131458 | noted |  |
| S3-130952 | S5-131789 | LS to SA3 on necessary information for certificate enrolment procedures in MvPnP | S5-131789 | replied-to | S3-131118 |
| S3-130952 | S5-131789 | LS to SA3 on necessary information for certificate enrolment procedures in MvPnP | S5-131789 | replied-to | S3-131071 |
| S3-130953 |  | LS Announcing TUAK, an alternative to Milenage | SAGE (13) 08 | noted |  |
| S3-130954 |  | LS on Full specification set for TUAK, an alternative to Milenage | SAGE (13) 09 | noted |  |
| S3-130955 |  | LS on Group Communication Security in LTE | ETSI TC TCCE | replied to | S3-131128 |
| S3-131048 |  | LS on Security Assurance Methodology for 3GPP Network Products | GSMA Security Group | noted |  |
| S3-131114 | S4-131309 | Reply LS on new work item on IMS support for RTP / RTCP transport multiplexing | S4-131309 | noted |  |
| S3-131169 |  | Reply LS on FQDN formats used in MvPnC procedures | C4 | noted |  |
| S3-131170 |  | LS on FQDNs to support Multi-vendor Plug and Play (MvPnP) feature | CT4 | noted |  |

### C2: Outgoing liaison statements

| Document | Title | To | Cc | reply to i/c LS |
| --- | --- | --- | --- | --- |
| S3-131116 | Draft LS reply security aspects of CMPv2 protocol used for certificate enrolment procedures in MvPnP | SA5 | - | S5-131458 |
| S3-131117 | Reply LS on security aspects of protocol architectures for small cell enhancements | R2 | RAN3, CT1, SA2 | R2-133018, R2-133650 |
| S3-131118 | Draft reply to SA5 on 'necessary information for certificate enrolment procedures in MvPnP' | SA5 | - | S3-130952 |
| S3-131128 | Reply to: LS on Group Communication Security in LTE | ETSI TC TCCE | - | S3-130955 |
| S3-131152 | LS on timing synchronization | RAN1 | SA2, RAN2 | - |
| S3-131155 | Reply to: LS on interactions of oneM2M with Underlying Networks | OneM2M | SA1, SA2 | S3-130942 |
| S3-131163 | [SECAM] LS to GSMA SG | GSMA SG | - | - |
| S3-131187 | Reply LS on DASH Authentication | SA4 | - | - |
| S3-131193 | (Draft) LS on WebRTC Security Responsibility | SA2 | SA1 | - |

## Annex D: List of agreed/approved new and revised Work Items

| Document | Title | Source | new/revised |
| --- | --- | --- | --- |
| S3-131182 | Work Item proposal: 3G Security: Specification of the TUAK algorithm set | AT&T, BlackBerry, China Mobile, Ericsson, Deutsche Telekom, Morpho Cards, NTT DOCOMO, Orange, Vodafone | new WID |
| S3-131195 | Updated WID for ProSE | Rapporteur | revised WID |

## Annex E: List of participants

| Name | Representing | Status-Partner |
| --- | --- | --- |
| Aldén, Magnus (Dr.) | TeliaSonera AB | 3GPPMEMBER (ETSI) |
| Barry, Aguibou (Mr.) | VODAFONE Group Plc | 3GPPMEMBER (ETSI) |
| Blanchard, Colin (Mr.) | BT Group Plc | 3GPPMEMBER (ETSI) |
| Brusilovsky, Alec (Mr.) | INTERDIGITAL COMMUNICATIONS | 3GPPMEMBER (ETSI) |
| Buckley, Eoin (Dr.) | BlackBerry UK Limited | 3GPPMEMBER (ETSI) |
| Calvez, Christophe (Mr.) | EADS | 3GPPMEMBER (ETSI) |
| Castagno, Mauro (Mr.) | TELECOM ITALIA S.p.A. | 3GPPMEMBER (ETSI) |
| Chen, Jing (Mr.) | Huawei Technologies France | 3GPPMEMBER (ETSI) |
| Dolly, Martin (Mr.) | AT&T | 3GPPMEMBER (ATIS) |
| Dong, Olivier (Mr.) | NEC Technologies (UK) LTD | 3GPPMEMBER (ETSI) |
| Ejzak, Richard (Mr.) | Alcatel-Lucent | 3GPPMEMBER (ATIS) |
| Escott, Adrian (Dr.) | Qualcomm Japan Inc | 3GPPMEMBER (ARIB) |
| Frankel, Sheila (Mrs.) | U.S. Department of Commerce | 3GPPMEMBER (ATIS) |
| Fu, Xiao (Mr.) | Alcatel-Lucent Shanghai Bell | 3GPPMEMBER (CCSA) |
| Gamishev, Todor (Mr.) | ORANGE | 3GPPMEMBER (ETSI) |
| Habermacher, Loic (Mr.) | Orange Spain | 3GPPMEMBER (ETSI) |
| Haigh, Peter (Mr.) | CESG | 3GPPMEMBER (ETSI) |
| Hickey, John (Mr.) | Alcatel-Lucent Telecom Ltd | 3GPPMEMBER (ETSI) |
| Holtmanns, Silke (Dr.) | NOKIA UK Ltd | 3GPPMEMBER (ETSI) |
| Horn, Guenther (Dr.) | NSN Japan | 3GPPMEMBER (ARIB) |
| Jerichow, Anja (Miss) | NSN Oy | 3GPPMEMBER (ETSI) |
| Kohalmi, Steve (Mr.) | Juniper Networks | 3GPPMEMBER (ETSI) |
| Kraemer, Isabelle (Miss) | ORANGE | 3GPPMEMBER (ETSI) |
| Krainioukov, Oleg (Mr.) | Morpho Cards GmbH | 3GPPMEMBER (ETSI) |
| Leadbeater, Alex (Mr.) | BT Group Plc | 3GPPMEMBER (ETSI) |
| Lee, Joseph (Mr.) | Oracle Corporation | 3GPPMEMBER (ETSI) |
| Lehtovirta, Vesa (Mr.) | Ericsson Inc. | 3GPPMEMBER (ATIS) |
| Li, Yang (Mr.) | ZTE Corporation | 3GPPMEMBER (ETSI) |
| Li, Zhiwei (Dr.) | HuaWei Technologies Co., Ltd | 3GPPMEMBER (CCSA) |
| Luft, Achim (Mr.) | Intel | 3GPPMEMBER (ATIS) |
| Maeder, Andreas (Dr.) | NEC Telecom MODUS Ltd. | 3GPPMEMBER (ETSI) |
| Marchetto, Luisa (Miss) | AT&T GNS Belgium SPRL | 3GPPMEMBER (ETSI) |
| Miyamoto, Yoshihiro (Mr.) | NEC Corporation | 3GPPMEMBER (ARIB) |
| Mizikovsky, Semyon (Mr.) | Alcatel-Lucent Telecom Ltd | 3GPPMEMBER (ETSI) |
| Nair, Suresh (Mr.) | Alcatel-Lucent Deutschland AG | 3GPPMEMBER (ETSI) |
| Nishida, Katsutoshi (Mr.) | NTT DOCOMO INC. | 3GPPMEMBER (TTC) |
| Norp, Toon (Mr.) | KPN N.V. | 3GPPMEMBER (ETSI) |
| Norrman, Karl (Mr.) | Ericsson Japan K.K. | 3GPPMEMBER (ARIB) |
| Ohlsson, Oscar (Mr.) | Nanjing Ericsson Panda Com Ltd | 3GPPMEMBER (CCSA) |
| Palanigounder, Anand (Mr.) | Qualcomm Incorporated | 3GPPMEMBER (ATIS) |
| Pauliac, Mireille (Mrs.) | Gemalto N.V. | 3GPPMEMBER (ETSI) |
| Prasad, Anand (Dr.) | NEC Corporation | 3GPPMEMBER (ETSI) |
| Qi, Yuan (Ms.) | CATR | 3GPPMEMBER (CCSA) |
| Rajadurai, Rajavelsamy (Mr.) | SAMSUNG Electronics Co., Ltd. | 3GPPMEMBER (ARIB) |
| Sahlin, Bengt (Mr.) | Ericsson | 3GPPMEMBER (ETSI) |
| Schroeder, Stefan (Mr.) | Deutsche Telekom AG | 3GPPMEMBER (ETSI) |
| Schumacher, Greg (Mr.) | Sprint Nextel Corporation | 3GPPMEMBER (ATIS) |
| Suh, Kyungjoo Grace (Ms.) | Samsung R&D Institute UK | 3GPPMEMBER (ETSI) |
| Tamura, Toshiyuki (Mr.) | NEC Corporation | 3GPPMEMBER (TTC) |
| Tanaka, Itsuma (Mr.) | DOCOMO Communications Lab. | 3GPPMEMBER (ETSI) |
| Towe, Christopher (Mr.) | SouthernLINC Wireless | 3GPPMEMBER (ATIS) |
| Vanderveen, Michaela (Dr.) | QUALCOMM UK Ltd | 3GPPMEMBER (ETSI) |
| Wang, Xiaobo (Dr.) | HUAWEI TECHNOLOGIES Co. Ltd. | 3GPPMEMBER (ETSI) |
| Wang, Xin (Dr.) | HuaWei Technologies Co., Ltd | 3GPPWITHDRAW (ATIS) |
| Whorlow, Colin (Mr.) | CESG | 3GPPMEMBER (ETSI) |
| Williams, Scott (Mr.) | OTD | 3GPPMEMBER (ETSI) |
| Wong, Marcus (Mr.) | HUAWEI TECHNOLOGIES Co. Ltd. | 3GPPMEMBER (ETSI) |
| Xu, Lydia (Ms.) | Huawei Technologies (UK) | 3GPPMEMBER (ETSI) |
| Yip, Yew Seng (Mr.) | Motorola Solutions Danmark A/S | 3GPPMEMBER (ETSI) |
| Yu, Bo (Mr.) | China Unicom | 3GPPMEMBER (CCSA) |
| Zhang, Dajiang (Mr.) | NOKIA Corporation | 3GPPMEMBER (ETSI) |
| Zhang, Gaoshan (Mr.) | China Mobile Com. Corporation | 3GPPMEMBER (CCSA) |
| ZHANG, Xiaowei (Dr.) | NEC EUROPE LTD | 3GPPMEMBER (ETSI) |
| Zhu, Judy (Mrs.) | China Mobile Com. Corporation | 3GPPMEMBER (CCSA) |
| Zhu, Li (Dr.) | ZTE Corporation | 3GPPMEMBER (CCSA) |
| Zugenmaier, Alf (Dr.) | DOCOMO Communications Lab. | 3GPPMEMBER (ETSI) |

## Annex F: List of future meetings

| Title | Start date | End date (OP) | Town | Country | Reference |
| --- | --- | --- | --- | --- | --- |
| 3GPPSA3#74 | 20/01/2014 09:00:00 | 24/01/2014 17:30:00 | Taipei | TW | S3-74 |
| 3GPPSA3#52-LI | 04/02/2014 09:00:00 | 06/02/2014 17:30:00 | Sophia Antipolis | FR | S3-ah-31149 |
| 3GPPSA3#53-LI | 29/04/2014 09:00:00 | 01/05/2014 17:30:00 | US | US | S3-ah-31150 |
| 3GPPSA3#75 | 12/05/2014 09:00:00 | 16/05/2014 17:30:00 | Sapporo | JP | S3-75 |
| 3GPPSA3#54-LI | 22/07/2014 09:00:00 | 24/07/2014 17:30:00 | Sophia Antipolis | FR | S3-ah-31151 |
| 3GPPSA3#76 | 25/08/2014 09:00:00 | 29/08/2014 17:30:00 | Sophia Antipolis | FR | S3-76 |
| 3GPPSA3#55-LI | 28/10/2014 09:00:00 | 30/10/2014 17:30:00 | US | US | S3-ah-31152 |