3GPP TSG SA WG3 (Security) Meeting #75	S3-140912

12 May – 16 May, 2014;Sapporo (Japan)		revision of S3-14abcd

Source:	 Gemalto

Title:	Comments to S3-140803: WebRTC security:- pCR to TR 33.871

Document for:				Approval

Agenda Item:			  7.1.2 WebRTC security

Work Item / Release:	IMS_WebRTC / Rel-12

Abstract of the contribution: This pCR enables the reuse of IMS AKA for WebRTC IMS Client (WIC) authentication when WIC obtains IMS credentials from a network based entity that is operator owned or from a trusted 3rd party entity.

### Introduction

It is possible to have an architecture where WebRTC IMS Client (WIC), running on a device that does not have a UICC with ISIM or cannot access UICC on the device, is still able to use IMS-AKA to authenticate and register with the IMS core. This is possible by having a network based entity that functions like an ISIM, and is accessible to the WIC through secure means.

[Gemalto]

- The solution described below introduces a new concept consisting in storing the credentials of a 3GPP subscriber within a Server Function instead of being stored in the UE. This concept is totally new. SA1 has never identified the need for such a concept. This kind of concept should come from SA1 need and request.

- The WWSF storing the network-based ISIM, with IMS AKA credentials of the 3GPP subscriber, may not be in the operator domain.

- The credentials of 3GPP subscribers used to perform AKA authentication shall always be stored in the UICC.  This is a 3GPP security requirement.

- “ISIM” is a term dedicated to the UICC application as stated in the definition of ISIM (confer definition of ISIM  in 3GPP TS 33.203).

  - Extract of 3GPP TS 33.203: ISIM – IM Subscriber Identity Module: For the purposes of the present document the ISIM is a term that indicates the collection of IMS security data and functions on a UICC. The ISIM may be a distinct application on the UICC

In the scope of the definition of Common IMS within 3GPP, several proposals were made to use the term “ISIM” for collection of IMS security data and functions not stored in the UICC. All the proposals were rejected by 3GPP and it was clarified that the term “ISIM” was dedicated to collection of IMS security and functions stored on the UICC.

Consequently, Gemalto opposes the proposal to define a solution relying on network-based ISIM to perform IMS AKA authentication, as described in this contribution.

*** BEGIN CHANGES ***

*********************************************************

### 6.1.1	Authentication of WebRTC IMS Client re-using existing IMS authentication mechanisms

#### 6.1.1.1	General

Editor’s Note: It is assumed that the WebRTC IMS Client has access to IMS credentials and uses these to authenticate to the IMS.

In this scenario it is assumed that the user has a subscription with an individual IMPU and uses an IMS authentication mechanism (e.g., IMS digest) to authenticate with IMS. The eP-CSCF is assumed to relay the authentication information so that the message flows are unchanged.

Editor's Note: Access to the (U)SIM and the AKA algorithm from JavaScript is currently not supported in today's browsers (without requiring browser modifications or installation of proprietary plugins). It is ffs to use IMS AKA.

#### 6.1.1.2	Use of SIP Digest credentials

In this scenario that the WebRTC IMS Client implements the SIP Digest algorithm and sends the authentication information to the eP-CSCF. The use of SIP Digest in IMS is specified in Annex N of TS 33.203 [5].

Figure 6.1.1.2-1 shows the registration flow. In this figure SIP over secure WebSocket is used between the WebRTC IMS Client and the eP-CSCF. Other protocols (e.g. HTTP RESTful or JSON over WebSocket) can also be used as long as it is able to relay the IMPI and the digest challenge, challenge-response, and auth-info values.

It is recommended to maintain a clear separation between WebRTC IMS Clients and regular IMS UEs. A user accessing IMS from a WebRTC IMS Client should be assigned a separate subscription in the HSS with a unique IMPI and SIP Digest password.  In this way a compromised password will have an isolated impact and only affect the WebRTC IMS Client.

The solution requires that the IMPI and SIP Digest password are made available to the JavaScript in the WebRTC IMS Client.

The entities that have access to the IMPI and SIP Digest password, and thus needs to be trusted by the operator, are the user, the browser, the WWSF, and the IMS core network.  SIP Ddigest is therefore only intended to be used when the WWSF is controlled by the operator or a 3rd party trusted by the operator.

NOTE 1:	It is assumed that the credentials are entered by the user via the web GUI or retrieved from the WWSF over HTTPS. Note that the latter option requires that WWSF has authenticated the user previously.

NOTE 2:	The use of SIP Digest breaks the 3GPP security requirement mandating IMS AKA to connect to IMS when using a 3GPP access network, see 3GPP TS 33.203.

Figure 6.1.1.2-1: WebRTC client authentication using SIP Digest

NOTE 3:	The eP-CSCF can verify that the web-page establishing the signalling connection comes from a trusted domain by inspecting the value of Origin header. This header is inserted by the browser in the WebSocket handshake and in every HTTP request (requires the use of CORS, http://www.w3.org/TR/cors/). The protection mechanism works under the assumption that the browser is not under the attacker's control, which means that the contents of the Origin header can be trusted.

#### 6.1.1.3	IMS AKA based authentication using a network based ISIM

In this scenario the WIC is on a device that does not have aUICC with ISIM or is using a browser that cannot access USIM/ISIM, The user authentication using IMS AKA is achieved with the help of a network based ISIM. The network based ISIM can either be co-located with the WWSF or be securely accessed by the WWSF.

NOTE 1: It is assumed that network based ISIM is securely maintainted either by the operator or by the operator trusted 3rd party.

NOTE 2: It is assumed that user is authenticatesd successfully to the WWSF before the network based ISIM is utilized for IMS-AKA authentication of IMS subscription associated with the IMS credentials assigned to the WIC.

NOTE 3: The mechanism by which the user is authenticated is out of scope for this solution. For the purpose of this solution it is assumed that each user has a unique identifier (for eg. MSISDN) assigned by the network.

NOTE 4: The network based ISIM maintains the mapping between the user identifier and the IMS subscription of the user. The mechanism by which this mapping is established and maintained is out of scope of this solution.

The following call flow is an example of how IMS AKA based authentication can be achieved when network based ISIM  is collocated with the WWSF

Figure 6.1.1.3-1: WebRTC client authentication using IMS AKA

*******************************************************

*** END CHANGES ***