3GPP TSG SA WG3 (Security) Meeting #79bis	S3-151604

Sophia Antipolis, France, 29 – 30 June 2015

Source:	Telecom Italia

Title:	Resolving Editor’s Note in B.3.3.4.4.1 “MME Management and Maintenance interfaces”

Document for:	Discussion/approval

Agenda Item:	4

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to resolve the Editor’s Note in clause B.3.3.4.4.1 MME Management and Maintenance interfaces.

# Discussion

This contribution proposes to resolve the Editor notes in clause  “B.3.3.4.4.1 MME Management and Maintenance interfaces ” of TR 33.806 v.0.8.0.

There is an Editor note:

Editor's Note: These management entities need to be further defined.

Looking to the Editor Notes, Management Entity is an entity which provides management capabilities and that enables  to operate on the network product. Management Entities can be:

- Client workstation used for the remote management of the network product

- Client workstation used for local management of the network product

Anyway the requirement shall be focused on the network product and not on the Management Entities. Consequently the mutual authentication shall be supported by the protocol enabled in the network product and used for its management.

So this contribution proposes to remove the Editor Notes in B.3.3.4.4.1 and in addition to rephrase the requirement to underline its applicability to the protocols used for the network product management and not to the management entities. The protocols used for the network product management can be standard protocols that address this requirement but can be proprietary protocols that can provide encryption and integrity capabilities but not mutual authentication.

# Concrete proposal

***	FIRST CHANGE	***

##### B.3.3.4.4	Specific Authentication use cases

B.3.3.4.4.1 MME Management and Maintenance interfaces

Requirement Name: MME Management and Maintenance interfaces

Requirement Reference: to be done later

Requirement Description: The protocols used for the network product management shall support  mutual authentication mechanisms.

Threat References: TBA

Security Objective references: SECURE MME ADMINISTRATION.

Test case:TBA

***	END CHANGE	***