3GPP TSG SA WG3 (Security) Meeting #86	S3-160157

6-10 February 2017, Sophia Antipolis (France)

Source:	NEC

Title:	pCR to TR 33.899: Update solution #1.8 on key hierarchy for NextGen

Document for:	Approval

Agenda Item:	8.4.1

Work Item / Release:	FS_NSA / Rel-14

Abstract: This pCR updates solution #1.8 on key hierarchy in security area “Architectural aspects of NextGen security“

#### Introduction

There has been consolidated network architecture defined by SA2 in TS 23.501 v0.1.1. This pCR updates the key hierarchy to fit the architecture. The proposed key hierarchy supports (1) common C-plane keys for MM (Mobile Management) messages, (2) slice-specific keys for SM (Session Management) messages and U-plane data, (3) keys for other AN (Access Network) than 3GPP and (4) legacy keys for existing 3GPP radio network such as E-UTRAN and non-3GPP radio network.

This pCR also resolves the following three editor’s notes (ENs). The first EN is resolved by introduction of slice-specific keys for SM. The second EN is resolved by adding subsection 5.1.4.8.2.3 where key derivation is described. The third EN is removed, because the proposed key hierarchy has been updated in consideration of the consolidated architecture in SA2.

Editor’s Note: It is FFS to consider how to realize security isolation among network slices for single C-plane in core network.

Editor's note: The details on each key derivation (e.g., key derivation algorithm, input parameters) are FFS.

Editor’s Note:  The required levels of key hierarchy will be studied during the evaluation based on other working group progress

#### Proposal

************* Begin of Change ***********

#### 5.1.4.8	Solution #1.8: Key hierarchy for NextGen

##### 5.1.4.8.1	Introduction

This solution addresses the key issue “#1.7 key hierarchy” and proposes a key hierarchy for NextGen network in accordance with reference architecture defined in TS23.501.

The proposed key hierarchy supports (1) common C-plane keys for MM (Mobile Management) messages, (2) slice-specific keys for SM (Session Management) messages and U-plane data, (3) keys for other AN (Access Network) than 3GPP and (4) legacy keys for existing 3GPP radio network such as E-UTRAN and non-3GPP radio network.

##### 5.1.4.8.2	Solution details

###### 5.1.4.8.2.1	Architecture

The reference architecture for non-roaming scenario is shown in TS23.501.

NG MM function (MMF)

NG Session Management (SMF)

NG Policy Control Plane function (PCF))

AF

NG Subscriber Data Management (SDM)

NG UE

NG (R)AN

NG Core UP Functions

Data Network

Authentication Server and Credential Repository (ARP)

NG Auth. Function (AUF) (SCM + SEA)

NGt

NG12

NG8

NG11

NG7

NG5

NG1

NG2

NG4

NG3

NG6

NG9

NG MM function (MMF)

NG Session Management (SMF)

NG Policy Control Plane function (PCF))

AF

NG Subscriber Data Management (SDM)

NG UE

NG (R)AN

NG Core UP Functions

Data Network

Authentication Server and Credential Repository (ARP)

NG Auth. Function (AUF) (SCM + SEA)

NGt

NG12

NG8

NG11

NG7

NG5

NG1

NG2

NG4

NG3

NG6

NG9

In the architecture, AMF (Access and Mobility Management Function) includes Security Anchor Function (SEAF) and Security Context Management Function (SCMF). SEAF is a function that performs UE authentication for connection via different access networks, and interacts with AUSF for retrieving authentication data stored in UDM (Unified Data Management). SCMF manages security context for both C-plane and U-plane functionality.  The C-plane functions for MM and SM messages are decoupled and handled by AMF and SMF (Session Management Function), respectively, while U-plane data is handled by UPF.

This functional split requires new key hierarchy and corresponding security and key derivation procedures. In addition, the generation of keys for network slicing in NextGen system is also required to be addressed by this key hierarchy.

###### 5.1.4.8.2.2	Key Hierarchy

A key hierarchy for the NextGen system is shown in Figure 5.1.4.8.2.2-1. In general, the key hierarchy is similar to that of SAE/LTE with the differences like  (1) Separation of C-plane keys for NAS MM and SM (2) Ability to generate slice-specific keys for NAS SM and keys (KUP) for termination of the UP security at a user plane gateway (3) Provision of keys for other AN than 3GPP (4) Provision of legacy keys for existing 3GPP and non-3GPP radio network. It is assumed that network slicing exists only in core network and there’s no network slicing in RAN.

The description of each key in the key hierarchy is as follows.

| K | - The subscriber credential that is held in the UE and UDM. |
| --- | --- |
| CK, IK | - Confidentiality and integrity key for backward compatibility. |
| KSEAF | - Security anchor key derived from K for authentication and derivation of subsequent keys. |
| KNAS_MM | - Control plane key commonly used for NAS MM messages to derive integrity and encryption protection keys. |
| KNAS_SM | - Control plane key provided for NAS SM messages of each network slice to derive integrity and encryption keys. |
| KAN/NH | - Key provided to the AN (similar to KeNB/NH in LTE) to derive RRC and UP integrity and encryption keys. |
| KUP | - User plane key provided for each network slice when the user plane security terminates at the UPF. |
| KAN_other | - Key for other NextGen AN than 3GPP |
| K3GPP_AN | - Legacy key for existing 3GPP radio network |
| Knon-3GPP_AN | UE/(R)AN UE/(R)AN UE/UDM incl. ARPF UE/UDM incl. ARPF UE/AMF incl. SEAF, SCMF UE/AMF incl. SEAF, SCMF Network Slice (core network) Network Slice (core network) CK, IK CK, IK UE/AMF UE/AMF KNAS_MM KNAS_MM Knon-3GPP_AN Knon-3GPP_AN K3GPP_AN K3GPP_AN KAN_other KAN_other KNAS-MMint KNAS-MMint KNAS-MMenc KNAS-MMenc UE/SMF UE/SMF KSessNenc KSessNenc KSessNint KSessNint KSess1int KSess1int UE/SMF UE/SMF KNAS-SMint KNAS-SMint KNAS-SMenc KNAS-SMenc KNAS_SM KNAS_SM KUPint KUPint KUPenc KUPenc KRRCint KRRCint KRRCenc KRRCenc KAN/NH KAN/NH KSess1enc KSess1enc KUP KUP KSEAF KSEAF K K Legacy key for existing non-3GPP radio network |

Slice

UE/AN

UE/ARPF (AUSF)

UE/SEAF

KAN/NH

KRRCenc

KRRCint

KUPenc

KUPint

K

KSEAF

KSCMF

UE/UP-GW, SM

KSess1enc

KSess1int

KSessNint

KSessNenc

UE/SCMF

KUPx

KNASenc

KNASint

KCP-CN

UE/MM

Slice

UE/AN

UE/ARPF (AUSF)

UE/SEAF

KAN/NH

KRRCenc

KRRCint

KUPenc

KUPint

K

KSEAF

KSCMF

UE/UP-GW, SM

KSess1enc

KSess1int

KSessNint

KSessNenc

UE/SCMF

KUPx

KNASenc

KNASint

KCP-CN

UE/MM

Figure 5.1.4.8.2.2-1: Key hierarchy of the NextGen system.

5.1.4.8.2.3	Key derivation

As well as key hierarchy, key derivation also includes similar process to that of SAE/LTE. Firstly, KSEAF is derived during AKA (Authentication and Key Agreement) procedure in both UE and UDM, where NSSAI (Network Slice Selection Assistance Information) is additionally used as one of input parameters. A diagram of key derivation for the subsequent keys in the key hierarchy for NextGen system is shown in Figure 5.1.4.8.2.3-1. Each key is derived by inputting its upper layer key and parameters into KDF (Key Derivation Function). The three keys for NAS MM, NAS SM and AN are derived from KSEAF with parameters of COUNT for NAS MM key, SST (Slice/Service Type) and SD (Slice Differentiator) for NAS SM key, and NAS Uplink Count for AN key, where COUNT is a value incremented with mobility event in AMF. RAN slice parameters should be input if there is network slice in AN. The key for U-plane (KUP) is derived from NAS SM key with parameters of Counter, Time limit and Data volume, where Counter is incremented with update of KUP, Time limit is the stipulated time for which KUP is valid, and Data volume indicates the amount of data that is expected to be used. Finally, the keys for integrity and confidentiality protection, which have suffix of “int” and “enc”, respectively, are derived with its upper layer key and algorithm ID, where a value of Counter is also input for derivation of

KSessint and KSessenc.

KSEAF

KSEAF

KNAS_MM

KNAS_MM

KNAS_MMint

KNAS_MMint

KNAS_MMenc

KNAS_MMenc

KNAS_SMint

KNAS_SMint

KNAS_SMenc

KNAS_SMenc

KRRCint

KRRCint

KRRCenc

KRRCenc

KUPint

KUPint

KUPenc

KUPenc

KNAS_SM

KNAS_SM

KAN

KAN

KUP

KUP

KSessint

KSessint

KSessenc

KSessenc

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

KDF

COUNT

COUNT

SST (Slice/Service Type)

SST (Slice/Service Type)

SD (Slice Differentiator)

SD (Slice Differentiator)

NAS_MM-int-algo, ID

NAS_MM-int-algo, ID

NAS_MM-int-algo, ID

NAS_MM-int-algo, ID

KDF

KDF

NAS_SM-enc-algo, ID

NAS_SM-enc-algo, ID

NAS_SM-int-algo, ID

NAS_SM-int-algo, ID

RRC-int-algo, ID

RRC-int-algo, ID

RRC-enc-algo, ID

RRC-enc-algo, ID

AN-UP-int-algo, ID

AN-UP-int-algo, ID

AN-UP-enc-algo, ID

AN-UP-enc-algo, ID

NAS Uplink Count

NAS Uplink Count

RAN slice parameters

RAN slice parameters

Counter

Counter

Time limit

Time limit

Data volume

Data volume

UP-enc-algo, ID

UP-enc-algo, ID

UP-int-algo, ID

UP-int-algo, ID

Counter

Counter

KDF

KDF

KDF

KDF

##### Figure 5.1.4.8.2.3-1: Key derivation diagram for the NextGen system.

##### 5.1.4.8.3 	Evaluation

**************** End of Change *************