3GPP TSG SA WG3 (Security) Meeting #86	S3-170438

6-10 Feb 2017, Sophia Antipolis, France		Revsion of S3-170243

Source:	China Mobile

Title:	Clarifying the solutions in the section 5.2.4.17

Document for:	Approval

Agenda Item:	8.4.2

Work Item / Release:	Architecture and Security For Next Generation System / Rel-14

Abstract of the contribution: This pCR proposes to clarify the solutions in the section 5.2.4.17.

Introduction

The following editor’s notes were proposed for the solution #2.17:

Editor’s note: The evalution part should be updated following on the guideline agreed in ad-hoc meeting on FS_NSA.

Editor’s note: The solution should be modified to avoid the dependent on a 3rd party for the authentication of the reported equipment identifier (e.g., the operator doesn't need to make use of any entities outside of operator's network)

For the first editor’s note, this pCR proposes to update the evaluation descriptions according to the guiding principles in annex B.2.4 and delete this editor’s note. We clarify that both the solutions can protect the subscription privacy and explain how to mitigate the impact of the inquiry workload and the dependence of the operator on the manufacturer.

For the second editor’s note, many devices don’t know which operator’s service will be subscribed when these devices are at the manufacture, so it is reasonable that provisioning the device certificate or device public key by the manufacturer or a 3rd party at the manufacture.The dependence of the operator (or service provider) on the manufacturer or a 3rd party issued device certificate or public key may be mitigated by the following ways:

- The manufacturer or a 3rd party which issues the decive certificate or the device public key shall be trusted by the operators.

- The operators (or service provider) may to issue (e.g., using a CA owned or trusted by the operator) their own device certificate or device public key tosubstitute for the device certificate or device public key issued by the manufacturer or a 3rd party and then the operator will no longer need to trust the manufacturer or a 3rd party (cf.clause 5.2.4.10.2.2).

We propose to add the above clarifies and delete these editor’s notes.

In addition, we propose to delete the fresh challenge in the equipment identifier response because the NAS signalling has been protected to prevent the replay attack. We also propose to change some terms to be consistent with the current agreed terms.

Proposed pCR

***	BEGIN OF THE CHANGE	***

5.2.4.17	Solution #2.17: Equipment identifier Authentication using the device certificate and IMEI binding

5.2.4.17.1	Introduction

This solution uses the (IMEI, device certification) binding to addresses Key issue #2.4: Equipment identifier authentication. In this solution, the following pre-configurations are needed:

-	When a device is at the manufacture, a device certificate and the private key associated with the certificate are provisioned by the device’s manufacturer or trusted third party. The device certificate is signed by a Certificate Authority (CA). The CA can be a device manufacturer CA or a trusted 3rd party CA that is trusted by the manufacturer.

-	The private key associated with the device certificate shall be securely stored in the device.

-	The device certificate and the equipment identifier (i.e. IMEI) are bound and the (IMEI, Device Certificate) binding is published in the manufacturer’s device repository or third party’s repository. The certificate does not need to include the plain text of the equipment identifier because they have been bound to each other in the repository. So the equipment identifier will not be disclosed even if the device certificate is published.

NOTE: It is ffs what information will be wroten in the subject field.

-	An operator, if trusts the device manufacturer CA or a 3rd party CA, can inquire the (IMEI, Device Certificate) binding from the manufacturer’s device repository or third party’s repository.

-	The CA’s certificate or public key needs to be pre-configured in the operator’s AAA/HSS or CP-AU/CP-CN. It can also be stored with the (IMEI, device Certificate) binding in the repository.

5.2.4.17.2	Solution details

The following figure describes the message flows of equipment identifier authentication:

# Figure 5.2.4.17.2-1: equipment identifier authentication based on (IMEI, device cert) binding

- The device certificate has been pre-provisioned in the NG-UE and the private key associated with the device certificate has been securely stored in the device. The (IMEI, device certificate) binding has been published and stored in the repository of the manufacturer or 3rd party trusted by the manufacturer.

- The mutual authentication has been performed between the network and the NG-UE.

- The network entity (e.g. SEAF or CP-CN) sends the equipment identifier request..

- The NG-UE uses the private key to sign the IMEI.

- The NG-UE returns the equipment identifier response to the SEAF/CP-CN. This message includes the IMEI and the signature of the IMEI. This response should be encapsulated in a signalling message which can protect the identity from leaking and replay attack, like the NAS SMP message in LTE.

- The SEAF/CP-CN sends the device certificate inquiry including the received device’s IMEI to the AUSF/ARPF.

- The AUSF/ARPF inquires the device certificate using the IMEI from the (IMEI, device certificate) binding repository.

Editor’s Note: It is FFS whether the binding between the IEMI and device certificate can be cached in the AUSF/ARPF or not.

- The AUSF/ARPF returns the device certificate to the SEAF/CP-CN.

- The SEAF/CP-CN uses the CA’s public key to verify the device’s certificate. If the verification is successful, the SEAF/CP-CN uses the public key in the device certificate to check the signature. The equipment identifier is authenticated if the signature check is successful.

NOTE: 	The CA’s public key can be pre-configureted in SEAF/CP-CN and can also be acquired from the (IMEI, device certificate) binding repository via AUSF/ARPF if the CA’s public key is pre-stored in the repository.

5.2.4.17.3	Evaluation

This solution saves the valueable air resource as it does not transimit the device certificate in the air interface. Since the binding between the IMEI and the device certificate is maintained in the repository and only the authorized entities can inquire the binding, this solution can prevent the leakage of the equipment identifier.

The inquiring the (IMEI, device certificate) binding causes the inquiry workload less than the authentication of the device certificate directly (cf. the solution #2.10). Since the equipment identifier authentication is infrequent, the inquiry wokload does not seriously affect the network. The AUSF/ARPF can also store the (IMEI, device certificate) binding in the local to reduce the inquiry workload.

In this solution, the manufacturer or a 3rd party which issues the decive certificate nees to be trusted by the operators.

Editor’s note: The solution should be modified to avoid the dependency on a 3rd party for the authentication of the reported equipment identifier (e.g., the operator doesn't need to make use of any entities outside of operator's network)

***	END OF THE CHANGES	***