3GPP TSG SA WG3 (Security) Meeting #86bis	    S3-170815

27-31 March 2017, Busan (Korea)

Source:	NEC

Title:	pCR to TR 33.899: Intra AMF, Inter SMF, Inter NG RAN handover without Xn interface

Document for:	Approval

Agenda Item:	5.1.1

Work Item / Release:	FS_NSA / Rel-14

Abstract: This pCR proposes a solution for Intra AMF, Inter SMF, Inter NG RAN handover without Xn interface to Security Area #1 Architectural aspects of Next Generation security.

#### Introduction

#### This pCR proposes a solution for Intra AMF, Inter SMF, Inter NG RAN handover without Xn interface.

#### Proposal

***** Start of Change *****

#### 5.1.4.x	Solution #1.x Intra AMF, Inter SMF, Inter NG RAN handover without Xn interface

#### 5.1.4.x.1	Introduction

This solution proposes the handover procedure for Intra AMF, Inter SMF, Inter NG RAN (without Xn interface) scenario. This solution considers the use of keys for protection of NAS signalling between the UE and the SMF.

#### 5.1.4.x.2	Solution details

#### Figure 5.1.4.x.2-1 shows the procedure for Intra AMF, Inter SMF, Inter NG RAN handover without Xn interface.

The procedure for Intra AMF, Inter SMF, Inter NG RAN handover without Xn interface is as follows.

- If the handover is required, the UE sends the Measurement Report to the source NG (R)AN.

- The source NG (R)AN evaluates the report and makes the handover decision.

- The source NG (R)AN sends the Handover Required (UE Security Capabilities, Handover Restriction List, NSSAI) command to the AMF.

- The AMF selects the SMF based on NSSAI.

- The AMF sends the Create Session Request to the target SMF.

- The target SMF derives KNAS-SM* for NAS signalling protection between the UE and SMF.

- The target SMF selects the UPF for the slice.

- The target SMF derives the KUP and session keys for the slice.

Figure 5.1.4.x.2-1: Intra AMF, Inter SMF, Inter NG RAN handover without Xn interface.

- The target SMF sends the Create Session Response to the AMF.

- The AMF sends the Handover Request (UE Security Capabilities, Handover Restriction List, NSSAI) to the target NG (R)AN.

- The target NG (R)AN checks if the NSSAI can be supported or not.

- If the UE qualifies for accessing the slice via the new NG (R)AN, the target NG (R)AN refreshes the AN keys (derives KAN* and the subsequent RRC and UP integrity and encryption keys).

- It then sends the Handover Request ACK to the AMF.

- The AMF sends the Create Data Forwarding Tunnel Request to the source SMF.

- The source SMF creates the tunnel for data transfer to the target SMF.

- The source SMF sends the Create Data Forwarding Tunnel Response to the AMF.

- The AMF sends the Handover Command to the source NG (R)AN.

- The source NG (R)AN sends the Handover Command to the UE.

- Using the parameter sent in the Handover Command, the UE derives the KNAS-SM*.

NOTE: If keys for protection of NAS messages between UE and SMF are not required, the Steps 6 and 19 may be skipped and KUP can be derived from KSEAF in Step 20. This KUP is for UP security terminating at the UPF. This key could be distributed to any other function where UP security terminates (e.g. UP-STF), based on the decision in SA3.

- The UE then derives the KUP and session keys from KNAS-SM*.

- The UE derives KAN* and the subsequent RRC and UP integrity and encryption keys.

- The UE sends the Handover Complete command to the target NG (R)AN.

#### 5.1.4.x.3	Evaluation

#### FFS

***** End of Change *****