3GPP TSG SA3 Meeting #86bis	S3-170917

Busan, Korea; 27th – 31st  March 2017	was S3-170869

Source:	Juniper Networks, KPN

Title:	Comments on S3-170725 CR to TS 33.401 adding BEST functionality

Document for:				Approval

Agenda Item:	4.2

Work Item / Release:	BEST-MTC / Rel-15

Abstract of the contribution: This contribution corrects inconsistencies in EMSDP message names and exchanges, and also fixes some typos. All changes from the CR were accepted in this document and only changes made to the CR are highlighted in tracked changes. Furthermore, the contribution only includes sections where changes were added to the original CR.

**** First Change *

**** End of First Change ****

**** Second Change ****

**** End of Second Change ****

**** Third Change ****

**** End of Third Change ****

**** Fourth Change ****

# X	Security Procedures for Battery Efficient Security for Low Throughput Devices (BEST)

## X.1	Introduction

This clause defines elements, protocols and procedures that enable battery efficient security for low throughput devices such as MTC devices.  The BEST service is a secure channel between a UE and a HSE, optimised for low throughput and high latency devices that are battery constrained.  The security is between the UE and either an element in the service provider home network (the HSE) or an element in the enterprise domain (the EAS). The design is modular and extensible so that it can be used to satify a wide range of use cases.

The following services are defined:

- BEST key agreement only service – This service is a battery efficient service for key agreement between a BEST compliant UE and the HSE or the EAS. The data plane for this service is provided by the application layer between the UE and the EAS and is out of scope of this specification.

- BEST user plane integrity protected service – This service is a battery efficient integrity protected user plane service for low throughput devices. This service includes the key agreement and includes integrity protected security over small data over NAS User Plane.  The data plane for this service can be either terminated in the HSE or in the EAS. Control messages are always terminated in the HSE.

- BEST user plane confidential service – This service is a battery efficient integrity and confidentiality protected user plane service for low throughput devices. The data plane for this service can be either terminated in the HSE or in the EAS. Control messages are always terminated in the HSE.

It may be possible for the UE to have concurrent BEST sessions.

## X.2	BEST Framework Service Description

### X.2.1	Architecture

Figure X.2.1-1: The architecture of the extended user plane protection service

The BEST service requires the following components:

- Home Security Environment (HSE) – This is the termination point in the home network that performs the following functions:

  - Terminating the control plane for BEST between the UE and the HSE

  - Terminating the secure communication for BEST between the UE and the HSE if UE-to-HSE security is selected.

  - Routing the user plane traffic for BEST between the UE and the Enterprise Application Server (EAS) if UE-to-EAS security is selected.

  - Anchor for BEST Key agreement only service. Exposes an interface for EAS to obtain MNO provided preshared key.

- End to Middle Key Server (EMKS) – This is an optional key server element that manages the key communication with the HSS (for quintets) and stores keys to reduce loading on the HSE and HSS. The EMKS has interfaces to the HSS (S6a) and the HSE (S6a).

The BEST service uses the following interfaces:

- S6a between the HSS and the HSE

- S6a between the HSS and EMKS

- S6a between the EMKS and the HSE

Annex W describes a candidate interface based on Restful HTTP for the communication between the HSE and the EAS.

## X.3	Procedures between the UE and the HSE

### X.3.1	Overview of BEST procedures

To use the BEST service the UE shall first open a data session.  The UE may either use a locally stored IP address to locate the HSE or use a "BEST APN" where the traffic is directed by the PDN Gateway to the correct HSE for that UE.  Once a data session exists, the UE may initiate the BEST service.  It is up to the UE as to when it establishes the data session that is used for BEST and the bearer used.

The BEST service consists of 5 general processes between the UE and the HSE: session initiation and key agreement, key management, data transfer, session termination, and message rejection.  The details of the End to Middle Secure Data Protocol (EMSDP) used for the BEST service is detailed in Annex X.

When BEST user plane security services are used, UP data messages are between the UE and the HSE.

UE

HSE

Session Initiation and Key Agreement (Control Plane)

EMSDP Session Request (UE ID, BEST capabilities, Enterprise information (opt))

EMSDP Session Start (Key Agreement, Session Parameters, Request Validation, HSE ID(opt), EAS container (opt))

EMSDP Session Start Confirmation (Authentication result) (conditional)

Data Transfer

Data Transmission (User Plane)

EMSDP Manage Keys Request (Key ID) (opt)

EMSDP Manage Keys Response (Key ID, Key Agreement)

Session Key Management (Control Plane)

EMSDP Session Terminate Request

EMSDP Session Terminate Response

Session Termination (Control Plane)

Message Reject  (Control Plane)

EMSDP Message Reject

Note:
- Bidirectional arrows indicate that the message can be initiated by either party.
- Arrows with dashed lines indicate optional messages.

Figure X.3.1-1: Generalised BEST service flow

### X.3.2 BEST Session Initiation and Key Agreement

The UE shall initiate a BEST session using the EMSDP Session Request message following the establishment of the PDP Context.  To optimise the message flow for battery constrained devices, the EMSDP Session Response is combined with Session Key Agreement.

The EMSDP Session Request message shall include the UE Identity, BEST capabilities of the UE, the UE serving network and details of the enterprise service including the Enterprise server Id (EAS Id) that the BEST service is being used for.

The EMSDP Session Start message shall include the RAND and AUTN needed for a key agreement of the BEST keys, the BEST service parameters and a checksum validating the previous EMSDP Session Request message.

The HSE shall determine the parameters for the BEST service.  The HSE may use the location information provided by the UE to determine whether aspects of the BEST service, such as cyphering, can be used in that location.

As a result of the key agreement exchange the UE and HSE shall derive the keys to be used for future control plane and user plane security. In case of Key agreement only service, UE and HSE shall derive the intermediate key needed for application layer security at the enterprise layer.

To optimise the BEST service for battery constrained devices, confirmation of the BEST session start is not required.  The UE sending a UP message to the HSE is by itself is an implied confirmation.  However, if the BEST service is being used for key agreement only, the HSE shall require the UE to send EMSDP Session Start Confirmation by setting an indicator in the EMSDP Session Response message.

### X.3.4 BEST Session Key Management

At any time during the session, either the UE or the HSE may trigger re-negotiation of the keys being used for the BEST service using the EMSDP Manage Keys Request and Response exchange.

### X.3.5 BEST Session Termination

At any time, either the UE or the HSE may terminate the current BEST session using the EMSDP Session Termination Request and Response message exchange.  Once terminated, all relevant keys and IDs shall be discarded and both the UE and HSE shall ignore further messages using that session ID, unless a session with that ID is re-established using the session initiation process.

### X.3.6 BEST Message Reject

Either the UE or the HSE may at any time respond with a EMSDP Message Reject message, upon which the recipient shall discard all relevant keys and IDs of the session, and both the UE and HSE shall ignore further messages using that session ID.

The EMSDP Message Reject is also used when the HSE needs to prompt a UE to initiate a new session using the Session Start message.  For example, if it receives a UP packet from the UE on a BEST session for which it aged out the context.

## X.4 Procedures between the HSE and the EAS

The message exchanges between the HSE and the EAS are essentially a mirror of the ones between the UE and the HSE.  All control plane messages are terminated or initiated by the HSE.

When BEST user plane security services are used, the UP data messages are end-to-end between the UE and the EAS.

Note:  The actual details and standardization of the HSE to EAS interfaces is out of scope for this release.

EAS Session Start

UE

EAS

Session Initiation 
and Key Agreement 
(Control Plane)

EMSDP Session Request

EMSDP Session Start

EMSDP Session Start Confirmation

Data Transfer

Data Transmission 
(User Plane)

EMSDP Manage Keys Request

EMSDP Manage Keys Response

Session Key 
Management 
(Control Plane)

EMSDP Session Terminate Request

EMSDP Session Terminate Response

Session Termination 
(Control Plane)

Message Reject  
(Control Plane)

EMSDP Message Reject

HSE

EAS Message Reject

EAS Session Request

EAS Session Registration

EAS Session Start Confirmation

EAS Manage Keys Request

EAS Manage Keys Response

EAS Session Terminate Request

EAS Session Terminate Response

Figure X.4-1: Generalised BEST EAS service flow

### X.4.1 Registration

The EAS shall register for BEST service by sending the Session Registration message to the HSE. This results in a secure connection to be setup between the HSE and the EAS.

The EAS Session Registration message shall include the Enterprise server Id (EAS Id). The HSE sets up a session context for the EAS.

### X.4.2 Key Request

During the Key agreement procedure, described in clause X.3.2, HSE may forward the intermediate key to the EAS in the EAS Session Request message. When BEST is used for Key agreement only, the HSE shall forward an intermediate key that is specifically derived for the enterprise as defined by key definition rules in clause A.y.1.3. When BEST UE-to-EAS user plane services are used, the HSE shall forward the intermediate key as derived in clause A.y.1.1. The EAS shall respond with the EAS Session Start message. In the case of BEST UE-to-EAS user plane services this EAS Session Start message may include a key identifier for the Enterprise Key to be used.

### X.4.3 Key Management

The EAS initiated renegotiation of intermediate keys is FFS.

## X.5 BEST Data Service

Once the BEST session is successfully initiated, the UE or the HSE or EAS may send UP data using the negotiated keys.

If the BEST UP session is set up between the UE and the HSE, then the UP packets are initiated or terminated by the HSE.  In this case, the low power optimized EMSDP protocol, as detailed in clause Annex x, is used.

If the BEST UP session is setup for end-to-end security to the EAS, then the HSE passes the UP messages to the EAS after checking the message is formatted correctly and that it is a UP message. The key used by the UE and EAS to encrypt and decrypt data messages (when required) is KE2Eenc and to integrity protect the message is KE2Eint. Verification of any Key ID, counter, message integrity and deciphering is the responsibility of the EAS.

If the BEST key agreement service is used to set up a data session between the UE and EAS, the UE and EAS may use EMSDP or another protocol with BEST derived PSK, e.g., IKE/IPsec or (D)TLS for user plane data transmission. In this case, the HSE does not handle the UP messages.

## X.6	Key Management

### X.6.1	Key Agreement and Refresh

#### X.6.1.1	Key setup messaging between HSE and UE

New keys are agreed either at the start of a BEST session or as required due to key aging or counter thresholds being met.  Key agreement is based on the 3GPP AKA mechanism detailed in TS 33.102 [4] and the AKA challenge is transported between the HSE and the UE as part of the BEST service detailed in clause X.3.2.

The EMDSP protocol has 7 Key IDs each with a separate keyset consisting of an integrity Key (KE2Mint), an encryption key (KE2Menc), optionally an End to End Intermediate Key (KInt_E2E) and optionally an End to End Pre-Shared Key (Kint_PSK).

The End to End Intermediate Key (KInt_E2E) is used together with a local enterprise secret key to calculate KE2Eint and KE2Eenc when BEST User plane security services are used end to end between the UE and the EAS.

The End to End Pre-Shared Key (Kint_PSK) is used together with a separate enterprise identifier to calculate KInt_EAS_PSK.  There can be many KInt_EAS_PSK derived from one Kint_PSK.

Figure X.6.1.1-1 shows the generic key agreement process:

With EMKS

Without EMKS

UE

HSE

1. EMSDP Session Request

EMKS

HSS

2. Keys required?

3. Authentication-Information-Request

4. Authentication-Information-Answer

d. Authentication-Information-Answer

c. Authentication-Information-Answer (to deliver authentication vectors – if required)

a. Authentication-Information-Request

b. Authentication-Information-Request (to request authentication vectors - if required)

Authentication vectors needed?

8. EMSDP Session Start

5. Calculate HSE Keys

EAS

6. EAS Session Request

7. EAS Session Start

12. Calculate E2E Keys

9. EMSDP Session Start Confirmation (conditional).

11. Calculate UE Keys

10. EAS Session Start Confirmation

Figure X.6.1.1-1 Generic BEST key agreement process

The Key agreement steps are:

1.	EMSDP Session Request (UE ID, BEST capabilities, Enterprise information (opt), location ID). The UE shall send the EMSDP Session Request (UE ID, BEST capabilities, Enterprise information (opt) to set up a new BEST session.

2.	Keys required? - the HSE shall check to see if there are valid keys with valid counter values available in the HSE for that UE then the following is checked:

-	If the HSE has a valid set of keys for the indicated session and the UE ID is valid for that session then the HSE may start the BEST session without re-negotiating the keys (step 8).

-	If the UE ID is valid  for that HSE and the HSE does not have a valid set of keys for the indicated session or the HSE wishes to update the keys, then it shall first renegotiate the keys (steps 2 to 7) and then start the BEST session (step 8).

-	If the UE ID is not valid for that session ID or the UE does not support the level of service required by the HSE or the enterprise information is not valid for the HSE, then the HSE may reject the command.

3. Authentication-Information-Request over S6a interface – The HSE shall use the S6a interface to the HSS to request one or more authentication vectors using the UE IMSI.

4. Authentication-Information-Answer over S6a interface – The HSS uses the S6f interface to the HSE to return the requested authentication vectors.

a. Authentication-Information-Request over the S6a interface between HSE and EMKS – Where an EMKS is used, the HSE shall use the S6a interface to the EMKS to request an authentication vector using the UE IMSI.

b. Authentication-Information-Request over the S6a interface between EMKS and HSE – The EMKS shall use the S6a interface to the HSS to request one or more authentication vectors using the UE IMSI.

c. Authentication-Information-Answer over the S6a interface between EMKS and HSE – The HSS shall use the S6a interface to the EMKS to return the requested authentication vectors.  These vectors may be stored on the EMKS for later use.

d. Authentication-Information-Answer over the S6a interface between HSE and EMKS – The EMKS uses the S6a interface to the HSE to return the requested authentication vector.

5. Calculate HSE Keys -  See key derivation details in Annex Y.

6. The HSE may optionally send  "EAS Session Request" to the EAS over the SGi interface – The HSE shall use the SGi interface to the EAS to inform the Enterprise Server of the new UE-to-EAS session request and shall forward the intermediate E2E key (KInt_E2E)  to the Enterprise Server in case the intermediate E2E key (KInt_E2E) was calculated in the previous step.

7. The Enterprise Server sends a "EAS Session Start" to the HSE – The Enterprise Server shall respond by sending the "EAS Session Start" message. This message may contain an EAS container that may include an identifier for the Enterprise Key.

8. EMSDP Session Start message -  The HSE shall send a EMSDP Session Start (Key Agreement, Session Parameters, Request Validation, HSE ID(opt), EAS container (opt)).

9. EMSDP Session Start Confirmation - UE optionally, if requested in the Session Start, responds with an EMSDP Session Start Confirmation message.

10.  EAS Session Start Confirmation - The HSE may optionally send

11. Calculate UE Keys – See key derivation details in Annex Y.

12. Calculate E2E Keys – See key derivation details in Annex Y.

#### X.6.1.2	Usage of Keys

For UE-to-HSE BEST sessions, the following keys shall be used:

-	The KE2Menc shall be used by the UE and the HSE for the encryption of the user plane and the control plane, according to the agreed encryption algorithm.

-	The KE2Mint shall be used by the UE and the HSE for the integrity protection of the user plane and the Control plane according to the agreed integrity protection algorithm.

For UE-to-EAS BEST sessions, the following keys shall be used:

-	The KE2Menc shall be used by the UE and the HSE for the encryption of the control plane, according to the agreed encryption algorithm.

-	The KE2Mint shall be used by the UE and the HSE for the integrity protection of the control plane according to the agreed integrity protection algorithm.

-	The KE2Eenc shall be used by the UE and the EAS for the encryption of the user plane, according to the agreed encryption algorithm.

-	The KE2Eint shall be used by the UE and the EAS for the integrity protection of the user plane according to the agreed integrity protection algorithm.

For key-agreement only BEST service, the following key shall be used:

-	The KInt-PSK together with the Enterprise Application Server id shall be used by the HSE to generate EAS-specific pre-shared key (KInt_EAS_PSK). This is used for protection of the User plane as defined by the application layer security protocol used between the UE and the EAS.  The UE to EAS protocol in this case is out of scope of this specification.

Note: As the KInt_EAS_PSK is known to the HSE, the EAS and the UE will have to transform the key further to achieve end-to-end security.  This transformation is out of scope of this specification.

-	The KE2Menc shall be used by the UE and the HSE for the encryption of the control plane, according to the agreed encryption algorithm.

-	The KE2Mint shall be used by the UE and the HSE for the integrity protection of the control plane according to the agreed integrity protection algorithm.

#### X.6.1.3	Key Setup for BEST session end point modification

The modification of a BEST UE-to-EAS session to a BEST UE-to-HSE session does not require any new calculations of keys.

The modification of a BEST UE-to-HSE session to a BEST UE-to-EAS session requires new keys to be calculated when no EAS keys are available in the EAS. Before sending the EMSDP Modification command the HSE does the following:

-	Checks with the EAS whether it has keys in storage

-	If not, obtains a EAS container that may contain a key identifier for the Enterprise Key from the EAS and forwards this to the UE in the EMSDP Modification command.

#### X.6.1.4	BEST Key Handling

The aging of keys and any counter window used are out of scope of this specification.

### X.6.2	BEST Key Hierarchy

#### X.6.2.1	Introduction

This clause describes the key hierarchies for BEST for the following options:

-	Best Key Hierarchy in case a separate BEST domain is used.

#### X.6.2.2	BEST Key Hierarchy for Separate BEST Domain

Figure 17.6.2.2-1: Key Hierarchy for Separate BEST Domain

The KInt_E2E, KInt_PSK and all of the keys derived from them are generated when indicated to do so in the BEST CP messaging.

**** End of Fourth Change ****

**** Fifth Change ****

A.y Derivation of BEST Keys

## A.y.1	BEST key derivation for separate BEST Domain

## A.y.1.1	Derivation of UE-to-HSE keys

The following input string shall be used when the UE and the HSE derive the BEST end2end Intermediate key KInt_E2E, BEST keys KE2Menc or KE2Mint from CK, IK:

-	FC = 0x60,

-	P0 = HSE id if supplied else NULL,

-	L0 = length of HSE id (i.e. 0x00 0x03 if HSE id supplied or 0x00 0x00 if not),

-	P1 = SQN  AK

-	L1 = length of SQN  AK (i.e. 0x00 0x06)

-	P2 = algorithm id

-	L2 = length of algorithm id (i.e. 0x00 0x01)

Table A.y.1.1-1: Algorithm type distinguishers

| Algorithm distinguisher | Value |
| --- | --- |
| BEST encryption key (KE2Menc) | 0x01 |
| BEST integrity Key (KE2Mint) | 0x02 |
| BEST end2end Intermediate Key (KInt_E2E) | 0x03 |
| BEST end2end Intermediate Key (KInt_PSK) | 0x04 |

The input key shall be equal to the concatenation CK || IK of CK and IK.

## A.y.1.2	Derivation of UE-to-EAS keys

The following input string shall be used when the UE and the EAS derive the BEST User plane keys KE2Eenc or KE2Eint from KInt_E2E.:

-	FC = 0x61,

-	P0 = Enterprise Application Server id,

-	L0 = length of Enterprise Application Server id (i.e. 0x00 0x03),

-	P1 = algorithm id

-	L1 = length of algorithm id (i.e. 0x00 0x01)

Table A.y.1.2-1: Algorithm type distinguishers

| Algorithm distinguisher | Value |
| --- | --- |
| BEST e2e encryption key (KE2Eenc) | 0x01 |
| BEST e2e integrity Key (KE2Eint) | 0x02 |

The input key shall be equal to the concatenation KInt_E2E || Enterprise_Key of KInt_E2E  and Enterprise_Key. When the Enterprise container specifies no Enterprise key, the Enterprse_Key shall be set to zero (0..0).

Note: The Enterprise_Key provisioning is outside the scope of 3GPP.

## A.y.1.3	Derivation of EAS specific pre-shared key (KInt_EAS_psk) in HSE

The following input string shall be used when the UE and the HSE derive the pre shared key KInt_EAS_PSK from KInt_PSK

-	FC = 0x62,

-	P0 = Enterprise Application Server id,

-	L0 = length of Enterprise Application Server id (i.e. 0x00 0x03)

The input key shall be KInt_PSK, as derived in A.y.1.1.

**** End of Fifth Change ****

**** Sixth Change ****

# Annex X (normative): End to Middle Secured Data Protocol (EMSDP)

## X.1	Introduction

The EMSDP protocol is used for the BEST service. This protocol runs between the UE and the HSE and has been optimised for efficient, secure transfer of both user plane and control plane data.

## X.2	EMSDP Protocol Description

### X.2.1	Data Stack

Figures X.2.1-1, X.2.1-2 and X.2.1-3 shows the data stacks for the EMSDP transfers (based on 3GPP TS 23.401 [2]) between the HSE and the UE.

EMSDP

PDCP

RLC

MAC

L1

UE

RLC

MAC

E-UTRAN

UDP/IP

L2

L1

L1

PDCP

GTP-U

Relay

LTE - Uu

UDP/IP

L2

Serving GW

UDP/IP

L2

L1

L1

GTP-U

GTP-U

Relay

S1-U

UDP/IP

L2

IP

L2

L1

L1

S5

PDN GW

Transformation

GTP-U

UDP/IP

EMSDP

UDP/IP

IP

L2

L1

HSE

SGi

Figure X.2.1-1: Non-IP data stack for the EMSDP transfers over UP

NAS

PDCP

RLC

MAC

L1

UE

E-UTRAN

Relay

LTE - Uu

Serving GW

S1-MME

PDN GW

HSE

SGi

RLC

L1

MAC

PDCP

SCTP/IP

L2

L1

S1AP

EMSDP

IP

L2

L1

EMSDP

Relay

SCTP/IP

L1

L2

NAS

S1AP

MME

Relay

IP

L1

L2

UDP

GTP-U

GTP-U

IP

L2

L1

UDP

UDP

Relay

IP

L1

L2

IP

L2

L1

UDP

UDP

GTP-U

IP

L2

L1

GTP-U

UDP

S11

S5

Figure X.2.1-2: Non-IP data stack for the EMSDP transfers over NAS

If the UE selects the non-IP PDN data type as defined in clause 4.3.17.8. of 3GPP TS 23.401[2], the PDN GW encapsulates the non-IP EMSDP payload in PtP (IP and UDP as detailed in clause 4.3.17.8.3.3.2 of 3GPP TS 23.401[2]) for the communication with the HSE.  In case of the EMSDP is in IP PDN, the PtP encapsulation is unnecessary, and the UE’s IP packet is directly forwarded to the HSE.  This is shown on Figure X.2.1-2.

IP/UDP

PDCP

RLC

MAC

L1

UE

RLC

MAC

E-UTRAN

UDP/IP

L2

L1

L1

PDCP

GTP-U

Relay

LTE - Uu

UDP/IP

L2

Serving GW

UDP/IP

L2

L1

L1

GTP-U

GTP-U

Relay

S1-U

UDP/IP

L2

L2

L1

L1

S5

PDN GW

EMSDP

GTP-U

L2

L1

HSE

SGi

EMSDP

IP/UDP

Figure X.2.1-3: IP data stack for the EMSDP transfers

The dashed box indicates that for UE-to-EAS BEST sessions, the HSE may forward the EMSDP user plane unmodified to the EAS. The signalling plane always terminates in the HSE.

Note: It is up to the operator to decide the protocol between the HSE and the EAS. For example, an operator may agree to use the optional  Annex W. In that case, the HSE will have to encapsulate the user data in an agreed message format.

### X.2.2	EMSDP general structure

This clause details a type 01 control plane EMSDP message and a type 01 user plane EMSDP message.

Figure X.2.2-1 shows the general structure of the EMSDP type 01 message:

| Control Plane type 01 message: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UP / SP Flag | RFU | Key ID | CP COUNTER | Session ID | EMSDP Command | Cmd Options | MAC |
| 1 bit | 1 bit | 3 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |
| User Plane type 01 message: |  |  |  |  |  |  |  |  |
| UP / SP Flag | RFU | Key ID | UP COUNTER | Session ID | Data Length | Data | MAC |
| 1 bit | 1 bit | 3 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |

Note 1: The length of these fields is indicated in the BEST service discovery from the HSE.

Note 2: These fields are included in the integrity protection calculation.

Note 3: These fields are encrypted when encryption is used.  When encrypted these fields are replaced by the encrypted output.

Note 4: The length of this field is as defined in the BEST Counter Scheme being used.

Figure X.2.2-1: data stack for the EMSDP transfers

UP / CP Flag: 	This is a 1 bit field that when set to ‘1’ means that the data packet is a User Plane message and when set to ‘0’ means that the message is a control plane message.

RFU:	This is a 1 bit field that is reserved for future use. Set to ‘0’.

Key ID:	This is a 3 bit field that indicates the key being used for encryption and Integrity protection.  If no keys have been agreed between the HSE and the UE then this shall be ‘000’.

CP COUNTER:	This is a counter, used for control plane data messages, that is incremented every control plane message.  It is used to protect control plane data messages against replay attacks and its length is set based on the counter scheme indicated in the BEST service discovery (minimum length is 3 bits).  There are separate counter values for UE to HSE and HSE to UE.

UP COUNTER:	This is a counter, used for user plain data messages, that is incremented every user plain data message.  It is used to protect user plain data message against replay and its length is set based on the counter scheme indicated in the BEST service discovery (minimum length is 3 bits). There are separate counter values for UE to HSE and HSE to UE.

Session ID:	This indicates the identifier for the current session.  The value is assigned by the HSE.    Its length is the value set in the BEST service discovery. For signalling messages sent outside of a BEST session, the Session ID length shall be 1 octet and the Session ID value shall be all 0’s.

EMSDP Command:	This is a 1 byte field that contains the signalling command.  The defined commands are detailed below.

Cmd Options:	This is a TLV container that contains TLV elements that detail the options for the EMSDP command. The defined options TLVs are detailed below.

MAC:	This contains the truncated integrity result for this data packet calculated using the agreed integrity algorithm. Its length is set in the BEST service discovery.  For an EMSDP session request command the MAC shall not be present.

Data length:	This holds the length of the following data in this data packet. Its length is set in the BEST service discovery.  This is not present if the data length is set to 0 in the BEST Service Discovery.

Data:	This is the data being transferred.

### X.2.3	EMSDP Counter Schemes

#### X.2.3.1	Optimised EMSDP counter scheme

The optimised EMSDP counter scheme has a 3 bit counter length followed by the counter value.  This allows the counter length to be changed on a per message basis.  The 3 bits are used to indicate the number of octets the counter is on, the value “000” is reserved for future use.  So for instance:

Counter value “1” is represented as “001 00000001”

Counter value “257” is represented as “010 00000001 00000001”

Counter values are rejected if the counter value is less than or equal to a valid counter value already received.  The HSE and the UE may also reject a message with a counter higher than a specific offset from the last valid counter value received.

The optimised EMSDP counter scheme is defined as scheme 01.

### X.2.4	EMSDP Integrity protection

The integrity protection algorithm to be used and the length of the MAC is indicated in the BEST service discovery.

Integrity protection is mandatory for all control plane and user plane messages except for the following control plane commands when no valid keyset is agreed between the UE and HSE:

-	an EMSDP session request command originating from the UE or the HSE.  For this command the MAC shall not be present.

For an EMSDP start session command,  originating from the HSE, if the Key Id for the message is the same as the Key Id indicated in the Key agreement TLV then the MAC shall be calculated using the new keys resulting from the authentication vectors in the Key agreement TLV.

For all other signalling plane and user plane data packets the MAC shall be computed as follows:

First the following fields are calculated (where needed for the chosen algorithm):

INPUT-I	set to the essage Counter Value expanded and right padded with 0’s to a fixed size of 4 bytes.

COUNT-C	set to the essage Counter Value expanded and right padded with 0’s to a fixed size of 4 bytes

M (GSM)	set to the length of message in bytes. It is coded on 2 bytes.

LENGTH 	set to the length of message in bytes. It is coded on 2 bytes.

MESSAGE	the fields marked for integrity protection in figure 6.8.2.4.3.1: “data stack for the EMSDP transfers” concatenated in the order they appear in the data packet.

M (LTE)	the fields marked for integrity protection in figure 6.8.2.4.3.1: “data stack for the EMSDP transfers” concatenated in the order they appear in the data packet.

DIRECTION	The DIRECTION bit shall be “0” for UE to HSE data packets and set to “1” for HSE to UE data packets.

BEARER	For control plane messages this shall be set to “00000” and for user plane messages this shall be set to “10101”

FRAMETYPE	For control plane messages this shall be set to “00” and for user plane messages this shall be set to “AA”.

KI128	This is the agreed integrity key value truncated to the lowest 128 bits.

IK	This is the agreed integrity key value truncated to the lowest 128 bits.

If GIA4 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.241 [y] shall be used to generate the MAC value.  If the MAC length indicated in the BEST service discovery is less than the length of the MAC produced by the GIA4 function, then the MAC shall be truncated to the correct size from the right.

If GIA5 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.251 [x] shall be used to generate the MAC value. If the MAC length indicated in the BEST service discovery is less than the length of the MAC produced by the GIA5 function, then the MAC shall be truncated to the correct size from the right.

If UIA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.205 [w] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If UIA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.215 [14] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in Annex B.2.2 shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in Annex B.2.3 shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA3 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.221 [33] shall be used to generate the MAC-I value.  The MAC shall be set to the calculated MAC-I value truncated to the correct size from the right.

### X.2.5	EMSDP Encryption

The encryption protection algorithm to be used is indicated in the BEST service discovery.  If EEA0 is indicated then the message shall not be encrypted.

Encryption is mandatory for all control plane and user plane messages when an encryption algorithm other than EEA0 is indicated in the BEST service discovery, except for the following control plane commands when no valid keyset is agreed between the UE and HSE:

-	an EMSDP session request command originating from the UE or the HSE

-	an EMSDP start session command originating from the HSE

For all other messages the following encryption shall be applied the fields indicated in figure X.2.1-1: “data stack for the EMSDP transfers” to be encrypted.

First the following are computed (where relevant for the algorithm being used):

INPUT-I	set to the message Counter Value expanded and right padded with 0’s to a fixed size of 4 bytes.

COUNT-C	set to the message Counter Value expanded and right padded with 0’s to a fixed size of 4 bytes

M (GSM)	set to the length of message in bytes. It is coded on 2 bytes.

LENGTH 	set to the length of message in bytes. It is coded on 2 bytes.

MESSAGE	the fields marked for encryption protection in figure 6.8.2.4.3.1: “data stack for the EMSDP transfers” concatenated in the order they appear in the data packet.

M (LTE)	the fields marked for encryption protection in figure 6.8.2.4.3.1: “data stack for the EMSDP transfers” concatenated in the order they appear in the data packet.

DIRECTION	The DIRECTION bit shall be “0” for UE to HSE data packets and set to “1” for HSE to UE data packets.

BEARER	For signalling data packets this shall be set to “00000” and for user plane data packets this shall be set to “10101”

FRAMETYPE	For control plane messages this shall be set to “00” and for user plane messages this shall be set to “AA”.

KI128	This is the agreed encryption key value truncated to the lowest 128 bits.

CK	This is the agreed encryption key value truncated to the lowest 128 bits.

If GEA4 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.241 [y] shall be used to generate the OUTPUT value. The OUTPUT value replaces the fields that are encrypted in the data packet.

If GEA5 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.251 [x] shall be used to generate the OUTPUT value. The OUTPUT value replaces the fields that are encrypted in the data packet.

If UEA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.205 [w] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If UEA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.215 [14] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in Annex B.1.2 shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in Annex B.1.3 shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA3 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.221 [33] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

### X.2.6	EMSDP Commands

#### X.2.6.1	Overview

The following EMSDP commands are proposed Table X.2.6.1-1:

Table X.2.6.1-1: EMSDP commands

| Code (Hex) | Command |
| --- | --- |
| 10 | EMSDP Session Request |
| 11 | EMSDP Session Start |
| 12 | EMSDP Session Start Confirmation |
| 20 | EMSDP Session Terminate Request |
| 21 | EMSDP Session Terminate Response |
| 30 | EMSDP Manage Keys Request |
| 31 | EMSDP Manage Keys Response |
| 80 | EMSDP Message Reject |

##### X.2.6.1.1		EMSDP Session Request

The EMSDP Session Request command shall be used by the UE to trigger a new BEST session from the HSE.  This message shall include an identification of the UE, an indication of its BEST support and the end enterprise service that this data is a part of.

This message may be sent after a PDP context has been setup.

Note: the content of this message is used in the following EMSDP start session.

The cmd options for the EMSDP session request command are as follows:

Table X.2.6.1.1-1: Start EMSDP session request command options

| Name | M / C / O |
| --- | --- |
| IMSI TLV | M |
| BEST UE configuration TLV | M |
| Enterprise Setup Information Element TLV | O |
| Serving Network TLV | C |

IMSI TLV: This is a TLV that contains the IMSI as follows:

Table X.2.6.1.1-2: IMSI TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| IMSI TLV Tag | 1 byte | M | 01 |
| Length | 1 byte | M | Length of IMSI value (X) |
| IMSI value | X bytes | M | according to TS 24.008 [x]. |

BEST configuration TLV: This is a TLV that contains the BEST configuration details for the UE as follows:

Table X.2.6.1.1-3: BEST UE configuration TLV

| Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BEST UE configuration TLV Tag = '02' |  |  |  |  |  |  |  |  |
| Length of Best protocol ID contents = x bytes |  |  |  |  |  |  |  |  |
| BEST release supported by the UE |  |  |  |  | BEST counter schemes supported by the UE | Reserved for future use (set to 000) |  |  |
| BEST protocols supported for control plane messages |  | Reserved for future use (set to 000) |  |  | BEST protocols supported for user plane messages | Reserved for future use (set to 000) |  |  |
| User data push to UE Supported | BEST encryption algorithm GEA4 supported | BEST encryption algorithm GEA5 supported | BEST encryption algorithm UEA1 supported | BEST encryption algorithm UEA2 supported | BEST encryption algorithm EEA0 supported | BEST encryption algorithm 128-EEA1 supported | BEST encryption algorithm 128-EEA2 supported |
| BEST encryption algorithm 128-EEA3 supported | BEST integrity algorithm GIA4 supported | BEST integrity algorithm GIA5 supported | BEST integrity algorithm UIA1 supported | BEST integrity algorithm UIA2 supported | BEST integrity algorithm 128-EIA1 supported | BEST integrity algorithm 128-EIA2 supported | BEST integrity algorithm 128-EIA3 supported |

Where:

- BEST release supported by the UE – an indicator the release of the BEST solution that the UE has been designed to.  If no release is indicated this means that the BEST service is not supported.

  - Value: This shall be a 4 bit field where "0000" = Rel.14 and "0001" to "1111" are RFU,

- BEST counter schemes supported by the UE – a flag for each supported scheme. At least one scheme must be supported and indicated.

  - Value: '1' – Optimised EMSDP counter scheme.

- BEST protocols supported for control plane messages – a flag for each BEST control plane protocol that is supported by the UE. At least one scheme must be supported and indicated.

  - Value: '1' – EMSDP.

- BEST protocols supported for user plane messages – a flag for each BEST control plane protocol that is supported by the UE. At least one scheme must be supported and indicated.

  - Value: '1' – EMSDP.

- User data push to UE Supported – a flag to indicate whether the UE supports user data pushed to the UE without a request for user data from the UE.

  - Values:

    - '1' = User data push to UE supported,

    - '0' = User data push to UE not supported.

- BEST encryption algorithms supported by the UE – a flag for each supported algorithm: GEA4, GEA5, UEA1, UEA2, EEA0, 128-EEA1, 128-EEA2 and 128-EEA3. EEA0 shall always be supported and means no encryption.

  - Values:

    - '1' = Algorithm supported,

    - '0' = Algorithm not supported.

- BEST integrity algorithms supported by the UE – a flag for each supported algorithm: GIA4, GIA5, UIA1, UIA2, 128-EIA1, 128-EIA2 and 128-EIA3.

  - Values:

    - '1' = Algorithm supported,

    - '0' = Algorithm not supported.

Enterprise Setup Information Element TLV: This is a TLV element that contains information from the UE that is used by the HSE to setup the HSE to enterprise connection as follows:

Table X.2.6.1.1-4: Enterprise Setup Information Element TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Enterprise Setup Information Element TLV Tag | 1 byte | M | 03 |
| Length | 1 byte | M | Length of Enterprise URL (X+1) |
| UE-to-EAS flag | 1 bit | M |  |
| Enterprise URL | X bytes | O |  |

UE-to-EAS flag: If set to 0 it indicates that the UE requests a UE-to-HSE BEST secure session. It set to 1 it means that the UE requests a UE-to-EAS BEST secure session.

Enterprise URL: The enterprise URL is used by the HSE to identify the enterprise and the service that the data belongs to.  These services are out of scope of this specification.

Serving network TLV: This is a TLV that contains information on the serving network.

Table X.2.6.1.1-5: Serving Network TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Serving Network Element TLV Tag | 1 byte | M | 0B |
| Length | 1 byte | M | Length of IMSI value (3) |
| MCC/MNC | 3 bytes | M | MCC/MNC as defined in 3GPP TS 24.008 [x] |

Response:

If the HSE agrees to setup the session, it shall respond with an EMSDP Session Start command.

If the HSE does not agree to setup a BEST session, it may respond with a EMSDP Message Reject command.  This command may include the reason that the request has been rejected.

##### X.2.6.1.2		EMSDP Session Start

The EMSDP Session Start command is used by the HSE to setup a new BEST session.  This message shall contain information on the BEST service setup, key agreement details, a hash of the information sent by the UE in the prior EMSDP Session Request command and optionally, the HSE identity.

On recept of this command the UE shall:

- Perform a 3G security context authentication with the USIM using the RAND and AUTN combination from the Key Agreement TLV. If the USIM returns IK and CK values, the UE uses these keys and the HSE identity supplied (if the HSE Identity TLV is present) to generate the session keys for the EMSDP messages as detailed in clause A.y.  If the USIM determines re-syncronisation is required and returns an AUTS then the UE sends a EMSDP Message Reject command containing the AUTS to the HSE.

- Verify that the UE supports the BEST service indicated in the BEST Service configuration TLV.

- Verify the received message format, the CP COUNTER value and the message MAC value.

- Verify that the MAC supplied in the MAC TLV matches the MAC that would be produced for the previous EMSDP Session Request message if the BEST configuration in the BEST Service configuration TLV had been applied using the integrity key calculated from the Key agreement TLV.

The Start new EMSDP session response command has the following cmd options:

Table X.2.6.1.2-1: EMSDP Session Start command options

| Name | M / C / O |
| --- | --- |
| BEST Service configuration TLV | M |
| Key agreement TLV | M |
| EMSDP session request MAC TLV | C |
| HSE Identity TLV | O |
| EAS Container | O |

BEST Service configuration TLV: The BEST Service configuration TLV sets the BEST service parameters to be used in this session as follows:

Table X.2.6.1.2-2: BEST UE configuration TLV

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BEST HSE configuration TLV Tag = '04' |  |  |  |  |  |  |  |
| Length of Best protocol ID contents = x bytes |  |  |  |  |  |  |  |
| BEST Service Activated | BEST encryption algorithm GEA4 to be used | BEST encryption algorithm GEA5 to be used | BEST encryption algorithm UEA1 to be used | BEST encryption algorithm UEA2 to be used | BEST encryption algorithm EEA0 to be used | BEST encryption algorithm 128-EEA1 to be used | BEST encryption algorithm 128-EEA2 to be used |
| BEST signalling plane protocol identifier |  |  |  |  |  |  |  |
| BEST user plane protocol identifier |  |  |  |  |  |  |  |
| BEST encryption algorithm 128-EEA3 to be used | BEST integrity algorithm GIA4 to be used | BEST integrity algorithm GIA5 to be used | BEST integrity algorithm UIA1 to be used | BEST integrity algorithm UIA2 to be used | BEST integrity algorithm 128-EIA1 to be used | BEST integrity algorithm 128-EIA2 to be used | BEST integrity algorithm 128-EIA3 to be used |
| Reserved for future use (set to 000) |  |  | Local BEST configuration management allowed | EMSDP session ID length |  |  |  |
| Reserved for future use (set to 00) | Use EAS UP Keys | EMSDP MAC length |  | Size of EMSDP Data Length |  |  |  |

- BEST Service Activated – a bit flag that when set instructs the UE to use the BEST service and when clear instructs the UE not to use the BEST service,

- BEST signalling plane protocol identifier – 1 octet that is used to determine the BEST signalling protocol to be used from the following list (only one shall be indicated): 01 = type 01 signalling plane EMSDP message. All other values are reserved for future use.

- BEST user plane protocol identifier – 1 octet that is used to determine the BEST signalling protocol to be used from the following list (only one shall be indicated): 01 = type 01 user plane EMSDP message. All other values are reserved for future use.

- BEST encryption algorithm to be used – 1 octet that is used to define which of the following algorithms to use for encryption: GEA0, GEA4, GEA5, UEA0, UEA1, UEA2, EEA0, 128-EEA1, 128-EEA2 and 128-EEA3. Only one algorithm from this list shall be indicated.  If the Visited network indicated that BEST encryption is restricted, then the HSE shall indicate EEA0.

- BEST integrity algorithm to be used – 1 octet that is used to define which one of the following algorithms to use for integrity:GIA4, GIA5, UIA1, UIA2, 128-EIA1, 128-EIA2 and 128-EIA3). Only one algorithm from this list shall be indicated.

- BEST counter scheme to be used – a flag for each supported scheme.  4 bits that are used to define which one of the following counter schemes to use: "0001" = the optimised EMSDP counter scheme. All other values are reserved for future use. Only one counter scheme from this list is allowed to be indicated.

- Local BEST configuration management allowed – a flag to indicate that the software connected to the UE is allowed to manage the BEST service.

- EMSDP session ID length – 4 bits that indicates how many octets in the EMSDP data packet the EMSDP session ID will be on.

- New Session Required – 1 bit that indicates if a new session is required.  If this bit is set to 0 then the details agreed for the last session can be used and a new session is not required to be setup.

- EAS UP keys – If set to 0 it indicates that the UE should not derive the UE-to-EAS keys. If set to 1 it means that the UE shall derive UE-to-EAS keys to be used in a UE-to-EAS BEST secure session.

- EMSDP MAC length – 2 bits that indicates how many octets in the EMSDP data packet the integrity checksum (MAC) will be on, as follows: "00"=4 octets, "01"=8 octets, "10"=12 octets and "11"= 16 octets.  This value shall not be set to a size that is greater than MAC size produced by the chosen algorithm.

- Size of EMSDP Data Length – 4 bits that indicate how many octets are used for the EMSDP Data Length. "0000" is reserved for future use.

Any remaining bits are reserved for future use and are set to "0..0".

Key agreement TLV: this contains the RAND IE and AUTN IE specified in 3GPP TS 24.008 [z] as follows:

Table X.2.6.1.2-3: Key Agreement TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Key Agreement TLV | 1 byte | M | 05 |
| Length | 1 byte | M | 1 or 36 |
| Additional Information | 1 Byte | M | Additional Keys to be generated |
| RAND IE | 17 bytes | C | See 10.5.3.1 in 3GPP TS 24.008 [z] |
| AUTN IE | 18 bytes | C | See 10.5.3.1.1 in 3GPP TS 24.008 [z] |

If the 'Length of the Key agreement' is set to 1 then this means use current keyset agreed for this KEY ID.  In this case the RAND IE and AUTN IE shall not be present.

The Additional information are as follows:

Table X.2.6.1.2-4: Additional information

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Confirm Authentication flag | RFU | RFU | RFU | RFU | Key ID |  |  |

b8: 	Confirm Authentication flag:

1 = Confirmation message required.

0 = Confirmation message not required.

b7 to b4: RFU (set to 0)

b3 to b1: Key ID

Key ID to be used for this keyset.

EMSDP Session Request MAC TLV:  The EMSDP session request MAC TLV shall be present if the previous command was an EMSDP session request message. Its contents are as follows:

Table X.2.6.1.2-5: EMSDP session request MAC TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| EMSDP session request MAC Tag | 1 byte | M | 06 |
| Length | 1 byte | M | Length of HSE Identity (X) |
| EMSDP session request MAC | X bytes | M | Result of MAC calculation on previous EMSDP session request message using current keys and BEST configuration in this message. |

HSE Identity TLV: this contains a 4 octet numeric identifier for the HSE.  This should be unique to the HSE being used within the home network. It is formatted as follows:

Table X.2.6.1.2-6: HSE Identity TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| HSE Identity Tag | 1 byte | M | 07 |
| Length | 1 byte | M | Length of HSE Identity ('04') |
| HSE Identity | 4 bytes | M | 4 octet numeric identifier for the HSE |

The EAS Container TLV: this contains a 4 octent numeric identifier for the Enterprise Key ID.

Table X.2.6.1.2-7: EAS Container TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| EAS Container Tag | 1 byte | M | 08 |
| Length | 1 byte | M | Length of Enterprise Key ID |
| Enterprise Key ID | 4 bytes | M | numeric identifier for the Enterprise Key |

Response:

If the Confirmation message required flag in the Key agreement TLV is set and the message verifies, then the UE shall send an EMSDP start session confirmation message.

If the Confirmation message required flag in the Key agreement TLV is set and the message verifies, then the UE may send an EMSDP start session confirmation message.

If the message does not verify, then the UE shall respond with a Request Rejected command.  This command may include the reason that the request has been rejected.

If the USIM returns a AUTS as a result of the authentication, the the UE shall respond with a Request Rejected command with the reason "Authentication ReSync required" and including the AUTS.

##### X.2.6.1.3		EMSDP Session Start Confirmation message

The EMSDP Session Start Confirmation message is sent by the UE to confirm a previous EMSDP Session Start command.

This message is optional for the UE to send unless the "Confirmation message required" flag is set in the EMSDP Session Start command in which case this message shall be sent.

This command has the following cmd options:

Table X.2.6.1.3-1: EMSDP start session confirmation command options

| Name | M / C / O |
| --- | --- |
| AUTHENICATION RESPONSE TLV | M |

AUTHENICATION RESPONSE TLV: This TLV contains the authentication response for a successful authentication as follows:

Table X.2.6.1.3-2: AUTHENICATION RESPONSE TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| AUTHENICATION RESPONSE Tag | 1 byte | M | 08 |
| Length | 1 byte | M | Length of HSE Identity (X+1) |
| Key Information | 1 byte | M | Key identifier |
| RES | X bytes | M | As returned by the USIM. |

Where:

Key Information is coded:

Table X.2.6.1.3-3: Key Information information

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RFU | RFU | RFU | RFU | RFU | Key ID |  |  |

b8 to b4: RFU (set to 0)

b3 to b1: Key ID

Key ID to be used for this keyset.

##### X.2.6.1.4		EMSDP Session Terminate Request and Response

The EMSDP Session Terminate Request may be sent by either the UE or the HSE to terminate an existing EMSDP session.

The sending party shall close their session on sending this command and the receiving party shall close the session upon reception of this command.  All future BEST User Plane messages and BEST control plane messages for the session indicated in the session ID shall be either refused or ignored.

This command has no cmd options.

##### X.2.6.1.5	EMSDP Manage Keys Request

The EMSDP Manage Keys request command shall be sent by the UE to request the HSE to negotiate new key material.

If the BEST session requested includes cyphering then the UE shall send a Serving Network TLV (as defined in clause X.2.6.1.1).

##### X.2.6.1.6		EMSDP Manage Keys Response

The EMSDP Manage Keys command is used by the HSE to agree new keys.

The EMSDP Manage Keys command has the following cmd options:

Key agreement TLV: As detailed in clause X.2.6.1.2.

##### X.2.6.1.7		EMSDP Message Reject command

The EMSDP Message Reject command may be used by either the UE or the HSE to reject messages, data or control plane.

The EMSDP Message Reject command has the following cmd options:

Table X.2.6.1.7-1: Request Rejected command options

| Name | M / C / O |
| --- | --- |
| Rejection details TLV | C |
| AUTS TLV | C |

Where:

Rejection details TLV: Contains the rejection reason.  If the rejection reason is "Authentication ReSync required" then the Rejection details TLV and the AUTS TLV shall be present.  For all other reasons the rejection details TLV may be present.

Table X.2.6.1.7-2: Rejection details TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Rejection details Tag | 1 byte | M | 09 |
| Length | 1 byte | M | Length of HSE Identity (X+1) |
| Rejection reason | 1 byte | M | See below |

Rejection reason:

'00' = "Best session refused by the HSE"

'01' = "HSE not compatible with configuration request"

'02' = "UE not compatible with configuration request"

'03' = "HSE temporary error – try again later"

'04' = "Command Message error"

'05' = "Command message counter error"

'06 = "Authentication ReSync required"

'07' = "EMSDP session request MAC incorrect"

'08' = "Sesion ID not valid"

'09' = "Command not allowed"

Table X.2.6.1.7-3: AUTS TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Rejection details Tag | 1 byte | M | 0A |
| Length | 1 byte | M | Length of HSE Identity (X) |
| AUTS | 1 byte | M | AUTS as returned by the USIM |

### X.2.7	Procedures for BEST when using EMSDP

#### X.2.7.1	Introduction

This clause contains the procedures between the UE, HSE, HSS and EAS for the following BEST Services when using EMSDP:

- BEST key agreement only service in clause X.2.7.2

- BEST user plane integrity protected service in clause X.2.7.3

- BEST user plane confidential service in clause X.2.7.4

#### X.2.7.2	Procedures for BEST Key Agreement Only Service using EMSDP

Figure X.2.7.2-1 shows the messages exchanged between the UE, HSE, HSS/EMKS and EAS in order to setup a BEST Session when using EMSDP. In this figure, the EMKS and HSS are collapsed into one.

UE

HSE

P-GW

1. EMSDP Session Request

8. EMSDP Session Start

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

10. EMSDP Session Start Confirmation Message

5. EAS Session Request

7. EAS Session Start

9. Key derivation

6. EAS Key derivation

11. EAS Session Start Confirmation Message

Figure X.2.7.2-1: BEST Session Setup Procedure for BEST Key Agreement Only Service using EMSDP

In the above figure, steps 1-8 are mandatory and step 10 and 11 are optional depending on whether the HSE has set the 'Confirm Authentication' flag in the 'EMSDP Session Response' message. The contents of the respective commands are given in the respective clauses that detail the commands. Step 6 is only performed when a BEST UE-to-EAS UP session is setup.

Figure X.2.7.2-2 shows the messages exchanged between the UE, HSE, HSS/EMKS and EAS in order to refresh the keys when using EMSDP. In this figure, the EMKS and HSS are collapsed into one.

UE

HSE

P-GW

1. EMSDP Manage Keys Request

8. EMSDP Manage Keys Response

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

10. EMSDP Start Confirmation Message

5. EAS Manage Keys Request

7. EAS Manage Keys Response

9. Key derivation

6. EAS Key derivation

11. EAS Session Start Confirmation Message

Figure X.2.7.2-2: BEST Procedure for Key Refresh using EMSDP

In the above figure, steps 1-8 are mandatory and step 10 is optional depending on whether the HSE has set the 'Confirm Authentication' flag in the 'EMSDP Manage Keys Response' message. The contents of the respective commands are given in the respective clauses that detail the commands. Step 6 is only performed when a BEST UE-to-EAS UP session is setup.

#### X.2.7.3	Procedures for BEST User Plane Integrity Protected Service using EMSDP

Figure X.2.7.3-1 shows the messages exchanged between the UE, HSE, HSS/EMKS and EAS in order to setup a BEST Session when using EMSDP. In this figure, the EMKS and HSS are collapsed into one.

UE

HSE

P-GW

1. EMSDP Session Request

8. EMSDP Session Start

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

5. EAS Session Request

7. EAS Session Start

9. Key derivation

6. EAS Key derivation

Figure X.2.7.3-1: BEST Session Setup Procedure for BEST User Plane Integrity Only Service using EMSDP

In the above figure, steps 1-8 are mandatory. The contents of the respective commands are given in the respective clauses that detail the commands. Step 6 is only performed when a BEST UE-to-EAS UP session is setup.

Figure X.2.7.3-2 shows the messages exchanged between the UE, HSE, HSS/EMKS and EAS in order to refresh the keys when using EMSDP. In this figure, the EMKS and HSS are collapsed into one.

UE

HSE

P-GW

1. EMSDP Manage Keys Request

8. EMSDP Manage Keys Response

4. Key derivation

HSS / EMKS

2.Authentication Information Request

3. Authentication Information Answer

EAS

5. EAS Manage Keys Request

7. EAS Manage Keys Response

9. Key derivation

6. EAS Key derivation

Figure X.2.7.3-2: Best Session Setup Procedure for BEST Key Agreement Only Service using EMSDP

In the above figure, steps 1-8 are mandatory. The contents of the respective commands are given in the respective clauses that detail the commands. Step 6 is only performed when a BEST UE-to-EAS UP session is setup.

#### X.2.7.4	Procedures for BEST User Plane Confidential Service using EMSDP

The procedures for the BEST User Plane Confidentiality Service are the same as the one detailed in clause X.7.4.3.

**** End of Sixth Change ****

**** Seventh Change ****

**** End of Seventh Change ****

**** Eighth Change ****

**** End of Eighth Change ****

**** Nineth Change ****

**** End of Nineth Change ****