3GPP TSG SA WG3 (Security) Meeting #86bis	S3-170863

27-31 March 2017, Busan (S Korea)

Source:	NEC

Title:	pCR to TR 33.899: Removal of Editor’s Notes of Solution 8.1

Document for:	Approval

Agenda Item:	5.1.8

Work Item / Release:	FS_NSA / Rel-14

Abstract: This pCR clarifies the editor’s notes of Solution #8.1: Security isolation of network slices, Security mechanism differentiation of TR 33.899.

#### Discussion

This pCR clarifies the editor’s notes in this solution as follows, and removes all except for one relating to roaming scenario.

Editor’s note: The solution needs to describe the behaviour of the system when a mismatch occurs between the network slice ID requested by the UE and those available in the network.

UE has information on network slice as Configured NSSAI (Network Slice Selection Assistance Information).  The mismatch between the configured NSSAI in the UE and with the supported ones in the network is resolved according to the procedure specified in TS 23.502, clause 4.2.2.2.3. Within this procedure an appropriate AMF is selected and an updated NSSAI is provided to the UE.

Editor’s note: Privacy aspects related to the network slice ID are FFS.

Information on the UE transferred to the network such as NSSAI, SST (Slice/Service Type) and SD (Slice Differentiator) shall be protected. However this issue should be addressed separately by the solutions for key issue #7.8: Privacy protection of network slice identifier, and mentioned like this in introduction of this solution.

Editor’s note: Security profile ID role and definition are to be described. Furthermore, it is unclear which entity of the NextGen system this security profile ID is related to.

Security profile consists of sets of security configurations such as algorithm, key length, etc. for UE, and stored in SSS (Slice Security Server). Security profile ID is sent with the set of security configurations to other entities such as UDM, AUSF, AMF, SMF, UPF and UE for the management purpose. The text is added in step 6 of this solution.

Editor’s note: It’s FFS to consider the detailed procedures and interface between HSS and SSS, including how the UE is authenticated if the UE doesn't include the list of requested slice IDs in Step4, how to provision all security nodes with security profiles for all network slices, in Step 6 and how to support a RAN network slice in Step 10.

The security procedure has been updated with the consolidated architecture in phase 1 of SA2. The major change is that slice-specific security is established in session establishment phase. RAN slicing has yet to be mentioned in this solution, but RAN slice security is considered to be supported in the same way as core network. The text is added in step 16 of this solution.

Editor’s note: It’s FFS to consider the case when legacy UEs without any awareness of network slicing would  access the next generation network with network slicing.

Legacy UEs (Pre-Rel-15) do not support 5G NAS and thus cannot connect to the 5G core network. Legacy UEs can only connect to the EPC.

Editor’s note: It’s FFS whether the security procedures are executed by HSS in Step 7.

Authentication and Key Agreement (AKA) and NAS MM (Mobility Management) are executed by SEAF (Security Anchor Function), which is co-located in AMF.

#### Proposal

************* Begin of Change ***********

##### 5.8.4.1.1	Introduction

This solution addresses several key issues of #8.1 Security isolation of network slices and #8.2 Security mechanism differentiation for network slices. Information on the UE transferred to the network such as NSSAI, SST (Slice/Service Type) and SD (Slice Differentiator) shall be protected. However this issue should be addressed separately by the solutions for key issue #7.8: Privacy protection of network slice identifier.

##### 5.8.4.1.2	Solution details

The Slice Security Server (SSS) is a repository for the different slice security requirements and provides the appropriate security configuration per slice. The SSS may be collocated with UDM. Figure 5.8.4.1.2-1 shows security

UE

RAN

CP

Function

1. Attach (Slice ID(s))

3. Identity Request/Response

2. Attach

(Slice

ID(s))

HSS

4.

Authentication Data Request (IMSI,

Serving

Network

ID,

Network

Type, Slice ID(s))

8.

Authentication Data

Response (Authentication Vector(s) with Slice ID(s))

9. Control Plane Layer Security Setup *(AKA, Integrity Algorithm, Ciphering Algorithm

, Slice ID)

10. Access Layer Security Setup *(

Integrity Algorithm,

Security Algorithm

, Slice ID

)

14. AS Security Mode Complete (MAC

-

I)

SSS

5. Slice Security

Request

(Slice ID(s))

6. Slice Security Response (Slice ID(s), Slice Security Requirements)

7. Select per slice the relevant security

algorithm based on the requirements,

create Authentication Vectors per slice

11. Configure UL filter

for each slice

12. Bearer Setup per slice with individual security on radio &

network bearer

UE

RAN

CP

Function

1. Attach (Slice ID(s))

3. Identity Request/Response

2. Attach

(Slice

ID(s))

HSS

4.

Authentication Data Request (IMSI,

Serving

Network

ID,

Network

Type, Slice ID(s))

8.

Authentication Data

Response (Authentication Vector(s) with Slice ID(s))

9. Control Plane Layer Security Setup *(AKA, Integrity Algorithm, Ciphering Algorithm

, Slice ID)

10. Access Layer Security Setup *(

Integrity Algorithm,

Security Algorithm

, Slice ID

)

14. AS Security Mode Complete (MAC

-

I)

SSS

5. Slice Security

Request

(Slice ID(s))

6. Slice Security Response (Slice ID(s), Slice Security Requirements)

7. Select per slice the relevant security

algorithm based on the requirements,

create Authentication Vectors per slice

11. Configure UL filter

for each slice

12. Bearer Setup per slice with individual security on radio &

network bearer

procedure with the SSS in registration and session establishment with the following steps.

4.Auth. data

request

4.Auth. data

request

3.Auth. data

request

3.Auth. data

request

2.Registration

request

2.Registration

request

1.Registration

request

1.Registration

request

7. Select relevant security configurations and create Authentication Vectors

7. Select relevant security configurations and create Authentication Vectors

20.Slice security

config request

20.Slice security

config request

21.Slice Security

config response

21.Slice Security

config response

12. NAS MM SMC complete

12. NAS MM SMC complete

11. NAS MM (Mobility Management) SMC (Security Mode Command)

11. NAS MM (Mobility Management) SMC (Security Mode Command)

UE

UE

(R)AN

(R)AN

AMF

AMF

AUSF

AUSF

UDM

UDM

Network Slice

Network Slice

UPF

UPF

SSS

SSS

SMF

SMF

5.Security

config request

5.Security

config request

6. Security

config response

6. Security

config response

8. Auth. data response

8. Auth. data response

9. Auth. data

response

9. Auth. data

response

10. Authentication & Key Agreement

10. Authentication & Key Agreement

15. AS SMC

15. AS SMC

17. Session

request

17. Session

request

18. Session

request

18. Session

request

19. Session request

19. Session request

28. Session

request

28. Session

request

16. AS SMC complete

16. AS SMC complete

25. Session

accept

25. Session

accept

24. Session

accept

24. Session

accept

23. Session accept

23. Session accept

30. U-plane data SMC complete

30. U-plane data SMC complete

29. U-plane data SMC

29. U-plane data SMC

14. Registration

accept

14. Registration

accept

13. Registration

accept

13. Registration

accept

22. Session authorization

22. Session authorization

26. NAS SM (Session Management) SMC

26. NAS SM (Session Management) SMC

27. NAS SM SMC complete

27. NAS SM SMC complete

Figure 5.8.4.1.2-1 Security procedure with Slice Security Server (SSS) in registration and session establishment.

Step 1: UE sends registration request to (R)AN including Configured NSSAI (Network Slice Selection Assistance Information), which includes information on network slices for the UE.

Step 2: (R)AN selects AMF (Access and Mobility Management Function) and forwards the registration request including the NSSAI to the AMF.

Step 3: AMF extracts all the acceptable SM (Session Management)-NSSAI, which are used to select SMF (Session Management Function), and creates Accepted NSSAI and temp ID, which are stored in the AMF. AMF sends authentication data request to AUSF, which includes Accepted NSSAI and temp ID. In case of slice information mismatch between UE and network, the NSSAI is updated according to according to the procedure specified in TS 23.502, clause 4.2.2.2.3.  The AMF may query a Network Slice Selection Function (NSSF) for the slice selection, e.g. in case the UE does not provide a valid NSSAI.

Step 4: AUSF forwards the authentication data request to UDM (Unified Data Management).

Step 5: UDM sends security configuration request to SSS in order to get all the information on security configurations needed for the UE. The UDM requests the security configuration from the SSS.

Step 6: SSS sends a security configuration response to the UDM, which includes the requested security configurations. The configurations can be stored as security profile in the SSS, which consists of sets of security configurations such as algorithm, key length, etc. for the UE and for a specific slice. Security profile ID is sent with the set security configurations to other entities such as UDM, AUSF, AMF, SMF, UPF and UE for the management purpose.

Step 7: UDM selects the relevant security configurations based on the service requirements of the UE to establish NAS MM (Mobility Management) security and AS security such as algorithm, key length, etc., and creates AVs (Authentication Vector).

Step 8: UDM sends authentication data response to AUSF, which includes the selected security configurations and AVs.

Step 9: AUSF forwards the authentication date response to AMF.

Step 10: AKA (Authentication and Key Agreement) is carried out between UE and AMF using AVs provide by UDM.

Step 11: AMF sends NAS MM (Mobility Management) SMC (Security Mode Command) to UE, which includes the security configurations selected by UDM.

Step 12: UE sends NAS MM SMC complete to AMF, and NAS MM security is established between UE and AMF.

Step 13: AMF sends registration accept to (R)AN, which includes the security configurations and temp ID.

Step 14: (R)AN forwards the registration accept to UE.

Step 15: (R)AN sends AS SMC to UE, which includes the security configurations selected by UDM.

Step 16: UE sends AS SMC complete to (R)AN, and AS security is established between UE and (R)AN. When RAN slicing is applied, the security configurations for each slice are provided and AS SMC is established for each slice.

Step 17: UE sends session request to (R)AN, which includes SM-NSSAI and temp ID.

Step 18: (R)AN forwards the session request to AMF.

Step 19: AMF selects SMF (Session Management Function) based on the information included in the request and sends the session request to the SMF.

Step 20: SMF sends slice security configuration request to SSS, which includes SM-NSSAI and temp ID, if SMF doesn’t have the configurations to establish NAS SM (Session Management) and U-plane data security.

Step 21: SSS sends slice security configuration response to SMF, which includes the requested configurations.

Step 22: SMF selects UPF, and session authorization to access DN (Data Network) is carried out through the UPF.

Step 23: SMF sends session accept to AMF.

Step 24: AMF forwards the session accept to (R)AN.

Step 25: (R)AN forwards the session accept to UE.

Step 26: SMF sends NAS SM (Session Management) SMC to UE, which includes the security configurations.

Step 27: UE sends NAS SM SMC complete to SMF, and NAS SM security is established between UE and SMF.

Step 28: SMF sends session request to UPF, includes the security configurations.

Step 29: UPF sends U-plane data SMC to UE, which includes the security configurations.

Step 30: UE sends U-plane data SMC complete to UPF, and U-plane data security is established between UE and UPF.

If a UE would like to register to a slice not included in the registration request of step 1, then the UE would be required to send another registration request with the corresponding SM-NSSAI it would like to be registered to. The AMF detects that the UE is already registered to the network and would not perform authentication with the UE, but still query the SSS for the security configurations of the slice and slice authorization.

In roaming scenarios depicted in the architectures specified in 23.501, the AMF, V-SMF and UPF reside always in the VPLMN while the AUSF and UDM reside in the HPLMN. There is no change from the reference points between AMF, SMF AUSF and UDM for home routed and local breakout scenarios.

Editor’s note: The solution needs to describe how the roaming scenarios are handled.

##### 5.8.4.1.3	Evaluation

FFS

**************** End of Change *************